SA WG2 Meeting #145E e-meeting	S2-2104202

May 17-28, 2021, Elbonia 	()

Source:	vivo

Title:	TS 23.304: Clause 5.9.3.3 EN remove

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution is to remove ENs in clause 5.9.3.3 Identifiers for Layer-2 UE-to-Network Relay.

# 1.	Introduction

This paper focuses on removing the following ENs in 5.9.3.3 Identifiers for Layer-2 UE-to-Network Relay.

Editor's note:	It is FFS what identifiers are used in addition to the identifiers specified in 5.9.3.1 for Layer-2 UE-to-Network Relay use case.

Editor's note:	It is FFS with what information the RSC is associated for Layer-2 UE-to-Network Relay use case.

For the first EN, considering the Remote UE may be subjected to the Mobility Restrictions, Remote UE needs to know the current serving cell of relay UE camps on whether is the remote UE’s allowed area for relay selection. an identifier of serving cell seems to be included in the relay discovery message, it is similar to Relay TAI applied to L3 relay. So it is proposed to and the following identifier in the L2 relay discovery message:

-	Relay TAI: indicates the Tracking Area Identity corresponding to the serving cell of the Layer-2 UE-to-Network Relay.

For the second EN, Considering the PDU session for L2 relay belongs to remote UE, remote UE establishes the PDU session with network based on its own URSP, all the PDU session parameters are transparent for relay UE. Remote UE only needs to know the relay UE has the L2 relay capability and no need to know what PDU session parameters that the relay UE supports.

As for how the remote UE know whether the relay UE support L2 relay, since the RSC applied to L3 relay

is associated with a set of PDU session parameters (e.g. PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference) but the one of L2 relay has not. Thus the remote UE can, based on whether there is this association, be aware of the RSC whether is for L2 relay or L3 relay.

Thus there is no need to associate with additional info/parameters for RSC applied to L2 relay, the second EN can be removed by adding the following EN to explanation:

NOTE:	The Relay Service Code applied to L2 relay has no associated parameters.

# 2.	Proposal

It is proposed to capture the following changes into TS 23.304.

# * * * * First change * * * *

#### 5.9.3.3	Identifiers for Layer-2 UE-to-Network Relay

The following parameters may be used in addition to the parameters specified in 5.9.3.1 for Layer-2 UE-to-Network Relay selection:

-	Relay TAI: indicates the Tracking Area Identity corresponding to the serving cell of the Layer-2 UE-to-Network Relay.

# * * * * End of changes * * * *