SA WG2 Meeting #145E e-meeting	S2-2104208r04

May 17 – 28, 2021, Elbonia

Source:	vivo

Title:	TS 23.304: Principles for applying parameters for ProSe UE-to-Network Relay

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution is to add Principles for applying parameters for ProSe UE-to-Network Relay.

# 1.	Discussion

This pCR tries to add the Principles for applying parameters for ProSe UE-to-Network Relay. The basic idea is similar to ProSe direct discovery and communication.

R01: add a EN for mobility restriction.

# 2.	Proposal

It is proposed to capture the following changes into TS 23.304.

# * * * * First change * * * *

#### 5.1.4.2	Principles for applying parameters for ProSe UE-to-Network Relay

##### 5.1.4.2.1	Principles for applying parameters for ProSe UE-to-Network Relay discovery

For ProSe UE-to-Network Relay discovery over PC5 reference point, the operator may pre-configure the UEs with the required provisioning parameters for ProSe UE-to-Network Relay discovery, without the need for the UEs to connect to the 5GC to get this initial configuration. The following applies:

-	The provisioning parameters for ProSe UE-to-Network Relay discovery could be from different sources and their priorities are described in clause 5.1.1.

-	The ME provisioning parameters shall not be erased when a USIM is deselected or replaced.

The UE shall use radio resources for ProSe UE-to-Network Relay discovery as follows:

-	While a UE has a serving cell and is camped on a cell and the UE intends to use for ProSe UE-to-Network Relay discovery the radio resources (i.e. carrier frequency) operated by this cell, then the UE shall use the radio resource description indicated by this cell the UE is camped on and ignore any radio resource description of the same radio resource provisioned in the ME or the UICC. If the cell does not provide radio resources for ProSe UE-to-Network Relay discovery, the UE shall not perform ProSe UE-to-Network Relay discovery message transmission and reception on radio resources operated by this cell;

-	If the UE intends to use "operator-managed" radio resources (i.e. carrier frequency) for ProSe UE-to-Network Relay discovery that are not operated by the UE's serving cell, as specified in clause 5.1.4.1, or if the UE is out of coverage, the UE shall search for a cell in any PLMN that is operating the provisioned radio resources (i.e. carrier frequency) as defined in TS 38.300 [12] and TS 38.304 [13]; and:

-	If the UE finds such a cell in the registered PLMN or a PLMN equivalent to the registered PLMN, and authorization for ProSe UE-to-Network Relay discovery to this PLMN is confirmed, the UE shall use the radio resource description indicated by that cell. If that cell does not provide radio resources for ProSe UE-to-Network Relay discovery, the UE shall not perform ProSe UE-to-Network Relay discovery message transmission and reception on those radio resources;

-	If the UE finds such a cell but not in the registered PLMN or a PLMN equivalent to the registered PLMN, and that cell belongs to a PLMN authorized for ProSe UE-to-Network Relay discovery and provides radio resources for ProSe UE-to-Network Relay discovery then the UE shall perform PLMN selection triggered by ProSe UE-to-Network Relay discovery as defined in TS 23.122 [14];

-	If the UE finds such cell but not in a PLMN authorized for ProSe UE-to-Network Relay discovery the UE shall not use ProSe UE-to-Network Relay discovery;

-	If the UE does not find any such cell in any PLMN, then the UE shall consider itself "not served by NG-RAN" and use radio resources provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize ProSe UE-to-Network Relay discovery, then the UE is not authorized to transmit;

-	The UE is allowed to use "operator-managed" radio resources (i.e. carrier frequency) for ProSe UE-to-Network Relay discovery when the UICC shall indicate it;

-	If the UE intends to use "non-operator-managed" radio resources (i.e. carrier frequency) for ProSe UE-to-Network Relay discovery, according to TS 36.331 [15] or TS 38.331 [16] and as specified in clause 5.1.4.1, then the UE shall perform ProSe UE-to-Network Relay discovery using resource provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize ProSe UE-to-Network Relay discovery, then the UE is not authorized to transmit;

NOTE 1:	It is possible for operators to configure UEs (e.g. Public Safety UEs) to use only "operator-managed" radio resources (i.e. carrier frequency) for ProSe UE-to-Network Relay discovery when the UE is "not served by NG-RAN".

-	The UE provisioning shall support setting Geographical Areas;

NOTE 2:	It is possible for a UE to use other radio resources for ProSe UE-to-Network Relay discovery based on the Geographical Area instead of those operated by the serving NG-RAN cell, when provisioned in the UE, even if the UE's serving cell offers normal service and the SIBs for NR sidelink communication defined in TS 38.331 [16] indicates that the service (ProSe UE-to-Network Relay discovery) is available. This is to cover the scenario when e.g. the radio resources used for ProSe UE-to-Network Relay discovery are not owned by the serving network of the UE.

NOTE 3:	When cross-carrier operation is supported, according to TS 36.331 [15] or TS 38.331 [16], a UE can be instructed by its serving cell to perform ProSe UE-to-Network Relay discovery over a different carrier frequency. The UE is still considered as "served by NG-RAN" in this case.

NOTE 4:	The scenario that a cell is detected and the cell does not provide support for ProSe UE-to-Network Relay discovery when the UE attempts to use a carrier frequency configured for ProSe UE-to-Network Relay discovery, is considered a configuration error. Therefore, the UE does not transmit on that frequency to avoid interference to the network.

-	The ProSe UE-to-Network Relay discovery is only specified for NR.

Editor's note:	  It is FFS whether and how to apply the mobility restriction for UE-to-Network relay or remote UE.

##### 5.1.4.2.2	Principles for applying parameters for ProSe UE-to-Network Relay communication

For ProSe UE-to-Network Relay communication over PC5 reference point, the operator may pre-configure the UEs with the required provisioning parameters for ProSe UE-to-Network Relay communication, without the need for the UEs to connect to the 5GC to get this initial configuration. The following applies:

-	The provisioning parameters for ProSe UE-to-Network Relay communication could be from different sources and their priorities are described in clause 5.1.1.

-	The ME provisioning parameters shall not be erased when a USIM is deselected or replaced.

The UE shall use radio resources for ProSe UE-to-Network Relay communication as follows:

-	While a UE has a serving cell and is camped on a cell and the UE intends to use for ProSe UE-to-Network Relay communication the radio resources (i.e. carrier frequency) operated by this cell, then the UE shall use the radio resource description indicated by this cell the UE is camped on and ignore any radio resource description of the same radio resource provisioned in the ME or the UICC. If the cell does not provide radio resources for ProSe UE-to-Network Relay communication, the UE shall not perform ProSe UE-to-Network Relay communication message transmission and reception on radio resources operated by this cell;

-	If the UE intends to use "operator-managed" radio resources (i.e. carrier frequency) for ProSe UE-to-Network Relay communication that are not operated by the UE's serving cell, as specified in clause 5.1.4.1, or if the UE is out of coverage, the UE shall search for a cell in any PLMN that is operating the provisioned radio resources (i.e. carrier frequency) as defined in TS 38.300 [12] and TS 38.304 [13]; and:

-	If the UE finds such a cell in the registered PLMN or a PLMN equivalent to the registered PLMN, and authorization for ProSe UE-to-Network Relay communication to this PLMN is confirmed, the UE shall use the radio resource description indicated by that cell. If that cell does not provide radio resources for ProSe UE-to-Network Relay communication, the UE shall not perform ProSe UE-to-Network Relay communication message transmission and reception on those radio resources;

-	If the UE finds such a cell but not in the registered PLMN or a PLMN equivalent to the registered PLMN, and that cell belongs to a PLMN authorized for ProSe UE-to-Network Relay communication and provides radio resources for ProSe UE-to-Network Relay communication then the UE shall perform PLMN selection triggered by ProSe UE-to-Network Relay communication as defined in TS 23.122 [14];

-	If the UE finds such cell but not in a PLMN authorized for ProSe UE-to-Network Relay communication the UE shall not use ProSe UE-to-Network Relay communication;

-	If the UE does not find any such cell in any PLMN, then the UE shall consider itself "not served by NG-RAN" and use radio resources provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize ProSe UE-to-Network Relay discovery, then the UE is not authorized to transmit;

-	The UE is allowed to use "operator-managed" radio resources (i.e. carrier frequency) provisioned in the ME or the UICC for ProSe UE-to-Network Relay communication if the UICC indicates it is authorized;

-	If the UE intends to use "non-operator-managed" radio resources (i.e. carrier frequency) for ProSe UE-to-Network Relay communication, according to TS 36.331 [15] or TS 38.331 [16] and as specified in clause 5.1.4.1, then the UE shall perform ProSe UE-to-Network Relay communication using resource provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize ProSe UE-to-Network Relay communication, then the UE is not authorized to transmit;

NOTE 1:	It is possible for operators to configure UEs (e.g. Public Safety UEs) to use only "operator-managed" radio resources (i.e. carrier frequency) for ProSe UE-to-Network Relay communication when the UE is "not served by NG-RAN".

-	The UE provisioning shall support setting Geographical Areas;

NOTE 2:	It is possible for a UE to use other radio resources for ProSe UE-to-Network Relay communication based on the Geographical Area instead of those operated by the serving NG-RAN cell, when provisioned in the UE, even if the UE's serving cell offers normal service and the SIBs for NR sidelink communication defined in TS 38.331 [16] indicates that the service (ProSe UE-to-Network Relay communication) is available. This is to cover the scenario when e.g. the radio resources used for ProSe UE-to-Network Relay communication are not owned by the serving network of the UE.

NOTE 3:	When cross-carrier operation is supported, according to TS 36.331 [15] or TS 38.331 [16], a UE can be instructed by its serving cell to perform ProSe UE-to-Network Relay communication over a different carrier frequency. The UE is still considered as "served by NG-RAN" in this case.

NOTE 4:	The scenario that a cell is detected and the cell does not provide support for ProSe UE-to-Network Relay communication when the UE attempts to use a carrier frequency configured for ProSe UE-to-Network Relay communication, is considered a configuration error. Therefore, the UE does not transmit on that frequency to avoid interference to the network.

-	The ProSe UE-to-Network Relay communication is only specified for NR.

Editor's note:	 It is FFS whether and how to apply the mobility restriction for UE-to-Network relay or remote UE.

# * * * * End of changes * * * *