3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2104272r01

Elbonia, May 17– 28, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon, Philips International B.V.

Title:	Layer-2 link establishment for UE-to-Network Relaying

Document for:	Approval

Agenda Item:	8.8.2

Work Item / Release:	FS_5G_ProSe/Rel-17

Abstract: Updates of the procedure of Layer-2 link establishment for ProSe communication via UE-to-Network Relay.

1. Introduction/Discussion

The Layer-2 link establishment procedure used for a Remote UE and Layer-2/Layer-3 UE-to-Network Relay uses the procedure described in clause 6.4.3.1. However, current procedure in clause 6.4.3.1 is only for ProSe Direct communication. This contribution updates the procedure of Layer-2 link establishment for UE-to-Network Relaying.

This pCR further clarifies step 1 in the procedure. In this step a reference is made to Section 5.9.2 on how to determine the Layer-2 ID, which in particular in the case of UE-to-NW relay (but also for general ProSe applications and services) will most likely be determined during discovery and not through policy configuration. Hence the second sentence referring to Section 5.1.3 is highly confusing. Instead of repeating some of the text of Section 5.9.2.4 about determining the layer-2 IDs (e.g. based on discovery), it is proposed to remove that sentence and clarify the relay cases it destination Layer-2 ID is determined from the discovery procedure.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.304.

# * * * * First change * * * *

6.4.3.X	Layer-2 link establishment over PC5 reference point for ProSe UE-to-Network Relay

The Remote UE initiates a one-to-one communication connection with the selected ProSe UE-to-Network Relay over PC5 using the procedure as described in Figure 6.4.3.1-1 of clause 6.4.3.1, with the following differences and clarifications:

- In Step 2 , UE-1(i.e., the Remote UE) determines the Relay Service Code to be used.

- In step 3, Remote UE (UE-1) sends a unicast Direct Communication Request message to the selected ProSe UE-to-Network Relay using the destination Layer-2 ID obtained from the UE-to-Network Relay Discovery process. The Direct Communication Request message includes: the information described in clause 6.5.1 and 6.5.2.

-	Source User Info: The identity of the Remote UE requesting relay operation.

-	Target User Info: The identity of the UE-to-Network Relay provided to the Remote UE during UE-to-Network Relay discovery.

-	Relay Service Code: indicates the connectivity service provided by the UE-to-Network Relay as requested by the Remote UE.

NOTE X: If there is a one-to-one mapping between the Relay Service Code and Layer-2 ID of the UE-to-Network Relay, then Relay Service Code can be not included in the Direct Communication Request message.

-	Security Information: the information for the establishment of security.

- In step 4 and step 5, step 4a and step 5a are performed, and it corresponds to the Option A) UE oriented Layer-2 link establishment. In case of Layer-2 UE-to-Network Relay, the Remote UE does not send the IP Address Configuration, Link-Local IPv6 Address and QoS Info to the Layer-2 UE-to-Network Relay, and the Direct Communication Accept message does not include IP Address Configuration, Link-Local IPv6 Address and QoS Info.

- In case of Layer-2 UE-to-Network Relay, step 6 is not performed.

# * * * * End of changes * * * *