SA WG2 Meeting #145E (e-meeting)	S2-2104281r01

May 17 – 28, 2021, Elbonia

Source:	CATT

Title:	MBS Session state model

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract of the contribution: This contribution proposes the update to the description of MBS Session state model.

- Introduction

This contribution proposes some updates to the description of MBS Session state model in clause 4.3 of TS 23.247, to remove the EN, etc.

# 2. Proposal

It is proposed to include the following changes in TS 23.247.

******************** 1st Change ********************

4.3	MBS session state model

The following states are defined for the MBS session:

-	Configured state: MBS session is not established in 5GC, while some information about the multicast session (e.g. QoS information) is configured in 5GC NFs (e.g. MB-SMF) serving the multicast session (implicitly or explicitly), but no User Plane resources towards NG-RAN are reserved and no MBS data can be transmitted. For instance, TMGIs is allocated but no complete session information is provided. For multicast session, UEs may be allowed to join (subject to authorization check and configuration), but the first accepted UE join request will trigger the multicast session establishment.

NOTE 1:	The SMF is not involved in the multicast session while the multicast session is in configured state.

NOTE 2:	There may be several sub-states in the configured state, e.g. only TMGIs requested, or information about the MBS session provided. For multicast session, there may be a sub-states that the AF/AS requests to start the session but no UE joined.

-	Active state: MBS session is established. UEs are allowed to join the multicast session (subject to authorization check). 5GS resources for the MBS session are reserved, and MBS data can be transmitted via the 5GS. For multicast session, corresponding 5GS resources are reserved towards the NG-RAN and the UEs depending on participating UE locations, and UEs are allowed to join the multicast session (subject to authorization check).

-	Inactive state: Only applicable for multicast session. MBS session is established. No multicast data can be transmitted by 5GC nodes. UEs that joined the multicast session may be in CM CONNECTED or CM IDLE state. UEs are allowed to join the multicast session (subject to authorization check). Resources for 5GC Shared MBS traffic delivery method of the multicast session are released, while information associated to the 5GC Individual MBS traffic delivery method of the multicast session is kept in the UE context in RAN node if the UE that joined the multicast session is in CM CONNECTED state.

The following state transition procedures are defined:

-	MBS Session Configuration: State transition from De-configured to Configured state, which is optional. The AF/AS provides information about the MBS session and optionally request the allocation of a TMGI. Alternatively, there is network-internal configuration of the MBS session. No resources for both multicast and broadcast session, or only resources at MB-SMF, NEF and MB-UPF for multicast session are reserved and no MBS data can be transmitted. The configuration may indicate when a MBS session can become active. The AF/AS may provide configuration in several steps, e.g. to first request TMGIs and then provide full information about the MBS session and, for multicast session, allow it to be established.

-	MBS Session Establishment: State transition from Configured to Active or Inactive (only for multicast session) state. 5GC resources for the MBS session are being reserved. For broadcast session, User Plane resources towards the NG-RAN are being reserved, the establishment may be combined with the configuration. For multicast session, when the join request of the first UE for the multicast session is accepted, the multicast session is established either in inactive or active state, depending on configuration.

-	Multicast Session Activation: Only applicable for multicast session. State transition from Inactive to Active state. CM IDLE UEs that joined the multicast session are notified. Activation can be triggered by AF request, or reception of multicast data.

-	Multicast Session Deactivation: Only applicable for multicast session. State transition from Active to Inactive state. Deactivation can be triggered by AF request, or no reception of multicast data.

-	MBS Session Release: State transition from Active or Inactive (only for multicast session) to Configured state. All resources for the MBS session are released in both 5GC nodes and RAN nodes if the release is triggered by an AF/AS request. For multicast session, only resources at MB-SMF, NEF and MB-UPF are kept if the release is triggered by the last UE leaving the multicast session, UEs that joined the multicast session are notified if the release is triggered by an AF request. For multicast session, the release is possible for an active or inactive multicast session.

-	MBS Session Deconfiguration: State transition from other state to D-configured state. All information about the MBS session is removed from the 5GS, and TMGIs (if allocated during MBS Session Configuration) are deallocated. All resources for the MBS session are released in both 5GC nodes and RAN nodes.  The MBS Session Deconfiguration may be triggered by an AF request.

Figure 4.3-1 shows the MBS session states and state transitions, wherein the start point is De-Configured state or Configured state.

Figure 4.3-1: Multicast session states and state transitions

NOTE 3:	The state model and state transition is handled in MB-SMF, for multicast session, UE joining into a multicast session may not trigger the state transition.

******************** End of Change ********************