3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2104306

Elbonia, May 17– 28, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon, CBN

Title:	TS 23.247: Session activation/deactivation for MBS

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This contribution discusses the session activation / deactivation procedures and provides the update towards the current document.

1. Discussion

MBS Session activation/deactivation is regarded as the open issue of current study, and such issues was discussed in the pre-meeting CC#1. As analysed in the slideshow, responsibility of Session Activation procedure is twofold, firstly is to activate the MBS session context at NG-RAN side, secondly is to provide per UE N2 SM info to the NG-RAN nodes. In light of the current agreement of the session management procedures, it is proposed to let MB-SMF contact with SMF/AMF separately to achieve that.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.247.

# * * * * First change * * * * (All Text New)

7.2.5	MBS session activation and deactivation

7.2.5.1	General

MBS Session activation procedure is for multicast only. MBS Session activation procedure is triggered by MB-SMF, when it receives the notification from MB-UPF for the downlink MBS DL data, or when it receives the request directly from AF or via NEF. The MBS Session activation procedure is used for activating the resources for MBS data at NG-RAN node. The multicast session state transits from inactive to active after MBS Session activation procedure, see clause 4.3.

MBS Session deactivation procedure is for multicast only. MBS Session deactivation procedure is triggered by MB-SMF, when it receives the notification from MB-UPF in the case of no downlink data to be transmitted, or when it receives the request directly from AF or via NEF. The MBS Session deactivation procedure is used for deactivating the resources for MBS data at NG-RAN node. The multicast session state transits from active to inactive after MBS Session deactivation procedure, see clause 4.3.

7.2.5.2	MBS session activation procedure

The following can trigger the MBS session activation procedure:

- AF requests MB-SMF to activate the MBS session;

- MB-UPF receives the multicast data and notifies MB-SMF.

UE

RAN

AMF

SMF

MB-SMF

2. Session activation ()

1. MB-SMF triggers session activation

3. MBS_MT_Reachability request ()

5. AMF pages idle mode UEs

6. Service request ()

7. MBS_MT_Reachability Notify ()

8. Namf_Communication_N1N2MessageTransfer ()

9. N2 request (N2 SM)

11. Session Activation (TMGI)

12. NGAP activation message (TMGI)

10. Steps 7a to 7e and 9 to 12 as defined in clause 7.2.1.3

4. MBS_MT_Reachability response ()

Figure 7.2.5.2-1: MBS session activation procedure.

Editor's Note: names of messages are FFS.

- The procedure may be triggered by the following events:

- When MB-UPF receives downlink data for a MBS session, MB-UPF sends MB-N4 Notification (N4 Session ID) to the MB-SMF for activating the MBS session.

- AF sends MBS Activation request (TMGI) to the MB-SMF directly or via NEF.

- MB-SMF sends Session activation notification (TMGI) to SMF(s).

Based on the received TMGI, SMF finds the list of UEs that joined the MBS session identified by the TMGI. If SMF determines the user plane of the associated PDU session(s) of the UE(s) with respect to TMGI are activated already, step 3-9 will be skipped for those UE.

- SMF sends MBS_MT_Reachability Request to AMF, with including (UE list, TMGI).

For each UE in the list, the AMF determines CM state of the UE:

- [Optional] If the UE involved in the MBS Session is in CM-CONNECTED state, the AMF responds the list of the UE involved in the MBS Session and in CM-CONNECTED state, using MBS_MT_Reachability response (UE list). Step 5-6 will not be executed for that UEs.

- [Optional] If AMF determines that there are any UEs in CM-IDLE state and involved in the MBS Session, and AMF figures out the paging area considering all the UE(s), which need be paged. The AMF send a paging request message to the NG-RAN node(s) belonging to this Paging Area with the TMGI as the identifier to be paged if the related NG-RAN node(s) support the MBS session.

NOTE 1:	The details of the paging are specified by the RAN WGs.

- The UE in IDLE state sends Service Request message to AMF, see clause 4.2.3 of TS 23.502 [6].

- After receiving the Service Request sent by the UE, the AMF responds to MB-SMF with MBS_MT_Reachability Notify (UE ID) message.

- After receiving MBS_MT_Reachability Response message, SMF determines the related UE is in CM-Connected State and sends Namf_Communication_N1N2MessageTransfer (N2 SM message (MBS Session identifier, associated QoS profiles) to AMF for the UE which is identified in step 3.

- AMF sends N2 request message (N2 SM message (MBS Session identifier, associated QoS profiles) to the RAN node.

- If the shared tunnel has not been established before, the shared tunnel is established at this step, as steps 7a to 7e defined in clause 7.2.1.3. In addition, NG-RAN responses to SMF, as steps 9 to 12 defined in clause 7.2.1.3. The NG-RAN configures UE with RRC messages if needed.

- MB-SMF sends Session Activation (TMGI) to the AMF.

NOTE 2:	The messages in step 10 and 11 is MBS-specific messages and it is possible that the AMF(s) in step 10 are not associate to any UEs involved in the MBS Session.

- AMF sends NGAP activation message (TMGI) to the RAN nodes.

Editor's Note: It is FFS that if messages 11, 12 are sent only to the NG-RAN nodes for which the MB-N3 is setup at step 10.

Editor's Note: Whether it is the AMF or the MB-SMF that stores RAN ID for the interaction with RAN in step 11 needs to align with clause 7.2.1.

7.2.5.3	MBS session deactivation procedure

UE

RAN

AMF

SMF

MB-SMF

1. MB-SMF triggers session deactivation

3. MBS Session deactivation request (TMGI)

5. RAN node sets the MBS session state to inactive

6. NGAP deactivation response message ()

4. NGAP deactivation request message (TMGI)

7. MBS Session deactivation response ()

2. MBS Session deactivation request (TMGI)

Figure 7.2.5.2-1-2: MBS session deactivation procedure.

Editor's Note: service operation of messages are FFS.

1.	The procedure may be triggered by the following events:

- When MB-UPF detects there is no data receives for the MBS Session, MB-UPF sends MB-N4 Notification (N4 Session ID) to the MB-SMF for deactivating the MBS session.

- AF sends MBS Deactivation request (TMGI) to the MB-SMF directly or via NEF.

2.	The MB-SMF sends MBS Session deactivation request (TMGI) to the SMFs.

NOTE:	For 5GC Individual MBS traffic delivery, the SMFs may trigger the removal of the unicast QoS flows at NG-RAN node, which is used for the transmission of MBS session identified by the TMGI.

3.	The MB-SMF sends MBS Session deactivation Request (TMGI) to the AMFs.

4.	The AMF sends NGAP deactivation request message (TMGI) to the RAN nodes.

Editor's Note: Whether it is the AMF or the MB-SMF that stores RAN ID for the interaction with RAN in step 3 needs to align with clause 7.2.1.

5.	NG-RAN sets the MBS session state with respect to TMGI to inactive. In this procedure, the NG-RAN will not release the MBS session context, and MB-N3 tunnel for the MBS session.

6.	NG-RAN acknowledges the NGAP deactivation Response message.

7.	The AMF invokes MBS Session deactivation Response to acknowledge the service for MB-SMF.

# * * * * End of changes * * * *