3GPP TSG-SA WG2 Meeting #145E	S2-2104332r02

# Elbonia, May 17 – May 28 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2919 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 T3: Handling of SUPI/SUCI format when accessing to a SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Rel-16, the SUPI/SUCI of a SNPN-enabled UE can be format of NAI or IMSI, when a UE performs Initial Registration to an SNPN. The selected NID provided by the NG-RAN is used as the NID of the subscribed SNPN, as in Rel-16 an SNPN-enabled UE is only allowed to access the subscribed SNPN. In Rel-17, an SNPN-enabled UE can also access SNPNs different from the subscribed SNPN. This requires some clarifications on which types of SUPI/SUCIs and which characteristics SUPI/SUCI shall support in order to enable these new scenarios, specifically when IMSI based SUPI is used. For example, if the SNPN and the Credentials Holder are using the same PLMN ID and IMSI based SUPI is used, currently the HNI(MCC/MNC) of the corresponding SUCI is not able to distinguish if the UE is from the Credentials Holder or if the UE is a native subscriber. When IMSI based SUPI is used, the SNPN must choose different PLMN ID than Credentials Holder in order to establish relation with the Credentials Holder. And Credentials Holders do not share the same PLMN ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In order for a UE to be able to connect to an SNPN using credential from Credentials Holder and use IMSI based SUPI, the following changes are proposed: Do not share the PLMN ID between SNPN and CH, neither between CHs. The HNI of a SUPI in NSI format shall identify the subscribed SNPN. The SUPI type for SNPN access with CH using AAA is also specified (i.e. NSI based). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An SNPN-enabled UE can not access SNPNs different from the one it is subscribed to. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

#### 5.30.2.3	UE configuration and subscription aspects

An SNPN-enabled UE is configured with the following information for each subscribed SNPN:

-	PLMN ID and NID of the SNPN;

-	Subscriber identifier (SUPI) and credentials;

-	Optionally, an N3IWF FQDN and an identifier of the country where the configured N3IWF is located;

-	Optionally, if the UE supports access to an SNPN using credentials from a Credentials Holder:

-	User controlled prioritized list of preferred SNPNs;

-	Credentials Holder controlled prioritized list of preferred SNPNs;

-	Credentials Holder controlled prioritized list of GINs.

The Credentials Holder controlled prioritized lists of preferred SNPNs and GINs can be updated by the Credentials Holder.

Editor's note:	Whether the Credentials Holder controlled lists can be updated using the UE Parameters Update (UPU) via UDM Control Plane Procedure as defined in TS 23.502 [3] clause 4.20 or the Steering of Roaming (SoR) procedure as defined in TS 23.122 [17] Annex C will be determined based on feedback from CT WG1 and SA WG3.

A subscriber of an SNPN is either:

-	identified by a SUPI containing a network-specific identifier that takes the form of a Network Access Identifier (NAI) using the NAI RFC 7542 [20] based user identification as defined in TS 23.003 [19] clause 28.7.2. The realm part of the NAI may include the NID of the SNPN; or

-	identified by a SUPI containing an IMSI.

In the case of access to an SNPN using credentials owned by a Credentials Holder using AUSF/UDM as specified in clause 5.30.2.9.3, the SUPI shall also contain identification for the Credentials Holder, i.e. the realm in the case of Network Specific Identifier based SUPI or the MCC and MNC in the case of an IMSI based SUPI as follow:

-	When IMSI based SUPI is used, the IMSI shall be globally unique as defined in TS 23.003 [19], the SNPN shall use a different PLMN ID than the Credentials Holder, and the Credentials Holder shall not use a PLMN ID that is used by another network.

-	When Network Specific Identifier based SUPI is used, the realm of the SUPI shall uniquely identify the Credentials Holder as defined in TS 23.003 [19].

NOTE 1:	When Credentials Holder is an SNPN, and the MCC and MNC of the SNPN is not unique, then IMSI based SUPI is not supported as the MCC and MNC need not be unique always; instead USIM credentials are supported using Network Specific Identifier based SUPI.

NOTE 2:	Network Specific Identifier based SUPI is not supported for the case the Credentials Holder is provided by a PLMN.

In the case of access to an SNPN using credentials owned by a Credentials Holder using AAA Server as specified in clause 5.30.2.9.2, the SUPI shall contain a Network Specific Identifier.

An SNPN-enabled UE that supports access to an SNPN using credentials from a Credentials Holder and that is equipped with a PLMN subscription may additionally be configured with the following information for SNPN selection and registration using the PLMN subscription in SNPN access mode:

-	User controlled prioritized list of preferred SNPNs;

-	Credentials Holder controlled prioritized list of preferred SNPNs;

-	Credentials Holder controlled prioritized list of preferred GINs.

The Credentials Holder controlled prioritized lists of preferred SNPNs and GINs can be updated by the Credentials Holder.

Editor's note:	Whether the Credentials Holder controlled lists can be updated using the UE Parameters Update (UPU) via UDM Control Plane Procedure as defined in TS 23.502 [3] clause 4.20 or the Steering of Roaming (SoR) procedure as defined in TS 23.122 [17] Annex C will be determined based on feedback from CT WG1 and SA WG3.

*** End of Changes ***