3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2104417r02

Elbonia, May 17 –28, 2021	(revision of S2-2103241, 2473r03)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0291 | rev | 3 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NWDAF discovery and selection based on ML model information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | See discussion in S2-2102477. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the NWDAF registers the Nwdaf_MLModelProvision services when it has trained ML model(s) for the Analytics ID. - Add the description of NWDAF discovery and selection based on the Analytics Filter and the Target of Analytics Reporting for the trained ML model per Analytics ID(s), if available. - Delete the “determine to generate an initial ML model for the subscription” in the ML model provisioning procedure to align with the 2 bullets above. - Clarify the content of the Analytics Filter and the Target of Analytics Reporting for the trained ML model. |  |  |  |  |  |  |  |  |
|  |  | l |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ML model consumer could not discovery and select a NWDAF(MTLF) accuratly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 5.2, 6.2A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 5.1	General

The NWDAF provides analytics to 5GC NFs, and OAM as defined in clause 7. An NWDAF can be decomposed into:

-	Analytics logical function (AnLF): An NWDAF containing the Analytics logical function, denoted as NWDAF(AnLF), can perform inference, derive analytics information (i.e. derives statistics and/or predictions based on Analytics Consumer request) and expose analytics service i.e. Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo.

-	Model Training logical function (MTLF): An NWDAF containing the Model Training logical function, denoted as NWDAF(MTLF), trains ML models and exposes new training services (e.g. providing trained model) as defined in clause 7.

NOTE 1:	NWDAF can contain Model Training logical function, Analytics logical function, or both. An NWDAF contains both logical functions and is denoted as NWDAF, unless explicitly mentioned as NWDAF(AnLF) and NWDAF(MTLF).

NOTE 2:	Pre-trained ML model storage and provisioning to NWDAF is out of the scope of 3GPP.

NOTE 3:	Sharing of models or model meta data is limited to single vendor environments in this Release of the specification.

Analytics information are either statistical information of the past events, or predictive information.

Different NWDAF instances may be present in the 5GC, with possible specializations per type of analytics. The capabilities of a NWDAF instance are described in the NWDAF profile stored in the NRF.

In order to support NFs to discover and select an NWDAF instance containing MLTF, AnLF, or both, that is able to provide the required service (e.g. analytics exposure or ML model provisioning) for the required type of analytics, each NWDAF instance should provide the list of supported Analytics ID(s) (possibly per supported service) when registering to the NRF, in addition to other NRF registration elements of the NF profile. NFs requiring the discovery of an NWDAF instance that provides support for some specific service(s) for a specific type of analytics may query the NRF for NWDAFs supporting the required service(s) and the required Analytics ID(s).

NOTE X:	The NWDAF(MTLF) registers the ML model provisioning service when it has trained ML model(s) for the Analytics ID.

The consumers i.e. 5GC NFs and OAM decide how to use the data analytics provided by NWDAF.

The interactions between 5GC NF(s) and the NWDAF take place within a PLMN.

The NWDAF has no knowledge about NF application logic. The NWDAF may use subscription data but only for statistical purpose.

The NWDAF architecture allows for arranging multiple NWDAF instances in a hierarchy/tree with a flexible number of layers/branches. The number and organisation of the hierarchy layers, as well as the capabilities of each NWDAF instance remain deployment choices.

In a hierarchical deployments, NWDAFs may provide data collection exposure capability for generating analytics based on the data collected by other NWDAFs, when DCCF, MFAF are not present in the network.

# * * * * Next change * * * *

## 5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information and required analytics capabilities and/or requested Machine Learning (ML) model information by using the NWDAF discovery principles defined in clause 6.3.13, TS 23.501 [2].

If the required analytics is related to NF(s) (e.g. NF load analytics) and from the NWDAF discovery response it is not clear which NWDAF can provide such analytics, the NWDAF service consumer should select an NWDAF with analytics aggregation capability, preferably with PLMN wider serving area, if available.

If the required analytics is related to UE(s) (e.g. UE mobility analytics) and the NWDAF service consumer (other than an NWDAF) can not provide an Area of Interest for the requested data analytics, it should select an NWDAF with analytics aggregation capability, preferably with PLMN wide serving area, if available.

If no such NWDAF exists, the consumer selects an NWDAF from the NWDAF discovery response returned by the NRF.

NOTE 1:	If the requested analytics is related to UE(s) and the selected NWDAF can not provide analytics for the UE(s) (e.g, the NWDAF serves a different serving area), the selected NWDAF queries the NRF with the tracking area where the UE is located to discover another target NWDAF serving the area where the UE(s) is located and

-	(if AnalyticsInfo_Request) redirect the AnalyticsInfo_Request to that target NWDAF, or

-	(if AnalyticsSubscription_Subscribe) trigger an analytics subscription transfer (see clause 6.1A) to that target NWDAF.

Editor’s note: How the selected NWDAF and / or NRF is aware of the tracking area where the UE is located is FFS.

Editor’s note:	If multiple NWDAFs are serving the same tracking area, it is FFS to determine which one should be selected.

Editor's note:  It is FFS whether the selected NWDAF can query UDM to obtain the target NWDAF.

NOTE 2:	If the requested analytics is related to NF(s) and the selected NWDAF can not provide the requested data analytics, e.g. due to the NF(s) to be contacted being out of serving area of the NWDAF, the selected NWDAF queries the NRF with the service area of the NF to be contacted to determine another target NWDAF and

-	(if AnalyticsInfo_Request) redirects the AnalyticsInfo_Request to that target NWDAF, or

-	(if AnalyticsSubscription_Subscribe) trigger an analytics subscription transfer (see clause 6.1A) to that target NWDAF.

Editor’s Note: It is FFS if other mechanism can be used to determine the target NWDAF, e.g. other parameters in the query to NRF.

NOTE 3: 	Procedures above is only applicable when the target of analytics reporting is a UE or group of UEs. If the target of analytics reporting is any UE, analytics filter on the area of interest is mandatory from the NWDAF service consumer.

In order to discover an NWDAF(MTLF):

-	During the NWDAF(MTLF) registration, an NWDAF(MTLF) may include in the registration request to NRF, the Analytics Filter information as described in clause 6.2A.2 for the trained ML model per Analytics ID(s), if available.

-	During the NWDAF(MTLF) discovery, the NRF may return one or more candidate NWDAF(MTLF) instance(s) to the NF consumer and each candidate NWDAF(MTLF) instance includes the Analytics Filter information for the trained ML model per Analytics ID(s) , if available .

# * * * * Next change * * * *

### 6.2A.1	ML Model Subscribe/Unsubscribe

The procedure in Figure 6.2A.1-1 is used by an NWDAF service consumer, i.e. an NWDAF (AnLF) to subscribe/unsubscribe at another NWDAF, i.e. an NWDAF containing MTLF, to be notified when ML model information on the related Analytics becomes available, using Nnwdaf_MLModelProvision Services as defined in clause 7.5. The ML model information is used by an NWDAF containing AnLF to derive analytics. The service is also used by an NWDAF to modify existing ML Model Subscription(s). An NWDAF (e.g. an NWDAF (MTLF+AnLF)) can be at the same time a consumer of this service provided by other NWDAF(s) and a provider of this service to other NWDAF(s).

Figure 6.2A.1-1: ML Model for analytics subscribe/unsubscribe

NOTE:	In Figure 6.2A.1-1 an NWDAF service consumer may host both an Analytics Logical Function and a Model Training Logical Function.

1.	The NWDAF service consumer (i.e. an NWDAF (AnLF)) subscribes to, modifies, or cancels subscription for a (set of) ML Model(s) associated with a (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelProvision_Unsubscribe service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.2A.2.

When a subscription to an ML model information for the Analytics is received, the NWDAF containing MTLF may:

-	determine whether an existing trained ML Model can be used for the subscription; or

-	determine whether triggering further training for an existing trained ML models is needed for the subscription

.

If the NWDAF containing MTLF determines that further training is needed, this NWDAF may initiate data collection from NFs, DCCF, or OAM as described in clause 6.2, required for the training of the ML model.

If the service invocation is for a subscription modification or subscription cancelation, the NWDAF 1 includes an identifier (Subscription Correlation ID) to be modified in the invocation of Nnwdaf_MLModelProvision_Subscribe.

2.	If the NWDAF service consumer subscribes to a (set of) ML model(s) associated to a (set of) Analytics ID(s), the NWDAF containing MTLF notifies the NWDAF service consumer with the ML model information (containing the file address of the ML model) by invoking Nnwdaf_MLModelProvision_Notify service operation. The content of ML model information that can be provided by the NWDAF containing MTLF is specified in clause 6.2A.2.

The NWDAF containing MTLF also invokes the Nnwdaf_MLModelProvision_Notify service operation to notify the available of a re-trained ML model when the NWDAF containing MTLF determines that the previously provided trained ML Model required re-training at step 1.

# * * * * End of changes * * * *