3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2104463

# Elbonia, May 17 – May 28, 2021                                                     (revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2927 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of MUSIM capability exchage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, LG Electronics, Nokia, Nokia Shanghai Bell, Intel, Samsung, Apple, Spreadtrum, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MUSIM |  |  |  |  |  | Date: |  |  | 2021-05-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Intoduction of MUSIM capability exchange. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In order to use the MUSIM features that defined in clause 4.3.x, UE and network need to exchange the supported capability as below. 1. Connection Release UE may provide this support capability to AMF, AMF can use this capability together with 3) and 5) to decide whether support paging cause for the UE or not. If AMF supports this feature, AMF will send this support capability to UE. 2. Paging Cause Because paging cause for voice is only applied to the MUSIM UE, UE need to provide this capability to AMF, AMF will decide whether apply the paging cause in the following paging procedure for the UE. If AMF supports this feature, AMF will send this capability to UE. 3. Reject paging UE provides this capability to AMF, AMF can use this capability togheter with 1) and 4) to decide whether apply paging cause for the UE or not. If AMF supports this feature, AMF will send this capability to UE. 4. Paging restriction UE provides this capability as 1) and 3). If AMF supports this feature, AMF will send this capability to UE. According to the above, UE provides the capability 1)-4) if UE supports the related features. AMF will provide the capabilities 1)-4) is AMF supports the capabilities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new agreed feature from the FS_MuSIM is not introduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** BEGIN 1st CHANGE *****

5.4.4a	UE MM Core Network Capability handling

The UE MM Core Network Capability is split into the S1 UE network capability (mostly for E-UTRAN access related core network parameters) and the UE 5GMM Core Network Capability (mostly to include other UE capabilities related to 5GCN or interworking with EPS) as defined in TS 24.501 [47] and contains non radio-related capabilities, e.g. the NAS security algorithms etc. The S1 UE network capability is transferred between all CN nodes at AMF to AMF, AMF to MME, MME to MME, and MME to AMF changes. The UE 5GMM Core Network Capability is transferred only at AMF to AMF changes.

In order to ensure that the UE MM Core Network Capability information stored in the AMF is up to date (e.g. to handle the situation when the USIM is moved into a different device while out of coverage, and the old device did not send the Detach message; and the cases of inter-RAT Registration Area Update), the UE shall send the UE MM Core Network Capability information to the AMF during the Initial Registration and Mobility Registration Update procedure within the NAS message.

The AMF shall store always the latest UE MM Core Network Capability received from the UE. Any UE MM Core Network Capability that an AMF receives from an old AMF/MME is replaced when the UE provides the UE MM Core Network Capability with Registration signalling.

If the UE's UE MM Core Network Capability information changes (in either CM-CONNECTED or in CM-IDLE state), the UE shall perform a Mobility Registration Update procedure when it next returns to NG-RAN coverage. See clause 4.2.2 of TS 23.502 [3].

The UE shall indicate in the UE 5GMM Core Network Capability if the UE supports:

-	Attach in EPC with Request type "Handover" in PDN CONNECTIVITY Request message (TS 23.401 [26], clause 5.3.2.1).

-	EPC NAS.

-	SMS over NAS.

-	LCS.

-	5G SRVCC from NG-RAN to UTRAN, as specified in TS 23.216 [88].

-	Radio Capabilities Signalling optimisation (RACS).

-	Network Slice-Specific Authentication and Authorization.

-	Parameters in Supported Network Behaviour for 5G CIoT as described in clause 5.31.2.

-	Receiving WUS Assistance Information.

-	CAG, see clause 5.30.3.3.

A UE operating two or more USIMs and that supports and intends to use one or more Multi-USIM features in a PLMN for a USIM, shall indicate in the UE 5GMM Core Network Capability for this USIM in this PLMN that the UE supports the corresponding Multi-USIM specific features (see clause 5.x)for this USIM by indicating it supports one or more of the following MUSIM features:

-	Connection Release Supported.

-	Paging Cause Supported.

-	Paging Restriction Supported.

-	Reject Paging Supported.

When the UE intends to stop using any (or all) of these Multi-USIM feature(s) for a USIM in a PLMN, it shall indicate it not supports this (or all)  Multi-USIM feature(s) in the UE 5GMM Core Network Capability for this USIM in this PLMN. When the UE supporting Multi-USIM features doesn’t operate two or more USIMs anymore, it shall indicate in the UE 5GMM Core Network Capability that all the Multi-USIM features are not supported.

***** END OF CHANGES *****