3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2104537

Elbonia, May 17 – 28, 2021	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2939 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to scenarios of external authentication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current specification, it is specified as below. “After completing the primary authentication with the AAA server successfully, the AMF and SMF shall retrieve the UE subscription data from UDM using SUPI”. However, when the AMF and SMF have subscription data for UE already, the AMF/SMF shall not retrieve the UE subscription data again. For example, AMF already retrieve subscription during registartion procedure, the AMF shall not retrieve it again during service request procedure after primary authentication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct that the AMF and SMF shall retrieve the UE subscription data from UDM using SUPI if AMF and SMF does not have subscription data for the UE. - Some clarification in the general description clause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AMF and SMF retrieve subscription redundently. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.30.2.9	SNPN connectivity for UEs with credentials owned by Credentials Holder

##### 5.30.2.9.1	General

SNPNs may support UE access using credentials owned by a Credentials Holder separate from the SNPN. In this case the Session Management procedures (i.e. PDU Sessions) terminate in an SMF in the SNPN.

Two architectures of SNPN connectivity for UEs with credentials owned by Credentials Holder are supported. Credentials Holder using AAA Server for primary authentication and authorization is described in clause 5.30.2.9.2 and Credentials Holder using AUSF and UDM for primary authentication and authorization is described in clause 5.30.2.9.3.

##### 5.30.2.9.2	Credentials Holder using AAA Server for primary authentication and authorization

The AUSF in SNPN may support primary authentication and authorization of UEs that use credentials from an AAA Server in a Credentials Holder (CH).

-	If the UDM instructs AUSF that primary authentication by a AAA server in a CH is required, the AUSF shall discover and select the AAA server, and then forward EAP messages to the AAA server which is acting as the EAP Server for the purpose of primary authentication.

-	The SUPI is used to identify the UE during primary authentication and authorization towards the AAA sever. SUPI privacy is achieved according to existing methods in Annex I.5 of TS 33.501 [29].

-	After completing the primary authentication with the AAA server successfully, the AMF retrieves the UE subscription data from UDM using SUPI as described in TS 23.502 [3] clause 4.2.2.2.2.

Figure 5.30.2.9.2-1 depicts the 5G System architecture for SNPN with Credentials Holder using AAA Server for primary authentication and authorization.

Figure 5.30.2.9.2-1: 5G System architecture with access to SNPN using credentials from Credentials Holder using AAA Server

Editor's note:	Whether an intermediate function is needed between the AUSF and the AAA-S is FFS.

Editor's note:	The Interface between AAA Server and SNPN is FFS.

##### 5.30.2.9.3	Credentials Holder using AUSF and UDM for primary authentication and authorization

Figure 5.30.2.9.3-1 depicts the 5G System architecture for SNPN with Credentials Holder using AUSF and UDM for primary authentication and authorization.

NOTE:	The architecture for SNPN and Credentials Holder is depicted as a non-roaming reference architecture as the UE is not considered to be roaming, even though some of the roaming architecture reference points are also used.

Figure 5.30.2.9.3-1: 5G System architecture with access to SNPN using credentials from Credentials Holder using AUSF and UDM

# * * * * End of changes * * * *