3GPP TSG-WG SA2 Meeting #145E e-meeting  	S2-2104546r04

# Elbonia, May 17 – 28, 2021	(revision of S2-2102980)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2832 | rev | 4 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE configuration for remote provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It is to refect the UE configuration in the conclusions for “Remote provisioning for SNPN credentials (Component 2 of KI#4)” in clause 8.4.1 of TR23.700-07, Onboarding configuration is introduced to User Plane Remote Provisioning for SNPN credentials. In addition to S2-2102980, some more reason for changes are given: - Current spec describe that SMF shall send the PVS address to UE during remote provisioning. However, 3rd party may verify the UE to decide whether the PVS address can be sent or not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the UE configuration for UP Remote Provisioning. In addition to S2-2102980, some more changes are given: - It is proposed that the SMF may trigger a secondary authentication before sending the PVS FQDN and/or PVS IP address(es) to ensure the UE has right to access the PVS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE configuration for Onboarding configuration is not introduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.X.2.7.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The UE constructed FQDN for PVS discovery according to 3GPP rules are expected to be defined in TS 23.003. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Start of changes *****

5.30.2.X.4	Remote Provisioning of UEs in Onboarding Network

5.30.2.X.4.3	User Plane Remote Provisioning of UEs when Onboarding Network is a ON-SNPN

In case Onboarding Services are provided using a restricted PDU Session for remote provisioning of UE via User Plane, the AMF selects an SMF used for Onboarding Services using the SMF discovery and selection functionality as described in clause 6.3.2. The AMF Onboarding Configuration Data may contain S-NSSAI(s) and DNN(s) used for Onboarding to select an SMF used for Onboarding Services or may contain a configured SMF for the DNN used for Onboarding.

When a UPF is selected for Onboarding Services, the UPF selection function described in clause 6.3.3 for normal services is applied considering the DNN used for Onboarding.

The SMF or the PCF may store S-NSSAI and DNN information used for Onboarding. Onboarding Configuration Data available to PCF (for details see TS 23.503 [45]) and/or SMF may include PVS FQDN and PVS IP address(es).

When the UE registered for Onboarding successfully completes the user plane remote provisioning of SNPN credentials via the Onboarding Network, then the UE should deregister from the Onboarding Network.

Initial QoS parameters used for establishing Onboarding Services are configured in the SMF when dynamic PCC is not used.

Dynamic PCC may be used for a PDU session that is established for Onboarding Services as described in TS 23.503 [45]).

The QoS Flows of a PDU Session associated with the restricted DNN shall be dedicated to Onboarding Services. The SMF may configure PDR and FAR including PVS and DNS server IP addresses for the UPF to block any traffic that is not from or to PVS and DNS server addresses.

If the UE is registered for Onboarding, the network should apply S-NSSAI and DNN used for Onboarding for the PDU Session Establishment request from the UE.

***** Second changes *****

5.30.2.X.4.4	User Plane Remote Provisioning of UEs when Onboarding Network is a PLMN

Onboarding Services for a UE may be provided via PLMN using PDU Session  for DNN(s)/S-NSSAI(s) used for remote provisioning. Subscription of such a UE shall contain the DNN/S-NSSAI used for remote provisioning.

The AMF selects an SMF used for remote provisioning using the SMF discovery and selection functionality as described in clause 6.3.2, considering the DNN/S-NSSAI used for remote provisioning provided by the UE or the default DNN/S-NSSAI provided by UDM.

The UPF selection function described in clause 6.3.3 is applied, considering the DNN/S-NSSAI used for remote provisioning.

The SMF may be configured with one or more PVS FQDN and/or PVS IP address(es) per DNN/S-NSSAI used for remote provisioning. The SMF may send the PVS FQDN and/or PVS IP address(es) associated to the DNN/S-NSSAI of the PDU Session to the UE via PCO during PDU Session establishment procedure. Based on local policy, the SMF sends the PVS FQDN and/or PVS IP address(es) to the UE if it determines the PDU Session for the UE is allowed.

***** Third changes *****

5.30.2.X.4.5	Control Plane Remote Provisioning of UEs

Editor’s Note:	If to be supported, this clause should specify the control plane remote provisioning procedures of UEs.

***** End of changes *****