3GPP TSG-WG SA2 Meeting #145E e-meeting  	S2-2104573

# Elbonia, May 17 – 28, 2021	(revision of S2-2103480)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0543 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#5 – Total bandwidth per S-NSSAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, KPN, Nokia, Nokia Shanghai Bell, Xiaomi, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Include conclusions on KI#5 in TR 23.700-40 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define new policy control data on slice level that includes the Maximum data rate per S-NSSAI that . is provisioned by OAM Define how PCF monitors the aggregate data rate in the slice that includes to aggregate the MBR for each PDU session in that slice. Define how to enforce the Maximum Data rate in the slice, that implies that no new PDU session or QoS flows are established or changes in the existing QoS flows may happen depending on a policy decision in the PCF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification of the conclusions in KI#5. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.y (new), 6.2.1.y.1 (new), 6.2.1.y.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 2 aligns terminology with other related CRs and updates the description of the PCF enforcement actions. |  |  |  |  |  |  |  |  |

---------------------------------------First Change------------------------------------------------

---------------------------------------Next Change------------------------------------------------

#### 6.2.1.y	Monitoring the data rate per Network Slice

##### 6.2.1.y.1	General

The PCF supports monitoring and limitation of the  aggregate data rate per S-NSSAI.

If the S-NSSAI is subject todata rate limitation the Remaining Data Rate per S-NSSAI is stored in the UDR per PCF as Data Set "Policy Data" and Data Subset “Remaining Data Rate” as defined in clause 6.2.1.3.

The PCF interacts with the UDR to retrieve and update its the Remaining Data Rate per S-NSSAI. The usage of multiple PCFs for the same S-NSSAI is enabled by managing the Remaining Data Rate for the S-NSSAI per PCF.

##### 6.2.1.y.2	Monitoring the data rate per Network Slice by using QoS parameters

For the purpose of monitoring the data rate per S-NSSAI, the PCF shall perform the following for an SM Policy Association with an S-NSSAI subject to data rate limitation:

-	At the establishment of a SM Policy Association, the PCF shall check whether the Remaining Data Rate is higher than the Authorized Session-AMBR. If so, the PCF shall deduct the value of the Authorized Session-AMBR allocated to the PDU Session from the remaining data rate for that S-NSSAI in the UDR. If the Remaining Data Rate is not sufficient, the PCF shall reject the establishment of the SM Policy Association.

-	At the time a PCC Rule of a GBR service data flow is installed, modified or activated in the SMF, the PCF requests reporting “Successful resource allocation” with the corresponding Policy Control Request Trigger. When the SMF reports that the resources are successfully allocated, the PCF shall deduct the value of the MBR authorized for the GBR service data flow from the Remaining Data Rate for that S-NSSAI in the UDR.

-	At the time a change of Authorized Session-AMBR occurs or a PCC Rule containing a GBR value is removed or modified, the PCF shall update the Remaining Data Rate for that S-NSSAI in the UDR accordingly.

-	If the Remaining Data Rate for that S-NSSAI reaches a (operator defined) threshold that indicates that it is closer to zero, the PCF may apply a policy decision to strengthen the traffic restrictions for individual PDU Sessions or PCC rules (e.g. reject new SM Policy Associations, reject new GBR service data flows with high GBR requirements, change Authorized Session-AMBR values (if allowed by the HPLMN), change MBR values in PCC rules belonging to GBR service data flows) and interact with the SMF accordingly. When the Remaining Data Rate for that S-NSSAI increases again, the PCF may relax the traffic restrictions for individual PDU Sessions or PCC rules.

NOTE:	It is recommended to avoid frequent policy decisions which trigger a signalling with the UE (like change of Authorized Session-AMBR or change of MBR in a PCC rule belonging to a GBR service data flow).

-	At the time the SM Policy Association for PDU sessions or GBR service data flows associated to an  an S-NSSAI subject to data rate limitation is terminated, the PCF shall add the value of their Authorized Session-AMBR or the MBR n to the Remaining Data Rate for that S-NSSAI in the UDR.

---------------------------------------End of Changes---------------------------------------------