3GPP TSG SA WG2 Meeting #145E (e-meeting)		S2-2104780r06

May 17-28 2021

Title:	[DRAFT] LS Response to LS on multiple TACs per PLMN

Response to:	S2-2103770 / R2-2104377

Release:	Rel-17

Work Item:	5GSAT_ARCH

Source:	SA2

To:	RAN2, CT1

Cc:	RAN3

Contact person:	Stephen Edge

sedge AT qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2104774, S2-2104777

1	Overall description

SA2 thanks RAN2 for the LS on multiple TACs per PLMN. SA2 notes that RAN2 has included the following information and a question in this LS:

- The network may broadcast more than one TACs per PLMN in a cell.

- Option 1: AS still reports only one TAC for one PLMN even if more than one TACs per PLMN are broadcasted in an NTN cell.

- Option 2: AS indicates all received TAC(s) for one PLMN to NAS layer.

- Majority of companies think option 1 has larger RAN2 impact, as AS layer has to select one TAC from the broadcast ones to report to NAS layer, thus RAN2 has preference for option 2

- Is option 2, i.e. “AS indicates all received TAC(s) for one PLMN to NAS layer”, feasible from a NAS layer perspective?

SA2 has discussed the two options mentioned in the RAN2 LS. SA2 believes that Option 2 is feasible to support UE reachability and paging, although there is an impact to the NAS layer but this is also something CT1 needs to analyse from stage 3 perspective.

For Option 1, SA2 tentatively agrees that reporting of only one TAC for one PLMN by AS to NAS would require no change to NAS. However, there may be drawbacks with this approach as it is not clear:

- whether the reported TAC reflects the real UE location

- whether and how the NAS layer could be informed of other TACs, e.g. in the case that a reported TAC is not in a UE registration area but another unreported TAC is.

In addition to reachability and paging, there are other aspects related to supporting multiple TACs per cell that require further study, such as how Service Area Restrictions and Forbidden Areas would be supported.

SA2 will further analyse the options, and also take any CT1 reply into consideration, and provide further feedback when available.

2	Actions

To RAN2 and CT1

ACTION: 	SA2 asks RAN2 and CT1 to take the above feedback into account.

3	Dates of next TSG SA WG2 meetings

SA2#146E		16-27 August 2021		Electronic Meeting

SA2#147E		18-22 October 2021		Electronic Meeting