SA WG2 Meeting #S2-146E	S2-2106366r02

16 August – 27 August 2021, Electronic, Elbonia	(revision of S2-210xxxx)

Source:	ZTE

Title:	update the clause 7.2.6 Multicast session update procedure

Document for:	Agreement

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract of the contribution: This paper update the clause 7.2.6 Multicast session update procedure to resolve the ENs.

# 1. Proposal

It proposes to adopt the following change.

- AMF store the RAN ID. The MB-SMF does not send the RAN ID to AMF. Remove the related EN.

- Remove the EN on “Updating the associated PDU Session's QoS based on the update of MBS Session QoS by NG-RAN requires RAN WGs collaboration” In the step 8, the PDU session modification will be triggered if the QoS parameters other than ARP needs to be updated.

## *****************************************Start of Change**********************************************

- 7.2.6	Multicast session update procedure

Multicast session update procedure is invoked by the AF to update the ARP for an ongoing multicast session. For the interaction between AF and MB-SMF, see clause 7.1.1.5 and 7.1.1.6.

Figure 7.2.6-1: Multicast session update procedure.

1.	This procedure is triggered by the MB-SMF receiving the updated policy for MBS, see clauses 7.1.1.5 and 7.1.1.6.

2.	The MB-SMF generates the QoS profile for the multicast MBS session, and invokes Namf_MBSCommunication_N2MessageTransfer service operation (MBS Session ID, N2 SM information (TMGI, QoS profiles for multicast)) to the AMF(s).

3.	The involved AMF sends N2 MBS session request (N2 SM information) to NG-RAN nodes based on the NG-RAN node IDs stored in the AMF for the MBS session.

4.	NG-RAN updates the MBS session context, and if only the ARP of QoS parameters is updated, NG-RAN node also updates the QoS parameters of the associating PDU Sessions.

5.	The NG-RAN(s) acknowledges N2 MBS session request by sending an N2 MBS session Response message to the AMF.

6.	The AMF uses the Namf_MBSCommunication_N2MessageTransfer reply to the MB-SMF.

7.	MB-SMF sends Nmbsmf_MBSSession ContextStatusNotify request (TMGI, QoS profiles for multicast) to SMF.

8.	SMF determines the UE list regarding the TMGI included in the message. If the individual delivery is used, or the QoS parameters other than ARP needs to be updated, SMF triggers PDU Session Modification procedure for each UE as defined in TS 23.502 [6].

## *******************************************End of Change**********************************************