3GPP TSG|WG-SA WG2 Meeting #147E	S2-2107273

e-Meeting, 18-22 October 2021

Source:	China Mobile

Title:	New WID on 5G Architecture Enhancements for RAN Slicing for NR

Document for:	Approval

Agenda Item:	9.1.1

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

Title:	5G Architecture Enhancements for RAN Slicing for NR

Acronym:	ARCH_NR_Slice

Unique identifier: TBD

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent and child Work Items

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one but are not part of the hierarchical structure.}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 860039 | Study on enhancement of RAN slicing for NR | RAN work for slice list and priority information for cell reselection |

3	Justification

RAN2 and RAN3 carried out a Rel-17 study item on the enhancement of RAN Slicing for NR. The analysis and conclusions for scenarios, key issues and candidate solutions are captured in TR 38.832. The RAN2 part of the study was completed in Q1 2021 with a conclusion that both slice based cell reselection and RACH configuration are recommended for normative work.

TSG RAN has approved work item "Core part: Enhancement of RAN slicing for NR (NR_Slice-Core)" in TSG RAN Meeting #91(March 2021) to standardize the enhancement on RAN support of network slicing. RAN2 sent LS to SA2 in R2-2108928 to point out the solution agreed in RAN2#114e and #115e for cell reselection for the normative phase has the following impacts on NAS protocol:

For the List of Slices with Slice Priority, UE Access Stratum (AS) expects to receive a list from NAS containing a slice priority for each of the slices contained in the list when/ before it moves to RRC_IDLE/RRC_INACTIVE and when the list and/or priorities changes while the UE is in RRC_IDLE/RRC_INACTIVE.

Furthermore, RAN2 has been discussing a Slice Group concept, where a slice group consists of one or multiple slices, one slice belongs to one and only one slice group and each slice group is uniquely identified by a slice group identifier. This can avoid publishing slice identities (S-NSSAI) in System Information (security concern and SI size concern).

On the other hand, SA2 has started a Rel-17 work item “Study on Enhancement of Network Slicing Phase 2”, which focuses on the feasibility of further enhancement on network slicing. The current scope of eNS_Ph2 doesn’t cover the requirement of RAN2 for slice based cell reselection.

Considering the above, the work for architecture enhancement to support the slice priority and slice group information broadcasting for cell (re)selection should be carried out within Rel-17 in SA2, and it is suggested not to put this work in Rel-18, since RAN specification has carried out the work in Rel-17.

- Objective

The objective of the work item is to develop the specifications according to the slice based cell reselection requirements from RAN WGs work item NR_Slice-Core. The following areas of work are expected to be covered (non-exhaustive):

- Specify how the 5GC obtains and sends the Slice Priority information to the UE.

- Specify  how the NG-RAN obtains the information about the S-NSSAI(s) and corresponding Slice Group Identifiers including:

  - whether and how the 5GC NF determins the relationship between Slice Group Identifiers and S-NSSAI(s).

- Specify how the UE get the S-NSSAI(s) and corresponding Slice Group Identifiers through NAS signalling.

- Specify  when and how to update the S-NSSAI(s) and corresponding Slice Group Identifiers in UE,e.g. UE mobility or configuration udpate.

NOTE 1: the SA2 work may need to be coordinated with RAN WGs.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Proposed Spec no. or series | Type (see note 4) | Rapporteur(s) | For info at TSG# | For approval at TSG# | Remarks |

Note 4:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
“Internal TR” is intended for 3GPP internal use only whereas “External TR” may be transposed by Ops.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |
| --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# |
| 23.501 | Specify system enhancements to support of slice list , priority information and its signalling |  |
| 23.502 | Specify system enhancements to support of slice list , priority information and its signalling |  |

6	Work item Rapporteur(s)

Dan Wang, China Mobile

wangdanyjy@chinamobile.com

7	Work item leadership

SA2.

8	Aspects that involve other WGs

RAN2, RAN3, and CT1

9	Supporting Individual Members

Supporting IM name

China Mobile

ZTE