3GPP TSG-SA2 Meeting #147E (e-meeting)	S2-2107313

# Oct 18 – 22, 2021											 	 	                         (revision of S2-210XXXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3277 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on remote provisioning of credentials for NSSAA or secondary authentication/authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For remote provisioning of credentials for NSSAA or secondary authentication/authorization, the UP Remote Provisioning are incomplete and CP Remote Provisioning is still missing. In previous meeting, it is argued whether the UE may be configured with S-NSSAI without related credential. Currently, Clause 5.30.3.1 NOTE 3 clarifies that “After successful provisioning of the credentials (for NSSAA or for secondary authentication/authorization) to the UE, specific service subscription data (e.g. to enable the use of PNI-NPN) can be activated in the UE Subscription data in the UDR/UDM. This can result in a change of the UE Subscription Data to include new S-NSSAI, DNN or CAG information, which can trigger update of the UE configurations.” This implies that the credentials are provisioned to the UE before the related DNN, S-NSSAI are configured in the UE. Therefore, it is proposed to add clarification that “it is not expected to configure the UE with the new S-NSSAI, DNN or CAG information before the successful provisioning of the related credentials to the UE”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the UP Remote Provisioning and CP Remote Provisioning for remote provisioning of credentials for NSSAA or secondary authentication/authorization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition For remote provisioning of credentials for NSSAA or secondary is not incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.39.1, 5.39.3, 5.39.4, 5.30.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

5.39	Remote provisioning of credentials for NSSAA or secondary authentication/authorization

5.39.1	General

The UE and the HPLMN may provide functionalities to provision or update the credentials used for NSSAA or credentials for secondary authentication/authorization to the UE. The provisioning via UE Parameters Update procedure as defined in clause 4.20 of TS 23.502 [3] and via User Plane are both supported.

Editor's note:	It is FFS whether and how the credentials can be sent from a provisioning server (e.g. using an AF) to the UDM and whether NEF is needed.

Credentials for NSSAA or secondary authentication/authorization are associated with dedicated DNN(s) and/or S-NSSAI(s) and it can be provisioned by a network using PDU Session used for remote provisioning or using control plane signalling.

For User Plane provisioning, the UE establishes a PDU session that is used for remote provisioning, e.g. by using DNN(s)/S-NSSAI(s) which can access the provisioning server. If the SMF is configured with the PVS address(es) and/or PVS FQDN(s), the SMF shall send the address of the provisioning server per DNN/S-NSSAI to the UE via PCO during PDU Session establishment procedure. Alternatively, the UE may be configured with an address of a provisioning server or the provisioning server may subscribe for UE Reachability Notification and may use the Application Triggering procedure as specified in TS 23.502 [3] to trigger the UE to initiate the setup of connection for remote provisioning.

The UE Subscription data at the UDM should contain the DNN(s)/S-NSSAI(s), of which credentials can be provisioned by the network.

****************** NEXT CHANGE ***************

5.39.3	User Plane Remote Provisioning of UEs

If the UE is configured with a DNN and/or a S-NSSAI used for UP remote provisioning, the UE setup a PDU session with the DNN and/or the S-NSSAI used for user plane provisioning. The AMF selects an SMF used for remote provisioning using the SMF discovery and selection functionality as described in clause 6.3.2, considering the DNN/S-NSSAI of the PDU session requested by the UE.

The UPF selection function described in clause 6.3.3 is applied, considering the DNN/S-NSSAI of the PDU session requested by the UE.

The SMF may be configured with one or more PVS FQDN and/or PVS IP address(es) per DNN/S-NSSAI. The SMF may send the PVS FQDN and/or PVS IP address(es), which are associated with the UE subscribed DNN(s)/S-NSSAI(s), to the UE as part of Protocol Configuration Options (PCO) in the PDU Session Establishment Response.

5.39.4	Control Plane Remote Provisioning of UEs

Optionally, the UDR can be preconfigured with the DNN(s)/S-NSSAI(s) associated credentials for NSSAA or secondary authentication/authorization.

The UDM of the network should provide DNN(s)/S-NSSAI(s) associated credentials for NSSAA or secondary authentication/authorization to the UE via UPU procedure, if it is available.

****************** NEXT CHANGE ***************

5.30.3.1	General

Public Network Integrated NPNs are NPNs made available via PLMNs e.g. by means of dedicated DNNs, or by one (or more) Network Slice instances allocated for the NPN. The existing network slicing functionalities apply as described in clause 5.15. When a PNI-NPN is made available via a PLMN, then the UE shall have a subscription for the PLMN in order to access PNI-NPN.

NOTE 1:	Annex D provides additional consideration to consider when supporting Non-Public Network as a Network Slice of a PLMN.

As network slicing does not enable the possibility to prevent UEs from trying to access the network in areas where the UE is not allowed to use the Network Slice allocated for the NPN, Closed Access Groups may optionally be used to apply access control.

A Closed Access Group identifies a group of subscribers who are permitted to access one or more CAG cells associated to the CAG.

CAG is used for the PNI-NPNs to prevent UE(s), which are not allowed to access the NPN via the associated cell(s), from automatically selecting and accessing the associated CAG cell(s).

NOTE 2:	CAG is used for access control e.g. authorization at cell selection and configured in the subscription as part of the Mobility Restrictions i.e. independent from any S-NSSAI. CAG is not used as input to AMF selection nor Network Slice selection. If NPN isolation is desired, operator can better support NPN isolation by deploying network slicing for PNI-NPN, configuring dedicated S-NSSAI(s) for the given NPN as specified in Annex D, clause D.2 and restricting NPN's UE subscriptions to these dedicated S-NSSAI(s).

The UE and PNI-NPN may support remote provisioning of credentials for NSSAA or credentials for secondary authentication/authorization to the UE, as specified in clause 5.39.

NOTE 3:	After successful provisioning of the credentials to the UE, specific service subscription data (e.g. to enable the use of PNI-NPN) can be activated in the UE Subscription data in the UDR/UDM. This can result in a change of the UE Subscription Data to include new S-NSSAI, DNN or CAG information, which can trigger update of the UE configurations, e.g. described in clause 5.15.5.2.2. It is not expected to configure the UE with the new S-NSSAI, DNN or CAG information before the successful provisioning of the related credentials to the UE.

NOTE 4:	The UE always has subscription to the HPLMN providing the PNI-NPN and has a USIM that contains primary authentication credentials.

****************** END CHANGES ***************