3GPP TSG-SA WG2 Meeting #147E e-meeting	S2-2107319

# Elbonia, Oct 18 – 22, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1248 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction in HSSGID procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_IMSGID |  |  |  |  |  | Date: |  |  | 2021-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 23.228 has mentioned about passing HSS Group ID in SIP signalling procedure. In addition, every IMS node receiving an HSS Group ID in SIP signalling (e.g. S-CSCF and IMS AS) should store it as part of the UE information, and should use the received HSS Group ID to select an SBI capable HSS. Additionally, an IMS node receiving an HSS Group ID in SIP signalling should include it in subsequent SIP signalling related to the above procedures. If the HSS Group ID is different for originating and terminating user then it will lead to call failure Scenario: - Originating S-CSCF sends the INVITE to terminating I-CSCF. This INVITE will have HSS Group ID for the originating user - Terminating I-CSCF does S-CSCF selection by querying HSS for terminating user. Here if I-CSCF use the received HSS Group ID then it will get user not found from HSS leading to call failure Hence the proposal is not to send the HSS group ID to the terminating side. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following change is proposed: - IMS nodes should not send the HSS Group ID to the terminating side |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Call failure |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.A.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR xxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * start of 1st change * * * *

### AA.3.3.3	Handling of HSS Group ID in IMS Procedures

The HSS group ID may be transported in SIP signalling related to the following procedures:

-	Initial IMS Registration procedure.

-	IMS Terminating session establishment procedure.

-	IMS Originating session establishment procedure.

An SBI enabled I-CSCF receiving an HSS Group ID during the NRF-based HSS discovery procedure should include the HSS Group ID for transportation to the next hop in subsequent SIP signalling related to the above procedures.

In addition, every IMS node receiving an HSS Group ID in SIP signalling (e.g. S-CSCF and IMS AS) should store it as part of the UE information, and should use the received HSS Group ID to select an SBI capable HSS. Additionally, an IMS node receiving an HSS Group ID in SIP signalling should include it in subsequent SIP signalling related to the above procedures.

The HSS Group ID in SIP signalling shall not be sent to the terminating side and outside the HPLMN.

* * * * end of changes * * * *