SA WG2 Meeting #S2-147E	S2-2107331

18 - 22 October, 2021, Electronic meeting

3GPP TSG-SA3 Meeting #104-e 	S3-213035

e-meeting, 16 - 27 August 2021

Title:	[draft]Response to LS on full registration request message to be rerouted via RAN

Response to:

Release:	Rel-17

Work Item:	FS_AMFREAL_SEC

Source:	SA2

To:	SA3

Cc:	CT1, RAN3

Contact person:	Kundan Tiwari

Kundan.tiwari@india.nec.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2107332

# 1	Overall description

SA2 thanks SA3 for the LS. SA2 provides the answer to the question as below. SA2 has agreed a CR as attached capturing the clarification in the TS 23.502.

Q1: Is the full Registration Request message the complete Registration Request message that contains both clear text IEs and non-clear text IEs, as defined in clause 4.4.6. in TS 24.501?

Answer: Yes. It is the registration request message as received in the NAS PDU of Initial UE message.

Q2: What is the Initial UE message in step 7b?

Answer: The Initial UE message consists of the Initial UE message as received in the step 1 which is sent in the Initial UE message IE of the re-route message and optionally allowed NSSAI IE or Source to Target AMF Information Reroute IE as received in the re-route message.

Q3: Does the Registration Request message the initial AMF sends out refer to

- the message step 1 of 4.2.2.2.3 Registration with AMF re-allocation, or

- the complete registration request received by the initial AMF in NAS Security mode complete message when the message in step 1 is not protected as UE had no security context; and the complete registration request message in the NAS container in the message in step 1 which is protected as UE had security context?

Answer: If the initial registration request contains 5G-GUTI and is security protected then it is the message in step 1 of the 4.2.2.2.3 Registration with AMF re-allocation. If the registration request message contains SUCI then it is NAS container IE of the NAS security mode complete message

Q4: Is it feasible from SA2 perspective if the initial AMF sends the message in Step 1 of 4.2.2.2.3 for reroute?

Answer: Yes. Please refer the answer above and attached CR.

# 2	Actions

To SA2

ACTION:

SA2 kindly requests SA3 to take above answers into consideration.

# 3	Dates of next TSG SA WG 2 meetings

SA2#148-e                                 November 8-19, 2021                                       Electronic meeting