3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2107351

# Elbonia, October 18 – 22, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0451 | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify the Slice load level analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-10-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As indicated in the LS (S2-2107030/C3-214444) from CT3, the load level of the network slice instance as defined in Realease 15 and Release 16 is removed in Release 17 which is lead the backward compatibility issues. So the load level analytics should be resumed. On the other hand, in TS 23.288, for some Analytics ID such as Slice load level, NF load and so on, a new parameter i.e. “list of analytics subset” is used to indicate which analytics subset in the output analytics table should be provided by the NWDAF. However, the definition for the “list of analytics subset” parameter is missing in the Contents of Analytics Expoure in clause 6.1.3. At the same time, the CT3 LS (i.e. S2-2107030/C3-214444) is also asking how the NWDAF know which analytics subsets in the Slice Load Level analytics should be provided in the analytics result. Furthermore, currently the threshold value and matching direction are only applied to the Nnwdaf_AnalyticsSubscription_Subscribe service, however, these parameters can be also provided in the Nnwdaf_AnalyticsInfo_Request service to help NWDAF determine to response the consumer NF when the threshold value and matching direction are met. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Resume the load level of network slice instance; 2. Add the definition of “list of analytics subset” parameter into clause 6.1.3. 3. Clarify that reporting thresholds and matching direction can be provided with Nnwdaf_AnalyticsInfo_Request service operation when Analytics ID is set to "Load level information". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. Backward compatibility issues exist. 2. The definition for the “list of analytics subset” parameter is unclear. 3. It is not sure how the NWDAF determine to response the consumer NF when the consumer NF also has requirement for the reporting threshold and matching direction. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3, 6.3.3A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.1.3	Contents of Analytics Exposure

The consumers of the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operations described in clause 7 provide the following input parameters listed below.

-	A list of Analytics ID(s): identifies the requested analytics.

-	Analytics Filter Information: indicates the conditions to be fulfilled for reporting Analytics Information. This set of optional parameter types and values enables to select which type of analytics information is requested. Analytics Filter Information are defined in the analytics related clauses.

-	Target of Analytics Reporting: indicates the object(s) for which Analytics information is requested, entities such as specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from NWDAF with this subscription.

-	Related to analytic consumers that aggregate analytics from multiple NWDAF subscriptions:

-	[OPTIONAL] (Set of) NWDAF identifiers of NWDAF instances used by the NWDAF service consumer when aggregating multiple analytics subscriptions. See clause 6.1A.

-	Analytics Reporting Information with the following parameters:

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Analytics Reporting Parameters as per Event Reporting parameters defined in Table 4.15.1-1, TS 23.502 [3].

NOTE 1:	When the Analytics Reporting Parameters indicates a periodic reporting mode and the periodicity of the report is equal to or greater than the Supported Analytics Delay associated with the Analytics ID (if available) defined in clause 6.2.6.2 of TS 23.501 [2], it is expected that the periodic reporting can be provided by the NWDAF as requested.

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Reporting Thresholds, which indicate conditions on the level of each requested analytics that when reached shall be notified by the NWDAF. A matching direction may be provided such as below, above, or crossed. If no matching direction is provided, the default direction is crossed.

NOTE 2:	Reporting thresholds and matching direction can be provided with Nnwdaf_AnalyticsInfo_Request service operation when Analytics ID is set to "Load level information".

-	Analytics target period: time interval [start..end], either in the past (both start time and end time in the past) or in the future (both start time and end time in the future). An Analytics target period in the past is a request or subscription for statistics. An Analytics target period in the future is a request or subscription for predictions. The time interval is expressed with actual start time and actual end time (e.g. via UTC time). When the Analytics Reporting Parameters indicate a periodic reporting mode, the time interval can also be expressed as positive or negative offsets to the reporting time, which indicates a subscription for predictions or statistics respectively. By setting start time and end time to the same value, the consumer of the analytics can request analytics or subscribe to analytics for a specific time rather than for a time interval.

-	[OPTIONAL] Data time window: if specified, only events that have been created in the specified time interval are considered for the analytics generation.

-	Preferred level of accuracy of the analytics ("Low", "Medium", "High" or "Highest").

-	[OPTIONAL] Accuracy level per analytics subset ("Low", "Medium", "High" or "Highest"). When an accuracy level is expressed for a given analytics subset, it takes precedence for this subset over the above preferred level of accuracy. Analytics subsets are defined in the "Output Analytics" clause of applicable analytics.

-	[OPTIONAL] Dataset Statistical Properties: information in order to influence the data selection mechanisms to be used for the generation of an analytics ID, assuring that the generated analytics ID reflects the statistical characteristics of the data that are relevant for the NWDAF consumer. The following dataset statistical properties are allowed:

-	Uniformly distributed datasets, which indicates the use of data samples that are uniformly distributed according to the different aspects of the requested analytics (e.g. equivalent data samples for each UE listed as a target of analytics reporting or for S-NSSAIs included in the analytics filter information).

-	Datasets with or without outliers, which indicates that the data samples shall consider or disregard data samples that are at the extreme boundaries of the value range.

-	Time when analytics information is needed (if applicable): indicates to the NWDAF the latest time the analytics consumer expects to receive analytics data provided by the NWDAF. It should not be set to a value less than the Supported Analytics Delay of the selected NWDAF if applicable.  If the time is reached the consumer does not need to wait for the analytics information any longer, yet the NWDAF may send an error response or error notification to the consumer. "Time when analytics information is needed" is a relative time interval as the gap with respect to analytics request /subscription (e.g. "in 10 minutes").

NOTE 3:	If the Time when analytics information is needed is provided and it is less than the Supported Analytics Delay associated with the Analytics ID (if available) defined in clause 6.2.6.2 of TS 23.501 [2], it is expected that the NWDAF may not be able to treat the Analytics ID on time.

NOTE 4:	When an aggregator NWDAF receives Time when analytics information is needed from the analytics consumer with the value greater than or equal to its Supported Analytics Delay, the NWDAF should ensure the sum of time needed for the aggregation plus the Supported Analytics Delay of other NWDAFs where analytics are collected from is not beyond its registered Supported Analytics Delay when it applies analytics aggregation for related Analytics ID(s). If this cannot be ensured, the aggregator NWDAF may reject the analytics request.

-	[OPTIONAL] Maximum number of objects requested by the consumer (max) to limit the number of objects in a list of analytics per Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response.

-	[OPTIONAL] Preferred order of results when a list of analytics is returned, possibly with a criterion for identifying the property of the results to which the preferred ordering is applied.

-	[OPTIONAL] Maximum number of SUPIs (SUPImax) requested by the consumer to limit the number of SUPIs in an object. When SUPImax is not provided, the NWDAF shall return all SUPIs concerned by the analytics object. When SUPImax is set to 0, the NWDAF shall not provide any SUPI.

-	[OPTIONAL] Output strategy: indicates the relevant factors for determining when the analytics reported. The following values are allowed:

-	Binary output strategy: indicates that the analytics shall only be reported when the requested level of accuracy is reached within a cycle of periodic notification as defined in the Analytics Reporting Parameters.

NOTE 5:	If preferred level of accuracy is more important than providing an output, then the binary strategy is used so that all analytics outputs have equivalent confidence in the prediction.

-	Gradient output strategy: indicates that the analytics shall be reported according with the periodicity defined in the Analytics Reporting Parameters irrespective if the requested level of accuracy has been reached.

NOTE 6:	If having an analytics output is more important than reaching the preferred level of accuracy, then the gradient output strategy is used so that each NWDAF will timely provide the output indicating the level of accuracy at the moment of the output generation.

NOTE 7:	When no output strategy is included in the subscription, the analytics output will be generated based on the gradient strategy and includes the reached level of accuracy for the reporting period.

-	[OPTIONAL] Analytics metadata request: indicates a request from one NWDAF to another NWDAF to provide the "analytics metadata information" related to the produced output analytics. This input parameter indicates which parameters in "analytics metadata information" are required to aggregate the output analytics for the requested Analytics ID(s).

-	[OPTIONAL] A list of analytics subsets to indicate which analytics subsets in the output analytic should be provided by the NWDAF. If the list of analytics subsets are not provided, the NWDAF should provide all the output analytics information for the requested Analytics ID.

The NWDAF provides to the consumer of the Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request service operations described in clause 7, the output information listed below:

-	(Only for Nnwdaf_AnalyticsSubscription_Notify) The Notification Correlation Information.

-	For each Analytics ID the analytics information in the requested Analytics target period.

-	In addition, the following additional information:

-	Timestamp of analytics generation: allows consumers to decide until when the received information shall be used. For instance, an NF can deem a received notification from NWDAF for a given feedback as invalid based on this timestamp;

-	Validity period: defines the time period for which the analytics information is valid.

-	Probability assertion: confidence in prediction.

-	[OPTIONAL] For each Analytics ID the  Termination Request , that notifies the consumer that the subscription is cancelled due to user consent revoked

-	[OPTIONAL] Analytics metadata information: additional information required to aggregate the output analytics for the requested Analytics ID(s). This parameter shall be provided if the "Analytics metadata request" parameter was provided in the corresponding Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation.

-	Number of data samples used for the generation of the output analytics;

-	Data time window of the data samples;

-	Dataset Statistical Properties of the analytics output used for the generation of the analytics;

-	[OPTIONAL] Data source(s) of the data used for the generation of the output analytics;

-	[OPTIONAL] Data Formatting and Processing applied on the data;

-	Output strategy used for the reporting of the analytics.

-	(Only for error response or error notification) Revised waiting time: indicates to the consumer a revised waiting value for "Time when analytics information is needed". Each NWDAF may include this as part of error response or error notification to "Time when analytics information is needed" as described in clause 6.2.5. Revised waiting time is the minimum time interval recommended by NWDAF to use as "Time when analytics information is needed" for similar future analytics requests/subscriptions.

# * * * * Next change * * * *

6.3.3A	Output analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the following analytics:

-	Network Slice instance load statistics information as defined in Table 6.3.3A-1.

-	Network Slice load statistics information as defined in Table 6.3.3A-2.

-	Network Slice instance load predictions information as defined in Table 6.3.3A-3.

-	Network Slice load predictions information as defined in Table 6.3.3A-4.

Table 6.3.3A-1: Network Slice instance load statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice. |
| Network Slice instances (1…max) | List of Network Slice instance(s) within the S-NSSAI. |
| > NSI ID | Identification of the Network Slice instance. |
| > Number of UE Registrations (NOTE 1) | Number of UE registrations of the Network Slice instance (average, variance). |
| > Number of PDU Sessions establishment (NOTE 1) | Number of PDU Session establishments of the Network Slice instance (average, variance). |
| > Resource usage (NOTE 1) | The usage of assigned virtual resources currently in use for the NF instances (mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [8] clause 5.7, belonging to a particular Network Slice instance. |
| > Resource usage threshold crossings (optional) (NOTE 1) | Number of resource usage threshold crossings on the Network Slice instance provided if threshold is provided by the consumer as Analytics Filter. |
| > Resource usage threshold crossings time period (1…max) (optional) (NOTE 1) | Resource usage threshold crossing vector including time elapsed between each threshold crossing on the Network Slice instance provided that a threshold value is provided by the consumer as Analytics Filter. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

Table 6.3.3A-2: Network Slice load statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice. |
| > Number of UE Registrations (NOTE 1) | Number of UE registrations at the Network Slice (average, variance). |
| > Number of PDU sessions establishments (NOTE 1) | Number of PDU Session establishments at the Network Slice (average, variance). |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

Table 6.3.3A-3: Network Slice instance load predictions

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice. |
| Network Slice instances (1…max) | List of Network Slice instance(s) within the S-NSSAI. |
| > NSI ID | Identification of the Network Slice instance. |
| > Number of UE Registrations (NOTE 1) | Number of predicted UE registrations at the Network Slice instance. |
| > Number of PDU Sessions establishment (NOTE 1) | Number of predicted PDU Session establishments of the Network Slice instance. |
| > Resource usage (NOTE 1) | The predicted usage of assigned virtual resources for the NF instances (mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [8] clause 5.7, belonging to a particular Network Slice instance. |
| > Resource usage threshold crossings (optional) (NOTE 1) | Number of predicted resource usage threshold crossings at the Network Slice instance provided that a threshold value is provided by the consumer as Analytics Filter. |
| > Resource usage threshold crossings time period (optional) (1…max) (NOTE 1) | Predicted Resource usage threshold crossing vector including predicted time elapsed between each threshold crossing on the Network Slice instance provided that a threshold value is provided by the consumer as Analytics Filter. |
| > Probability assertion | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

Table 6.3.3A-4: Network Slice load predictions

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice. |
| > Number of UE Registrations (NOTE 1) | Predicted Number of UE registrations at the Network Slice. |
| > Number of PDU sessions establishments (NOTE 1) | Predicted Number of PDU Session establishments at the Network Slice. |
| > Probability assertion | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

NOTE:	If no NSI ID is provided as analytics filter, slice load level related output analytics are provided according to Tables 6.3.3A-2 and 6.3.3A-4. Otherwise slice instance load level related output analytics are provided according to Tables 6.3.3A-1 and 6.3.3A-3.

The NWDAF also directly reports when the load level of the Network Slice Instance, indicated by the S-NSSAI and the associated NSI ID (if applicable) in the Analytics Filter, crosses the threshold provided in the analytics subscription; in that case, if no threshold is provided in the subscription, the reporting (Notify operation) is assumed to be periodic.

# * * * * End of changes * * * *