3GPP TSG-SA WG2 Meeting #147E (e-meeting)	S2-2107354

18 – 22 Oct. 2021, Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0453 | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Per-UE Service Experience Request Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent, Tencent Cloud |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-10-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS 23.288, procedures to request service experiences for a UE has been specified. However, the procedure description is not clear since it describes that NF consumer include one Application ID. There is no need for such restriction as this procedure is not to request service experiences per application but per UE. There can be multiple applications on the UE which the NF consumer is interested in service experiences. Also, NF consumer may also be interested in the service experiences for the UE regardless of any specific application e.g. Internet data service experiences. It is necessary to add some clarifications to the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the NF consumer may include one or more Application IDs when request service experiences or request per-UE service experience regardless of any specific application. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear per-UE Service Experience Request Procedure |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.4.6	Procedures to request Service Experience for a UE

Figure 6.4.6-1: Procedure for NWDAF providing Service Experience for a UE

This procedure is similar to the procedure in clause 6.4.4, with the following differences. The consumer needs to request the Analytics ID "Service Experience" for a UE or a group of UEs identified by GPSI or SUPI. The consumer includes both the Application ID(s) for which their Service Experience is requested and  the Target of Analytics Reporting for a UE or a group of UEs. The consumer can also only indicate the Target of Analytics Reporting if the service experience is per a UE regardless of specific application(s).  The analytic filter can be set according to clause 6.4.1.

The Observed Service Experience per RAT per frequency for a UE when consumed by PCF could be used as described in the TS 23.503 [4], clause 6.1.1.3.

# * * * * End of changes * * * *