SA WG2 Meeting #147E (e-meeting)	S2-2107370

18-22 October 2021, Elbonia	(revision of S2-20xxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	Correction of IMC definition

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	eNPN / Rel-17

Abstract of the contribution: Proposes to endorse modification of term IMS Credentials (IMC) in TR 21.905 v17.0.0, clause 3.

# 1.	Discussion

Per current definition in TR 21.905, IMC (IMS Credentials) can only be used by terminals without 3GPP access technology. However, with Rel-17 a SNPN UE can also use IMC to access IMS services (see Annex AB.1 of TS 23.228). During the study phase it was clarified that IMC definition in TR 21.905 needs to be changed, if the concept of IMC is adopted for use in SNPN(s), see TR 23.700-07 v17.0.0, clause 6.21.2.

However, this change was not done so far and is still missing. Thus, we are proposing that SA2 endorses correction of the definition of term IMC in TR 21.905 to allow IMC use also for SNPN UEs. As TR 21.905 is not under SA2 control, a corresponding CR needs to be submitted to next SA plenary. Nevertheless, it is beneficial that SA2 discusses and endorses the change first. We propose the following modification of term IMS Credentials (IMC) in TR 21.905 v17.0.0, clause 3, to be endorsed and submitted as CR to SA#94:

IMS Credentials (IMC): A set of IMS security data and functions for IMS access by a terminal that does not support any 3GPP access technology or by a terminal accessing IMS via SNPN access. The IMC is not including an ISIM or a USIM. The IMC is not used if ISIM or USIM is present.

# 2.	Proposed way forward

If the proposed text change in TR 21.905 is acceptable, SA2 is asked to endorse the proposed modification and SA2 chair should document the agreement in the meeting notes. Nokia will submit a corresponding CR for TR 21.905 to next SA#94 plenary in December 2021.