3GPP TSG-SA WG2 Meeting #147E	S2-2107453

# E-Meeting, 18th – 22nd October 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0018 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handover procedure with limited area MBS session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Handover procedure with limited area MBS session need correct. considering e.g. UE moving out of limited service area from MBS non-capable RAN. As UE moves, based on the UE mobility event notification on the MBS service area provided from AMF, if the SMF detects that the UE is outside the MBS service area, the SMF removes the associated QoS Flow(s) used for 5GC individual MBS traffic delivery not to forward the corresponding MBS traffic to the UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Mixed term such as MBS service area and session area and service area are reworded as MBS service session area - Handover procedure with limited area MBS session is updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Handover procedure with limited area MBS session is not supported well. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.4.2, 7.2.4.3.3. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******Start of 1st changes*******

7.2.4.2	Support of multicast service with location-dependent content

7.2.4.2.1	UE join location dependent multicast session and establishment procedure

The local multicast session join and establishment procedure is performed as defined in clause 7.2.1 with the following additions:

-	The local multicast session is configured as described in clause 7.2.4.2.2.

-	If SMF has no information about the multicast MBS session context for the indicated MBS Session ID, the SMF requests MB-SMF information via Nnrf_NFDiscovery_Request Request (MBS Session ID, UE location), the NRF provides information about the MB-SMF(s) serving the multicast session at the indicated location and MBS service session areas and  Area Session IDs for the multicast session, via Nnrf_NFDiscovery_Request Response (MB-SMF profile(Area Session ID(s), MBS service session area(s)). The SMF selects the MB-SMF based on the location area where the UE is residing and interacts with MB-SMF to retrieve QoS information of the multicast QoS flow(s) for the MBS Session ID.

-	SMF requests the AMF to transfer an N2 message to the RAN node using the Nsmf_PDUSession_UpdateSMContext response, to provide the NG-RAN with multicast session information which additionally includes the Area Session ID(s) and MBS service session area(s) that the NG-RAN node belongs to. The SMF provides all MBS service areas information (Area session ID, MBS service session area) to NG-RAN.

-	The RAN uses the received MBS Session ID(s) and Area Session ID(s) to determine the local multicast session context and whether the user plane for the local multicast session is already established.

7.2.4.2.2	Configuration for local MBS

For local MBS, the configuration procedure for the UE is optional and performed as defined in clause 7.1.1.2 with the following additions:

-	Multiple AFs may start the same multicast session with different content in different MBS service session areas. The NEF selects MB-SMF as ingress control node(s) for different MBS service session areas.

-	If presented, the NEF maps possible external identifiers for MBS service areas to network-internal identifiers (e.g. list of cells, TAIs).

-	MB-SMF allocates Area Session ID, and updates its NF profile towards the NRF with the MBS Session ID, MBS service session area and Area Session ID.

NOTE:	For a location dependent service provided in different MBS service session areas within the same SMF service area, it is assumed that one MB-SMF is used for an MBS Session. If the MBS Session ID is TMGI, the MB-SMF updating NF profile can be skipped.

-	The policy of Multicast session is determined based on the service requirements per MBS service area.

-	The MB-SMF may select the MB-UPF based on the MBS service area.

-	The MBS service area(s) are indicated to the UE in the Service Announcement as defined in clause 6.11.

7.2.4.2.3	Handover procedure

The Handover procedure for the UE is performed as defined in clause 7.2.3 with the following additions:

-	Before the Handover, The UE is camping at Source RAN and receiving multicast data corresponding to the MBS Session ID and Area Session ID.

-	For the Xn Handover:

-	Source RAN includes MBS Session ID, Area Session ID and MBS service area to the Target RAN.

-	Target RAN determines whether to establish the resources for multicast distribution for MBS Session ID and Area Session ID provided by Source RAN, based on MBS Session ID, Area Session ID and MBS service area.

NOTE: Data forwarding issue needs the feedback of RAN WGs

-	Target RAN responses to Source RAN, with the accepted MBS Session ID, Area Session ID. When Target RAN supports multicast but the UE is no longer in the location area, Target RAN rejects to handover the multicast session with a cause indication.

-	For the N2 Handover:

-	The SMF includes all MBS session area information (MBS Session ID, Area Session ID and MBS service area) to the Target RAN in Handover request.

-	Target RAN determines whether to establish the resources for multicast distribution for MBS Session ID and Area Session ID provided by SMF, based on MBS Session ID, Area Session ID and location area. -	If the target RAN determines the shared delivery is not established for the multicast session ID and area session ID, the target NG-RAN initiates the shared delivery establishment as specified in clause 7.2.1.4.

*******Start of 2nd changes*******

7.2.4.3.3	Handover procedure with limited area MBS session

At handover, the request and allocation of resources by source and target NG-RAN node shall take the MBS service area information into account.

Based on the UE mobility event notification provided from AMF, if the SMF detects that the UE is outside the MBS service area, the SMF removes the associated QoS Flow(s) used for 5GC individual MBS traffic delivery not to forward the corresponding MBS traffic to the UE.

The Handover procedure for the UE is performed as defined in clause 7.2.3 with the following additions:

-	When the source NG-RAN supports MBS if the UE moves out of the MBS service area, the Xn/N2 based handover is executed as defined in clause 7.2.3.4, i.e. the MBS session information is not provided to the target NG-RAN.

-	When the source NG-RAN does not support MBS and 5GC individual MBS traffic delivery is applied if the UE moves out of the MBS service area, the SMF removes the associated QoS Flow(s) and provided updated QoS flow information in N2 SM Info to the target NG-RAN.

Editor's note:	The details of Xn handover procedure will be aligned with RAN3 WG.

*******End of changes*******