3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2107489

# Elbonia, October 18 – 22, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0460 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Data Collection from the UE Application with UE IP address |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Detailed reason for change can be found in S2-2107488. Besides, application ID used to subscribe for UE data collection in 6.2.8.2.3 is missing in the clauses affected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Option 1) and Option 2) are provided at the same time. In Option 1), the NWDAF/NEF gets the SMF(s) serving the DNN/S-NSSAI from UDM and subscribes on the IP address allocation/release with APP_START of the application as reporting condition. In Option 2), the NWDAF/NEF gets the PCF(s) serving the DNN/S-NSSAI from BSF and subscribes to the IP address allocation/release with APP_START of the application as reporting condition. In both options, the missing application ID is included within the corresponding signaling message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Security risk would remain unresolved as IP addresses not used by the AF in untrusted domain might be exposed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.8.2.4.3, 6.2.8.2.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Option 1)* * * *

# * * * * First Change* * * *

6.2.8.2.4.3	AF in untrusted domain correlates UE data collection and NWDAF request

If the AF receives the Naf_EventExposure_Subscribe from NWDAF, via NEF, including Target for Event Reporting set to GPSI and not including the UE's IP addresses and the AF does not locally store the UE's IP address, the AF request the NEF to provide the allocated IPv4 address or IPv6 prefix or both as described in Figure 6.2.8.2.4.3-1.

NOTE 1:	The NWDAF can also provide the UE IP address to the AF as described in clause 6.2.8.2.4.1.

Figure 6.2.8.2.4.3-1: AF in untrusted domain correlates UE data collection and NWDAF request

0.	Same step as step 0 in figure 6.2.8.2.4.3-1.

1.	The AF receives a request to retrieve input data as described in clause 6.2.8.2.3 including a GPSI and External Application Identifier. The AF requests NEF to provide the IPv4address or IPv6 prefix or both serving the PDU session used to access the application identified by External Application Identifier for this GPSI towards the AF using Nnef_UEid_Get_Request.

2.	The NEF is configured with the DNN, S-NSSAI to access this AF. The NEF finds the SMF serving the PDU session(s) for this GPSI, DNN, S-NSSAI using Nudm_UECM_Get_Request including type of requested information set to SMF Registration Info and the S-NSSAI and DNN, as defined in clause 5.3.2.5.7 of TS 29.503 [26].

NOTE 2:	If there are more than one (DNN, S-NSSAI) combination to access this AF, the NEF will find the SMF(s) serving the PDU session(s) to any of these (DNN, S-NSSAI) combinations.

3.	The UDM provides the SMF id(s) and the tuple (PDU Session id (S-NSSAI, DNN) using Nudm_UECM_Get_Response to the NEF.

4.	The NEF sends Nsmf_EvenExposure_Subscribe to the SMF(s) identified in step 3, including the Target for Event Reporting set to the PDU Session id(s) provided in step 3 and the Event ID set to IP address/prefix allocation/change, together with APP_START event of the corresponding application ID as reporting condition.

5.	The SMF triggers the application detection for the application towards UPF as defined in clause 6.2.2.5 of TS 23.503[4]. Upon receiving the application detection report from UPF, the SMF provides the allocated IPv4 address or IPv6 prefix or both of the corresponding PDU Session to the NEF.

6.	The NEF provides the allocated IPv4 address or IPv6 prefix or both provided by SMF in step 5 to the AF.

7.	The AF correlates the UE data that includes the UE IP address and the NWDAF request for the GPSI using the retrieved IPv4 address or IP v6 prefix.

If the user plane session between the UE and the AF is released, the AF shall remove the stored correlation information between the UE IP address / prefix and GPSI.

# * * * * Second Change* * * *

6.2.8.2.4.4	NWDAF correlates UE data collection and NWDAF request for trusted AF and untrusted AF

NWDAF receives the analytics subscription from consumer and discover an AF as described in clause 6.2.8.2.3. NWDAF finds the PDU session(s) serving the SUPI, DNN, S-NSSAI from UDM and the allocated IPv4 address or IPv6 prefix from SMF as described in Figure 6.2.8.2.4.4-1.

Figure 6.2.8.2.4.4-1: NWDAF correlates UE data collection and NWDAF request

1.	The NWDAF finds the SMF(s) serving the PDU session(s) for this SUPI or GPSI using Nudm_UECM_Get_Request including SUPI or GPSI, type of requested information set to SMF Registration Info and the list of S-NSSAI and DNN combinations, as defined in clause 5.3.2.5.7 in TS 29.503 [26]. The NWDAF acquires the DNN, S-NSSAI used to access the AF using Nnrf_NFDiscovery_Request service operation or is configured with the DNN, S-NSSAI used to access the AF.

2. The UDM provides the SMF id(s) and the corresponding PDU Session id,(s), per S-NSSAI, DNN combination using Nudm_UECM_Get_Response to the NWDAF.

3. The NWDAF sends Nsmf_EvenExposure_Subscribe to the SMF identified in step 2, including the Target for Event Reporting set to the PDU Session id(s) provided in step 2 and the Event ID set to IP address/prefix allocation/change, together with APP_START event of the corresponding application ID as reporting condition.

4.	The SMF triggers the application detection for the application towards UPF as defined in clause 6.2.2.5 of TS 23.503. Upon receiving the application detection report from UPF, the SMF provides the allocated IPv4 address or IPv6 prefix or both of the corresponding PDU Session to the NWDAF.

5.	Step 3a for AF in trusted domain or step 3b for AF in untrusted domain in Figure 6.2.8.2.3-1 is performed with the exception that NWDAF sets the allocated IPv4 address or IPv6 prefix or both that were received in step 4 as target of event reporting.

If NWDAF subscribed for the PDU session used for the user plane connection between the UE and the AF is released notification in step 3, the SMF informs the NWDAF that the UE IP address / prefix is released via Nsmf_EventExposure_Notify. Based on this information, the NWDAF shall remove the stored correlation information between the UE IP address / prefix and SUPI.

# * * * * Option 2 * * * *

# * * * *First Change * * * *

6.2.8.2.4.3	AF in untrusted domain correlates UE data collection and NWDAF request

If the AF receives the Naf_EventExposure_Subscribe from NWDAF, via NEF, including Target for Event Reporting set to GPSI and not including the UE's IP addresses and the AF does not locally store the UE's IP address, the AF request the NEF to provide the allocated IPv4 address or IPv6 prefix or both as described in Figure 6.2.8.2.4.3-1.

NOTE 1:	The NWDAF can also provide the UE IP address to the AF as described in clause 6.2.8.2.4.1.

Figure 6.2.8.2.4.3-1: AF in untrusted domain correlates UE data collection and NWDAF request

0.	Same step as step 0 in figure 6.2.8.2.4.3-1.

1.	The AF receives a request to retrieve input data as described in clause 6.2.8.2.3 including a GPSI. The AF requests NEF to provide the IPv4address or IPv6 prefix or both serving the PDU session used to access the application identified by External Application Identifier for this GPSI towards the AF using Nnef_UEid_Get_Request.

2.	The NEF is configured with the DNN, S-NSSAI to access this AF. The NEF finds the PCF serving the PDU session(s) for this GPSI, DNN, S-NSSAI using Nbsf_Management_Subscribe including SUPI and/or GPSI and the S-NSSAI and DNN, as defined in clause 5.2.13.2.6 of TS 23.502[3].

NOTE 2:	If there are more than one (DNN, S-NSSAI) combination to access this AF, the NEF will find the PCF(s) serving the PDU session(s) to any of these (DNN, S-NSSAI) combinations.

3.	The BSF provides the PCF id(s) and the tuple (S-NSSAI, DNN) using Nbsf_Management_Notifyto the NEF.

4.	The NEF sends Npcf_EvenExposure_Subscribe to the PCF(s) identified in step 3, including the Target for Event Reporting set to the tuple(SUPI/GPSI, S-NSSAI, DNN) provided in step 3 and the Event ID set to IP address/prefix allocation/change, together with the APP_START event of the corresponding application as reporting condition.

5.	The PCF triggers the application detection for the application towards SMF as defined in clause 6.2.2.5 of TS 23.503[4]. Upon receiving the application detection report from SMF, the PCF provides the allocated IPv4 address or IPv6 prefix or both  for the corresponding PDU Session to the NEF.

6.	The NEF provides the allocated IPv4 address or IPv6 prefix or both provided by SMF in step 5 to the AF.

7.	The AF correlates the UE data that includes the UE IP address and the NWDAF request for the GPSI using the retrieved IPv4 address or IP v6 prefix.

If the user plane session between the UE and the AF is released, the AF shall remove the stored correlation information between the UE IP address / prefix and GPSI.

# * * * * Second change * * * *

6.2.8.2.4.4	NWDAF correlates UE data collection and NWDAF request for trusted AF and untrusted AF

NWDAF receives the analytics subscription from consumer and discover an AF as described in clause 6.2.8.2.3. NWDAF finds the PDU session(s) serving the SUPI, DNN, S-NSSAI from UDM and the allocated IPv4 address or IPv6 prefix from SMF as described in Figure 6.2.8.2.4.4-1.

Figure 6.2.8.2.4.4-1: NWDAF correlates UE data collection and NWDAF request

1.	The NWDAF finds the PCF(s) serving the PDU session(s) for this SUPI or GPSI using Nbsf_Management_Subscribe including SUPI or GPSI, and the list of S-NSSAI and DNN combinations, as defined in clause 5.2.13.2.6 of TS 23.502[3]. The NWDAF acquires the DNN, S-NSSAI used to access the AF using Nnrf_NFDiscovery_Request service operation or is configured with the DNN, S-NSSAI used to access the AF.

2. The BSF provides the PCF id(s) per S-NSSAI, DNN combination using Nbsf_Management_Notifyto the NWDAF.

3. The NWDAF sends Npcf_EvenExposure_Subscribe to the PCF(s) received in step 2, including the Target for Event Reporting set to the tuple(SUPI/GPSI, S-NSSAI, DNN) provided in step 2 and the Event ID set to IP address/prefix allocation/change, together with APP_START event of the corresponding application ID as reporting condition.

4.	The PCF triggers the application detection for the application towards the SMF as defined in clause 6.2.2.5 of TS 23.503[4]. Upon receiving the application detection report from SMF, the PCF provides the allocated IPv4 address or IPv6 prefix or both for the corresponding PDU Session to the NWDAF.

5.	Step 3a for AF in trusted domain or step 3b for AF in untrusted domain in Figure 6.2.8.2.3-1 is performed with the exception that NWDAF sets the allocated IPv4 address or IPv6 prefix that were received in step 4 as target of event reporting.

If NWDAF subscribed for the PDU session used for the user plane connection between the UE and the AF is released notification in step 3, the SMF informs the NWDAF that the UE IP address / prefix is released via Nsmf_EventExposure_Notify. Based on this information, the NWDAF shall remove the stored correlation information between the UE IP address / prefix and SUPI.

# * * * * End of changes * * * *