3GPP TSG SA WG 2 Meeting 147E		TDoc S2-2107582

October 18-22 2021

Title:	[DRAFT] Reply LS on Paging Subgrouping

Response to:	(S2-2106999/R2-2106552) LS on Paging Subgrouping

Release:	Rel-17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	SA2

To:	RAN2, RAN3, CT1

Cc:	-

Contact person:	Guillaume SEBIRE

Guillaume dot sebire at mediatek dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

TSG SA2 would like to thank RAN2 for their LS on Paging Subgrouping and notes the decision taken by RAN2 quoted hereafter.:

-	Adopt network-controlled subgrouping (based on individual UE characteristics, not specified or limited to paging prob as EUTRA, possibly with additional randomization)

-	If the network chooses to not provide specific subgrouping information, there will be configuration option where subgrouping can be supported by randomization (by UE-ID)

-	CN is responsible for allocating UEs to paging subgroups based on UE characteristics

-	Use same UE subgroups when in RRC_IDLE and RRC_INACTIVE.

TSG SA2 notes (G)WUS with E-UTRA (EPS, 5GS) was designed around UE paging probability information.

TSG SA2 intends to re-use the (G)WUS functionality framework to accommodate PEI paging subgrouping over NR in 5GS, while also ensuring adjustments are made to cover RRC_INACTIVE. However, SA2 could not understand what RAN2 agreement means and what its implications are with regards to:

-	"(based on individual UE characteristics, not specified or limited to paging prob as EUTRA, possibly with additional randomization)"

It is indeed not clear to SA2:

-	what "individual UE characteristics" refer to, where they are defined, and how they are used; and

-	if these UE characteristics are not specified, then isn’t subgrouping simply up to network implementation?

-	what "possibly with additional randomization" means esp. given the text refers to individual UE characteristics as opposed to subgrouping.

TSG SA2 would like to ask RAN2 to clarify the above, in a timely manner for SA2#148e meeting.

# 2	Actions

To RAN2

ACTION: 	TSG SA2 would like to ask TSG RAN2 to clarify what "(based on individual UE characteristics, not specified or limited to paging prob as EUTRA, possibly with additional randomization)" means.

# 3	Dates of next TSG SA2 meetings

TSG SA2#148e	15 – 19 November, 2021