3GPP TSG-SA2 Meeting #147-e	S2-2107605

# Online, , 18th-22nd Oct 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 032 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification on session join and shared tunnel establishment related procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo Mobile Communications Co. LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | [7.2.1.3] 1. number of the steps is not consecutive 2. Steps for shared tunnel configuration over N19mb interface is not clear 3. RAN has concluded that RAN node sends 5MBS supporting indication to SMF during HO, but different mechanism for same purpose is used in session join. 4. If multicast transport is used, the MBS data sent by the MB-UPF to different NG-RAN nodes and UPF will have same sequence number, it is not clear whether unicast transport also has the same characteristic, which helps the service continuity with packet duplication/loss detection. [7.2.1.4] 5. In step 3, the AMF may already have the MBS session context, in this case, always query MB-SMF from NRF is not necessary. 6. In steps 2, 3, 6, and 7, Area Session ID is used together with MBS Session ID to identify a MBS session, so should be encapsulated in same level. 7. Some editorial modification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | [7.2.1.3] 1. renumber the steps 2. In step 4, the MB-SMF sends multicast DL tunnel info (replaced LL MC) to SMF if known (e.g. created during previous join operation), which helps the SMF sets up the shared tunnel over N19mb, hence EN related to step 13b and 13e is removed, and step 13f is not needed (included in 13b and 13e). Clause 9.1.3.3 add the parameter in. 3. In step 11 and 12, 5G MBS support indication is used instead of accepted QoS flows. 4. Specify in steps 19 and 20 that same MBS data uses same sequence number by MB-UPF no matter whether multicast transport or unicast transport is used. [7.2.1.4] 5. In step 3, AMF also can query MB-SMF from locally stored inforamtion. 6. In steps 2, 3, 6, and 7, specify optional Area Session ID together with MBS Session ID, which is not included in N2 SM information. 7. Some editorial modification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1.3, 7.2.1.4, 9.1.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.2.1.3	Multicast session join and session establishment procedure

The following steps are executed before the UE requests to join the MBS session:

-	The MBS Session may have has been configured in the 5GC (see clause 7.1.1 for details).

-	The UE registers in the PLMN and establishes a PDU session.

-	The UE has known at least the MBS Session ID of a multicast group that the UE can join, e.g. via service announcement.

Figure 7.2.1.3-1: PDU Session modification for UE joining multicast session

1.	To join the multicast group, the UE sends a PDU Session Modification Request which additionally contains one or several MBS Session ID(s) and join request. The MBS Session ID(s) indicate the multicast MBS session(s) that UE wants to join.

2.	[Conditional] Based on the received MBS Session ID and join request, the SMF determines this is multicast MBS Session join request. The SMF checks whether the user is authorized to use the required multicast communication service.

3.	[Conditional] If SMF has no information about MBS Session context for the indicated MBS Session ID(s), SMF discovers and selects an MB-SMF for the MBS Session via the NRF as described in clause 7.1.2. If no MB-SMF is assigned for the MBS session ID, the SMF may select an MB-SMF and request it configure the multicast MBS session or the SMF may reject the join request.

NOTE 1:	Details how the SMF select an MB-SMF and request it configure the multicast MBS session are left to SMF implementation.

4.	[Conditional] For each MBS session in step 1, if the SMF has not subscribed the MBS session context, it invokes Nmbsmf_ MBSSession_ContextStatusSubscribe request (MBS Session ID) towards the MB-SMF to subscribe the events notification related to the multicast MBS session. The MB-SMF indicates multicast MBS session context information (multicast QoS flow information (e.g., QoS profile(s) for multicast MBS session), [start time], [session status indication (active/inactive)], [MBS session authorization information (MBS session open for any user)], [multicast DL tunnel info]) in the response to the SMF.

If it is the first time for the MB-SMF to receive Nmbsmf_ MBSSession_ContextStatusSubscribe request of the indicated MBS Session from any SMF, the MB-SMF learns it is the first UE joining the multicast MBS session. For multicast transport between MB-UPF and content provider, if it is the first UE joining the multicast MBS session, and MB-UPF has not joined the multicast tree in the MBS configuration procedure described in clause 7.1.1, the MB-SMF requests the MB-UPF to join the multicast tree towards the AF/MBSF, otherwise MB-SMF will not send the request to the MB-UPF. The MB-SMF determines whether the user is authorized to join the multicast MBS session as follows:

-	The MB-SMF checks the user subscription data received from the UDM to determine whether the user is allowed to use any multicast communication service.

-	If so, the MB-SMF checks the received indication whether the multicast MBS session is open for any user.

-	If the multicast MBS session is not open to any user, the MB-SMF checks the user subscription data received from the UDM to determine whether the MBS session ID is included.

If a UE joins prior to the start time of the multicast MBS session, the SMF may accept the join request in step 5 and indicate to the UE the start time, or it may reject the join request with appropriate error cause and possible back-off time. If a UE joins while the multicast MBS session is inactive, the SMF accepts the join request in step 5. If authorization check fails, the SMF indicates cause value in the PDU Session Modification Reject sent to the UE and proceeds with step 5 without N2 SM information.

If dynamic PCC is deployed, the SMF interacts with the PCF with regard to the associated QoS flows.

NOTE 2:	The MB-SMF can answer the Nmbsmf_ MBSSession_ContextStatusSubscribe request either based on information received in the configuration procedures in Clause 7.1.1 or based on preconfigured information. The pre-configuration also includes information about the MBS session stored in the NRF. If the MB-SMF uses preconfigured information, the pre-configuration also includes MB-UPF configuration.

5.	The SMF responds to AMF through Nsmf_PDUSession_UpdateSMContext response (N2 SM information (PDU Session ID, MBS Session ID, [updated PDU Session information], [mapping information between unicast QoS flow(s) and multicast QoS flow (s)]), N1 SM container (PDU Session Modification Command)) to:

-	create an MBS session context for the indicated MBS session in the RAN, if it does not exist in the RAN already; and

-	inform the NG-RAN about the relation between the multicast MBS session context and the UE's PDU Session context by including the MBS session ID and the mapping between the multicast QoS flow(s) and associated QoS flow(s).

Based on operator policy, the SMF may prepare for 5GC individual MBS traffic delivery fall-back. The SMF maps the received QoS information of the multicast QoS Flow into PDU Session's unicast QoS Flow information, and includes the information of the QoS Flows and the mapping information about the QoS Flows in the SM information sent to RAN.

NOTE 3:	Detailed information included in N2 SM information will be aligned with by RAN WG3.

NOTE 4:	A PDU Session UP activation is not triggered by message 6 if the N2 SM information includes information related to the multicast MBS session and associated QoS flows and the NG RAN node is 5G MBS capable.

Editor's Note: The implication of not triggering PDU Session UP activation in NG-RAN when SMF informs the NG-RAN of UE join requires RAN collaboration.

Editor's note:	Possible PCF interactions related to the multicast QoS flows are FFS.

6.	The N2 message, which includes the multicast MBS session information and PDU session modification information is sent to the NG-RAN.

If the 5G MBS is not supported by NG-RAN, 5GC individual MBS traffic delivery may be used. Otherwise if the 5G MBS is supported by NG-RAN, 5GC shared MBS traffic delivery is adopted.

If the NG-RAN supports 5G MBS, the NG-RAN uses the MBS Session ID to determine that the PDU Session identified by the PDU Session ID is associated with the indicated multicast MBS session.

If the mapping information is received and the NG-RAN supports 5G MBS, the associated unicast QoS flow information is not used to allocate the radio resource and CN resource.

NOTE 5:	It is NG-RAN that decides whether radio resource is allocated or not, and it is NG-RAN/UPF that decides whether multicast transport or unicast transport is used between the NG-RAN/UPF and the MB-UPF.

7. [Conditional]	 If shared tunnel has not been established for the multicast MBS session towards the NG-RAN node, the procedures in clause 7.2.1.4 for the establishment of shared delivery toward NG RAN node are executed. Step 7 is executed separately for each multicast MBS session.

8.	The NG-RAN node performs AN specific signalling exchange with the UE to establish radio resource for the multicast MBS session if not established yet. If the NG-RAN does not support MBS, radio resource are reconfigured for unicast transmission of the MBS data over the associated PDU session. As part of the AN specific signalling exchange, the N1 SM container (PDU Session Modification Command) is provided to the UE.

9.	The NG-RAN node sends the PDU session modification response.

If the 5G MBS is supported by NG-RAN, the N2 SM response container includes the indication of supporting 5G MBS.

10.	The AMF invokes Nsmf_PDUSession_UpdateSMContext request ([N2 SM container]) to the SMF.

Per the indication of whether it supports 5G MBS, the SMF determines the delivery mode, i.e. whether 5GC individual MBS traffic delivery is used for multicast data transmission.

NOTE 6:	If the shared tunnel is used, no interaction with UPF is needed for the indicated multicast MBS session

11.	[Conditional] This step is used for 5GC Individual MBS traffic delivery, if the related NG-RAN does not support 5G MBS. If a shared tunnel between the UPF (PSA) and MB-UPF for 5GC individual MBS traffic delivery has not yet been established by the SMF for the multicast MBS session, steps 11a to 11e are executed.

11a.	 The SMF contacts the UPF to request the creation of a tunnel and provides the MBS session ID. If the multicast DL tunnel info of the multicast MBS session has known by the SMF, the SMF provides it to the UPF. The UPF indicates to the SMF whether the tunnel for this multicast MBS session is newly allocated (as there can be multiple SMFs interacting with the same UPF for the same multicast MBS Session).

If the UPF determines to use unicast transport over N19mb, the UPF allocates a DL N19mb Tunnel ID for the MBS session if the SMF request is the first one to allocate DL N19mb Tunnel ID for the multicast MBS Session in the UPF. The UPF includes the unicast DL tunnel Info in the response to the SMF. The unicast DL tunnel info includes the DL N19mb Tunnel ID and the UPF address.

If the UPF determines to use multicast transport over N19mb and the multicast DL tunnel info of the multicast MBS session is provided, steps 11b to 11e are skipped.

The SMF also provides the mapping information between the multicast QFI and the corresponding unicast QFI of the multicast QFI in the PDU Session to the UPF, the UPF is ready to forward the multicast data via the associated QoS flow(s) of the PDU Session based on the mapping information.

11b.	 If the UPF indicates the DL N19mb Tunnel is newly allocated, the SMF invokes Nmbsmf_MBSsession_ ContextUpdate request (MBS Session ID, [unicast DL tunnel info]) towards the MB-SMF for establishing the multicast MBS session transport between MB-UPF and UPF.

11c.	 If the unicast DL tunnel info of the UPF is received, the MB-SMF configures the MB-UPF to transmit the multicast MBS session data towards UPF using the received unicast DL tunnel info. If the unicast DL tunnel info is not received and it is the first time the MB-SMF receives the context update for the multicast MBS session, the MB-SMF configures the MB-UPF to transmit the multicast MBS session data.

11d.	 The MB-SMF responds to the SMF through Nmbsmf_MBSSession_ ContextUpdate response (MBS Session ID, [multicast DL tunnel info]). If the UPF DL tunnel info for unicast transport is not received by the MB-SMF, multicast transport between MB-UPF and UPF is to be used and multicast DL tunnel info is included, and the MB-SMF indicates multicast DL tunnel info that includes the common downlink tunnel ID and the LL SSM for the multicast MBS session.

11e.	 For multicast transport between MB-UPF and UPF, if the multicast DL tunnel info is received in step 11d, the SMF configures the UPF with the multicast DL tunnel info to receive the multicast MBS session data. The UPF joins the SSM of MB-UPF.

12.	The SMF responses to the Nsmf_PDUSession_UpdateSMContext towards the AMF.

13.	The MB-UPF receives multicast PDUs, either directly from the content provider or via the MBSTF that can manipulate the data.

Steps 14 to 16 are for 5GC shared MBS traffic delivery:

14.	The MB-UPF sends multicast PDUs in the N3mb tunnel associated to the multicast MBS session to the NG-RAN. There is only one tunnel per multicast MBS session and NG-RAN node, i.e., all the UEs which have joined the multicast MBS session via the NG-RAN node share this tunnel for reception of the multicast MBS session data.

15.	The NG-RAN selects PTM or PTP radio bearers to deliver the multicast PDUs to the UE(s) that have joined the multicast MBS session.

16.	The NG-RAN transmits the multicast MBS session data to the UE(s)  using the selected PTM or PTP radio bearer(s).

Steps 17 to 19 are for 5GC individual MBS traffic delivery:

17.	The MB-UPF sends multicast PDUs in the N19mb tunnel associated to the multicast MBS session to the UPF. There is only one tunnel per multicast MBS session and destination UPF, i.e., all associated PDU sessions served by the destination UPF share this tunnel. The MB-UPF uses same QoS flow level sequence number for same packet sent to different UPF.

18.	The UPF forwards the multicast data towards the NG-RAN via unicast (i.e. in the N3 tunnel of the associated PDU Session). The UPF uses same QoS flow level sequence number received via N19mb interface for the forwarding packet via unicast.

19.	The NG-RAN forwards the multicast MBS session data to the UE via unicast (i.e. over the radio bearer(s) corresponding to the associated QoS flow(s) of the associated PDU Session).

NOTE 7:	Details of the DL MBS data transmission are described in clause 6.7.

NOTE 8:	When the MBSF is involved in the multicast MBS session, the tunnel between MBSTF and MB-UPF has been established in the configuration procedure.

#### 7.2.1.4	Establishment of shared delivery toward RAN node

In the following cases, the shared tunnel for shared delivery is established between the NG-RAN and MB-UPF:

-	The first UE is included in the context of the MBS session in the NG-RAN.

NOTE 1:	When the multicast MBS session is deactivated, if there is at least one UE joining the multicast MBS session is in RRC-CONNECTED state in the NG-RAN, the shared delivery is not released.

NOTE 2:	Share delivery establishment procedures are used when 5G MBS supporting NG-RAN node(s) get involved in the multicast MBS session upon multicast MBS session activation.

-	Handover to the target NG-RAN when the shared delivery tunnel is not established in the target RAN node for this multicast MBS session.

NG-RAN

AMF

MB-SMF

MB-UPF

1. Decide to establish shared delivery for a multicast session

2. N2 Message ()

3. Nmbsmf_MBSSession_ContextUpdate Request

4. N4mb Session Modification

6. Nmbsmf_MBSSession_ContextUpdate Response

7. N2 Message ()

5. Store AMF ID for the MBS session

Figure 7.2.1.4-1: Establishment of shared delivery toward NG-RAN node

1.	A NG-RAN node decides to establish shared delivery for a multicast MBS session when it serves at least one UE within the multicast MBS session. For location dependent services, the NG-RAN node needs to establish shared delivery for the location dependent contents of a multicast MBS session if it serves at least one UE assigned to an MBS session ID and area session ID.

2.	The NG-RAN sends an N2 MBS Session request message (MBS Session ID, [Area Session ID], N2 SM information ([unicast DL tunnel Info])) towards the AMF.

If the NG-RAN node is configured to use unicast transport for the shared delivery, it allocates a GTP tunnel endpoint and provides the unicast DL tunnel Info in the request, which includes the GTP tunnel endpoint and NG-RAN node address. For location dependent MBS services, the NG-RAN node also provides the Area Session ID.

3.	The AMF selects the MB-SMF serving the multicast MBS session, e.g., using the NRF discovery service or locally stored information. It invokes Nmbsmf_MBSSession_ContextUpdate request (MBS Session ID, [Area Session ID], N2 SM information) to the MB-SMF.

The AMF stores the information of the NG-RAN nodes (e.g. NG-RAN node ID) for the subsequent signaling related to the multicast MBS Session.

4.	[Conditional] If the MB-SMF received unicast DL tunnel Info in step 3, it configures the MB-UPF to send multicast data for the multicast MBS session (or location dependent content of the multicast MBS session if an area session ID was received) towards that GTP tunnel endpoint via unicast transport.

5.	The MB-SMF stores the information of the AMF (e.g., AMF ID) in the multicast MBS session context (or location dependent part of the multicast MBS session context if an Area Session ID was received) to enable subsequent signalling towards that AMF.

6.	The MB-SMF sends Nmbsmf_MBSSession_ContextUpdate response (MBS Session ID, [Area Session ID], N2 SM information ([TMGI], multicast QoS flow information, [multicast DL Tunnel Info], [MBS service area])) to the AMF. If the MB-SMF did not receive unicast DL Tunnel Info in step 3, it provides the multicast DL tunnel info that includes transport multicast address (e.g. a LL SSM) and a GTP tunnel endpoint for multicast transport of the shared delivery.

7.	The AMF sends an N2 MBS Session response message (MBS Session ID, [Area Session ID], N2 SM information) to the NG-RAN node. If the NG-RAN node receives the multicast DL Tunnel Info of the shared delivery, it uses the transport multicast address included in the multicast DL tunnel info to join the multicast transport distribution.

# * * * * Second change * * * *

#### 9.1.3.3	Nmbsmf_ MBSSession_ContextStatusSubscribe service operation

Service operation name: Nmbsmf_ MBSSession_ContextStatusSubscribe

Description: Service Consumer NF can use this service operation to request information (e.g. QoS information) about a multicast session and to subscribe to notification of events about the multicast session context.

Inputs, Required: Multicast Session ID, notification target address, Events ID(s).

Inputs, Optional: immediate reporting indication.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID.

Outputs, Optional: If immediate reporting indication is set: Event information (e.g. QoS information for multicast session, multicast MBS session status (activated, deactivated), multicast session service area for local multicast service), Start time of multicast MBS session, multicast DL tunnel info, if consumer is SMF: indication that the multicast MBS session allows any UE to join.

# * * * * End of changes * * * *