SA WG2 Meeting #147e	S2-2107631

# October 18 – 22nd, 2021; Elbonia             		 		(revision of S2-21xxxx)

Title:	[draft] Response to LS on full registration request message to be rerouted via RAN

Response to:	S3-213035

Release:	Rel-17

Work Item:	FS_AMFREAL_SEC

Source:	SA2

To:	SA3

Cc:	CT1, RAN3

Contact Person:

#### Name: Alessio Casati

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 would like to thank SA3 for their LS on full registration request message to be rerouted via RAN".

SA2 has the following answers to the questions in this LS.:

Q1: Is the full Registration Request message the complete Registration Request message that contains both clear text IEs and non-clear text IEs, as defined in clause 4.4.6. in TS 24.501?

SA2 Answer 1:  yes, in our understanding this is the case. SA2 does not breakdown in their specifications the separate steps involved in retrieving the non-cleartext IEs.

Q2: What is the Initial UE message in step 7b?

SA2 Answer 2: it is this message defined in TS 38.413, which carries the NAS RR forwarded by the initial AMF to the target AMF.

### 8.6.1 Initial UE Message #### 8.6.1.1 General The Initial UE Message procedure is used when the NG-RAN node has received from the radio interface the first uplink NAS message to be forwarded to an AMF. #### 8.6.1.2 Successful Operation Figure 8.6.1.2-1: Initial UE message

In normal cases, the NG-AP Initial UE Message carries the NAS RR carried in RRC msg5.  In this case, however, the NAS message is not received from a RRC message over the radio interface, rather from the initial AMF so that is relayed to the new AMF and in principle it can contain both cleartext and non-cleartext IEs as this is not transmitted over the radio. Would SA3 consider that only the RR including cleartext IEs only should be forwarded?

Q3: Does the Registration Request message the initial AMF sends out refer to

-	the message step 1 of 4.2.2.2.3 Registration with AMF re-allocation, or

-	the complete registration request received by the initial AMF in NAS Security mode complete message when the message in step 1 is not protected as UE had no security context; and the complete registration request message in the NAS container in the message in step 1 which is protected as UE had security context?

SA2 Answer 3: in the stage 2 view of the world (that predated the introduction of the "Protection of initial NAS signalling messages" specified in TS 24.501 for UEs that have not yet a NAS security context when they send a Registration Request message), this is meant to be including all the information including the one retrieved with confidentiality protection (i.e. the non-cleartext IEs) via the Security Mode Complete message. If this is not advisable, and just the cleartext information should be sent in the Initial UE message from the Initial AMF to the RAN and from the RAN to the Target AMF, then SA2 kindly requests SA3 to specify what should be done (e.g., should SA2 clarify by means of a CR that only cleartext IEs shall be included?).

Q4: Is it feasible from SA2 perspective if the initial AMF sends the message in Step 1 of 4.2.2.2.3 for reroute?

SA2 Answer 4: SA3 should clarify whether only the cleartext IEs that are initially received from the UE in the RR should be included, or both cleartext and non-cleartext IEs that the AMF obtains by applying the  "Protection of initial NAS signalling messages" . Both options are feasible but there are the known security issues which we hope SA3 is guiding SA2 on, in the event that there was no N14.

2. Actions:

To RAN2:

ACTION: 	SA2 kindly asks SA3 to take the above information into account.

3. Date of Next TSG-SA WG2 Meetings:

NEXT SA2 MEETING DATES