3GPP TSG-SA WG2 Meeting #147E e-meeting	S2-2107724

# Elbonia, October 18 – 22, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0029 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network Initiated UUAA Revocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When the aerial subscription data in UDM is modified as the UAV UE is not allowed to use UAS services, then it is notified to AMF or SMF and the corresponding procedures, such as Network initiated De-registration or Network initiated PDU Session Release, would be performed. However, the USS cannot notice that the UAV UE lost its aerial subscription until it receives the PDU Session Status Event notification. Also, it is not clear to the USS whether the UUAA results stored in the network and the UAV UE are successfully revoked. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe the UUAA Revocation initiated by AMF/SMF when the aerial subscription data in Access and Mobility/Session Management Subscription Data in UDM is changed. UAS NF informs to USS that the authorization revocation procedure is triggered by Core Network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The USS cannot notice the authorization revocation at appropriate time and keep transmit the traffic towards the UAV UE which already lost the connection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.7, 5.2.7.x (new), 5.2.7.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First change *****

5.2.7	UUAA Revocation

5.2.7.x	USS/UTM initiated UUAA Revocation

Figure 5.2.7.1-1: Procedure for UAV authorization revocation by USS.

UAS NF stores the UAV UEs UUAA context after successful UUAA procedure as explained in clause 5.2.2.2 for UUAA-MM and in clause 5.2.3.2 for UUAA-SM procedure.

1.	The USS sends Naf_Auth_notification to UAS NF for authorization revocation. The USS includes GPSI, CAA-Level UAV ID, cause of revocation, PDU Session IP address if available in the authorization revocation request.

2.	UAS NF retrieves the UAV UE's stored UUAA context. From the stored UUAA context the UAS NF determines the target AMF or SMF for sending the notification.

3a or 3b.	The UAS NF sends Nnef_Auth_Notification request to notify the target NF, i.e. either the AMF or the SMF that the UAV is not authorized anymore.

4.	The UAS NF responds back to the USS indicating that authorization revocation request has been successfully initiated.

5a.	If UAS NF has subscribed to AMF for the Mobility Event Exposure with Event ID = Reachability Filter before, UAS NF unsubscribes to AMF for the mobility event notification by sending Namf_EventExposure_Unsubscribe request with Subscription Correlation ID.

5b.	The AMF acknowledges the un-subscription request from 5a by sending Namf_EventExposure_Unsubscribe response.

6.	If UE is in CM_Idle state, the target NF (i.e. either the AMF or the SMF) initiates the Network Triggered Service Request procedures as described in clause 4.2.3.3 of TS 23.502 [3].

7a.	If the target NF is AMF, the AMF initiates UCU procedure to inform the UE that UUA is revoked. The AMF shall also initiate the release of PDU Sessions related to UAS services.

7b. If the target NF is AMF, based on network policy the AMF may start network initiated de-registration process as described in clause 4.2.2.3.3 of TS 23.502 [3].

7c.	If the target NF is SMF, the SMF starts network initiated PDU session release process as described in clause 4.3.4 of TS 23.502 [3] to release the associated PDU session.

*** Next change (new text) *****

5.2.7.y	Network initiated UUAA Revocation

Figure 5.2.7.x: Procedure for UAV authorization revocation initiated by network.

0. UAS NF stores the UAV UEs UUAA context after successful UUAA procedure as explained in clause 5.2.2.2 for UUAA-MM and in clause 5.2.3.2 for UUAA-SM procedure.

1a.	After successful UUAA-MM procedure, the AMF may subscribe to be notified using Nudm_SDM_Subscribe when the aerial subscription in Access and Mobility Subscription Data is modified, UDM may subscribe to UDR by Nudr_DM_Subscribe.

1b.	After successful UUAA-SM procedure, the UPF may subscribe to be notified using Nudm_SDM_Subscribe when the aerial subscription in Session Management Subscription Data is modified, UDM may subscribe to UDR by Nudr_DM_Subscribe.

2.	The USS subscribes to the UAS NF using Naf_Authentication_Notification for update authorization data. The USS includes GPSI, CAA-Level UAV ID, Cause of revocation which indicates the change of aerial subscription, and PDU Session IP address if available in the authorization update request.

3a.	The UAS NF may subscribe to the AMF using Nnef_Authentication_Notification Subscribe (GPSI, Cause of revocation which indicates the change of aerial subscription) to be notified when the aerial subscription data is modified (e.g. AerialUESubscriptionInfo is changed to ‘not allowed’) in the Access and Mobility Subscription Data in the UDM.

3b. The UAS NF may subscribe to the SMF using Nnef_Authentication_Notification Notification Subscribe (GPSI, PDU Session IP address, Cause of revocation which indicates the change of aerial subscription) to be notified when the aerial subscription data is modified (e.g. Aerial service indication is removed) from Session Management Subscription Data in the UDM.

4. The aerial subscription data in the UDM is changed as the UAV UE is not allowed to use UAS service anymore.

5a.	If the aerial subscription is changed in Access and Mobility Subscription Data in step 4, then the UDM sends Nudm_SDM_Notification to the AMF.

5b.	If the aerial subscription is changed in Session Management Subscription Data in step 4, then the udm sends Nudm_SDM_Notification to the SMF.

6a.	The AMF notifies to the UAS NF that authorization data is updated using Nnef_Authentication_Notification Notify (GPSI, Cause of revocation which indicates the change of aerial subscription).

6b. The SMF notifies to the UAS NF that authorization data is updated using Nnef_Authentication_Notification Notify (GPSI, PDU Session IP address, Cause of revocation which indicates the change of aerial subscription).

7. The UAS NF notifies to the USS indicating that the authorization is updated with revocation caused by the change of aerial subscription of the UAV UE. When the USS receives Naf_Authentication_Notification Notify message, the USS notices that the authorization revocation procedure is triggered by the network.

8. step 2 to step 7 of USS/UTM initiated UUAA Revocation in Figure 5.2.7.1-1 are performed.

*** End of changes ***