3GPP TSG-SA2 Meeting #147E	S2-2107762

# Electronic, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3217 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rapporteur CR for editorial fixes – Inclusive Terminology |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shangai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-10-10 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Replace non-inclusive terminology (Master/Slave) with inclusive terminology as recommended in 3GPP drafting rules (Master -> Primary; Slave -> Secondary) – using LS exchange with IEEE, ensure also full alignment with IEEE specification. Snippet from TR 21.801 Annex K: Non-inclusive terminology and alternatives Table K.1 contains a list of non-inclusive terminology together with a non-exhaustive list of suggested alternatives. Table K.1: Non-inclusive terms and alternatives Non-inclusive term; Examples of alternative terms. master (when used in "master / slave" context); primary, controller, main. slave; secondary, standby. white list (NOTE); allow list, accept list. black list (NOTE); block list, drop list, forbidden list. grey list (a term which has been used in conjunction with white list and black list) should be replaced with e.g. track list, inspect list (NOTE).. NOTE: including single word and hyphenated versions.. |
| Summary of change: |  | - Replace non-inclusive terminology (Master/Slave) with inclusive terminology as recommended in 3GPP drafting rules (Master -> Primary; Slave -> Secondary) – using LS exchange with IEEE, ensure also full alignment with IEEE specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec has non-inclusive terminologies. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex F.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# F.3	BMCA procedure

Figure F.3-1: BMCA procedure

1.	PDU Session Establishment is performed as shown in F.1-1 step 1 and 2 for an Ethernet or IP type PDU Session to carry the UMIC or PMIC.

2.	The TSN AF or TSCTSF may subscribe to notifications for the PTP port state changes from UPF/NW-TT.

3.	The UPF/NW-TT receives the Announce message via User -plane from DS-TT connectivity established using PDU session, or via NW-TT port over N6.

As Announce message is a periodic message, after step 3, the UPF/NW-TT will receive Announce messages regularly.

4.	The NW-TT runs the BMCA algorithm in order to determine the PTP port state for the DS-TT port(s) and NW-TT port(s).

BMCA will be triggered after receiving the Announce message.

5.	If the BMCA procedure in NW-TT determines to use Announce message from the external grandmaster PTP instance, the UPF/NW-TT regenerates the Announce message based on the received Announce message for each Primary PTP port on the NW-TT and DS-TT(s) port for this PTP domain. The NW-TT/UPF forwards the regenerated Announce messages to the PDU session(s) related to the Primary PTP ports on the DS-TT(s).

6.	If the TSN AF or TSCTSF has subscribed for notifications for the PTP port state changes, the UPF/NW-TT reports any changes to the PTP port states to the TSN AF or TSCTSF via UMIC (for DS-TT ports) or PMIC (for NW-TT ports).

7.	Based on the notification for the PTP port state changes, the TSN AF or TSCTSF may request appropriate QoS treatment and PDU Session modification may then be triggered to modify the QoS Flow carrying the gPTP messages over user plane in order to be compliant with the IEEE Std 802.1AS [75] delay recommendation for carrying gPTP messages as in clause 5.27.1.6 of TS 23.501 [2].