3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2107786

# Elbonia, October 18 – 22, 2021	(revision of S2-21XXXXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3354 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NSAC when emergency/critical/priority service status changes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current text in clause 5.15.11.0 specifies that NSAC exemption is performed in the AMF (for NSAC for maximum number of UEs) or in the SMF (for NSAC for maximum number of PDU Sessions). However, the following scenarios are possible: - The UE initially registers with the S-NSSAI (or establish PDU Session) for non-priority service. The AMF or SMF perform the NSAC procedure. Afterwards a critical/priority service is started. The question occurs: does the AMF or SMF trigger the NSAC procedure to inform the NSACF that the S-NSSAI or PDU Session is used for critical/priority service? - The UE initially registers with the S-NSSAI (or establish PDU Session) for priority service. The AMF or SMF skips the NSAC procedure. Afterwards the critical/priority service is terminated. The question occurs: does the AMF or SMF trigger the NSAC procedure although the UE is already registered? It is proposed that the AMF or SMF updates the NSACF when the service using an S-NSSAI (or corresponding PDU Session) changes from critical/priority service to non-priority service and vice-versa. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed that the AMF or SMF trigger the NSAC procedure towards the NSACF and indicate whether the type of service used over the S-NSSAI is a critical/priority service (e.g. Emergency, MPS, MCS). The NSACF may exempt such a request from counting towards the maximum number of UEs (respectively PDU Sessions). When the service using an S-NSSAI (or corresponding PDU Session) changes from critical/priority service to non-priority service and vice-versa, the AMF or SMF sends an update to the NSACF. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use case of change of the status of critical/priority service is not specified. The NSACF does not track the number of UE or PDU Sessions using a critical/priority service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.11.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

### 5.15.11	Network Slice Admission Control

#### 5.15.11.0	General

The Network Slice Admission Control Function (NSACF) monitors and controls the number of registered UEs per network slice and the number of PDU Sessions per network slice for the network slices that are subject to Network Slice Admission Control (NSAC). The NSACF is also configured with the information indicating which access type is specified for the S-NSSAI subject to NSAC (i.e. 3GPP Access Type, Non-3GPP Access Type, or both).

The NSACF also provides event based Network Slice status notifications and reports to the consumer NF (e.g. AF).

Multiple NSACFs may be deployed in a network. One NSACF may be responsible for one or more S-NSSAIs in a service area. Within a service area, one S-NSSAI is handled by one NSACF, which can be one NSACF instance or one NSACF Set. A PLMN may have one or more service areas.

Each NSACF is configured with the maximum number of UEs per network slice and the maximum number of PDU Sessions per network slice which are allowed to be served by each network slice that is subject to NSAC.

Subject to operator policy and national/regional regulations, the NSAC for Emergency, Critical and Priority services (e.g. MCS, MPS) may be exempted and the following applies:

-	When the Registration Type indicates Emergency Registration or the establishment cause is associated with priority services (e.g. MPS, MCS), for the S-NSSAI to be subject to NSAC, or has been included in the Allowed NSSAI and is subject to NSAC, if the S-NSSAI is included in the Emergency Configuration Data, the AMF sends an update request to the NSACF indicating that the S-NSSAI is used for Emergency, Critical and Priority services. Correspondingly, when an Emergency, Critical and Priority service is terminated and the UE remains registered with the S-NSSAI, the AMF sends an update request to the NSACF indicating that the S-NSSAI is used for non-priority service.

-	When the emergency PDU Session is requested to be established or a priority header is received from the AMF during PDU Session establishment, if the S-NSSAI is subject to NSAC, the SMF sends an update request to the NSACF indicating that the S-NSSAI is used for Emergency, Critical and Priority service. Correspondingly, when an Emergency, Critical and Priority service is terminated and the UE continues to use the PDU Session, the SMF sends an update request to the NSACF indicating that the PDU Session is used for non-priority service.

The NSACF decides to exempt the request from AMF or SMF for the S-NSSAI. The NSACF does not count UEs (respectively PDU Session) using Emergency, Critical and Priority service towards the maximum number of UEs (respectively PDU Sessions). The NSACF may maintain a current number of UEs using non-priority service (i.e. for which the NSAC is applied) and a current number of UEs using Emergency, Critical and Priority service (i.e. for which the NSAC is exempted).

# * * * * End of changes * * * *