3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2107822

# Elbonia, October 18 – 22, 2021	(revision of S2- 2107598)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0468 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for analytics exposure via NWDAF(hosting DCCF) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-10-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In clause 7.4, Nnwdaf_DataManagement Service has been defined to help data consumer collect data and analytics from NWDAF, the related procedure is described in 6.2.6.2. Both the service definition and the procedure are very similar with the ones for data collection via DCCF. It may lead to a misunderstanding that Nnwdaf_DataManagement Service can also be used by analytics consumer to subscribe analytics from NWDAF, which has overlapped functionality with Nnwdaf_AnalyticsSubscription Service. But actually, this Nnwdaf_DataManagement Service can be used to obtain only historical analytics, which is just regarded as a kind of data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify Nnwdaf_DataManagement Service can be used to obtain historical analytics only, which is just regarded as a kind of data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may lead to a misunderstanding that Nnwdaf_DataManagement Service can also be used by analytics consumer to subscribe analytics from NWDAF, which has overlapped functionality with Nnwdaf_AnalyticsSubscription Service |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.2, 7.4.4, 7.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of changes ***************

7.4	Nnwdaf_DataManagement Service

7.4.1	General

Service Description: This service enables the consumer to subscribe/unsubscribe, be notified about data exposed by NWDAF, or fetch the subscribed data. It enables the consumer to request the generation of bulked data for Event IDs from NFs, as well as for Analytics IDs and retrieve the requested data.

7.4.2	Nnwdaf_DataManagement_Subscribe service operation

Service operation name: Nnwdaf_DataManagement_Subscribe.

Description: The consumer subscribes to receive data or historical analytics which is regarded as a kind of data, or if the data is already defined in NWDAF, then the subscription is updated.

Inputs, Required: Data Specification or Analytics Specification, Notification Target Address (+ Notification Correlation ID).

When the required data is a bulked data for Event IDs received from NFs, the Data Specification includes: set of Event IDs, Event Filter Information, Target of Event Reporting, and bulked data type as defined in clause 6.2.6.1.

When the required data is a bulked data for Analytics ID, the Data Specification includes: Target of Reporting with the set of Analytics ID(s) to generate bulked data, bulked data type, and analytics stage; Filter Information with Target of Analytics Information, Analytics Filter Information as defined in clause 6.2.6.1.

When the required data is historical analytics, the Analytics Specification is included in the required input parameters and identifies the historical analytics to be collected, the Analytics Specification includes: Analytics ID(s), Target of Analytics Reporting, Analytics Filter information and other input parameters for NWDAF services as defined in clause 7.2 and 7.3.

NOTE 1:	Event Filter Information, Target of Event Reporting and Bulked data type can be provided per individual Event ID in a set of Event IDs to generate bulked data.

NOTE 2:	Bulked data type, analytics stage, Target of Analytics Information, Analytics Filter Information can be provided per individual Analytics ID in a set of Analytics IDs to generate bulked data.

Inputs, Optional: Service Operation, Bulked Data Formatting and Processing, Data Source, ADRF information to store data used for generated bulked data, ADRF ID or NWDAF ID (or ADRF Set ID or NWDAF Set ID) storing historical data to be used for bulked data generation.

The Bulked Data Formatting and Processing parameters include: the parameters defined in clause 5A.4 as well as Periodic bulked data notification, Feature type, Time Window, Minimum and/or maximum number of samples, Fetch flag, Bulked data deadline, Notification Event Clubbing, Processing rules.

NOTE 3:	The Service Operation (in the case of Event IDs), Bulked Data Formatting and Processing, Data Source, and ADRF information to store data used for generated bulked data, ADRF ID or NWDAF ID (or ADRF Set ID or NWDAF Set ID) storing historical data to be used for bulked data generation can be provided per individual Event ID or Analytics ID included in the Data Specification.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of the requested subscription).

Outputs, Optional: None.

7.4.3	Nnwdaf_DataManagement_Unsubscribe service operation

Service operation name: Nnwdaf_DataManagement_Unsubscribe.

Description: The NF consumer deletes an event if already defined in NWDAF.

Inputs, Required: Subscription Correlation ID.

Outputs, Required: Operation execution result indication.

7.4.4	Nnwdaf_DataManagement_Notify service operation

Service operation name: Nnwdaf_DataManagement_Notify.

Description: NWDAF notifies the consumer instance of the requested data or historical analytics which is regarded as a kind of data according to the request.

Inputs, Required: Notification Correlation ID, time stamp.

Inputs, Optional: Requested Data or Historical Analytics, Fetch Correlation ID, target address where the data or analytics may be retrieved in case the Fetch Correlation ID is included in the notification, unsuccessful bulked data generation, expired bulked data deadline.

NOTE:	The Requested Data or Target address can be provided per Event ID or per analytics ID specified in the Data Specification field of the subscription. The Requested Historical Analytics or Target address can be provided per analytics ID specified in the Analytics Specification field of the subscription.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

7.4.5	Nnwdaf_DataManagement_Fetch service operation

Service operation name: Nnwdaf_DataManagement_Fetch.

Description: Consumer requests to NWDAF to retrieve the subscribed data or historical analytics which is regarded as a kind of data by such Consumer.

Inputs, Required: Notification Correlation ID, list of Fetch Correlation IDs.

Inputs, Optional: None.

Outputs, Required: Requested data or Historical Analytics.

NOTE:	The requested data can be provided per Event ID or Analytics ID specified in the Data Specification field of the subscription. The Requested Historical Analytics can be provided per analytics ID specified in the Analytics Specification field of the subscription.

Outputs, Optional: None.

*************** End of changes ***************