SA WG2 Meeting #147E	S2-2108005

Oct 18 - 22, 2021, Electronic	(revision of S2-2107404)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0014 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multicast session leave and session release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, ZTE, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - There are service operations and parameters to be aligned with the MB-SMF service definition in multicast session leave and session release procedures. - There are more scenarios for network initiated multicast session leave or session release, which need to be specified. And the procedure of SMF removing joined UEs from MBS session is better to be extended to accommodate those scenarios. - Other modifications to the procedure description are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update service operations and parameters in multicast session leave and session release procedures; - Modify SMF removing joined UEs from MBS session procedures, to accommodate more scenarios for network initiated multicast session leave or session release; - Other modifications, e.g. changing the "MBS Session" in sub-clause titles to "multicast session" (as in clause 7.2.1). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unaligned service operations and uncovered scenarios for multicast session leave and session release. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.1, 7.2.2.3 |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change ********************

7.2.2.1	General

MBS Session Leave requested by the UE procedure is used by UEs to inform the 5GC of the UE leaving an multicast MBS Session.  The network can also request the UE to leave an multicast MBS Session, or request the release of an multicast MBS Session.

******************** 2nd Change ********************

7.2.2.3	Multicast session leave or session release requested by the network

This procedure applies to the following scenarios:

1.	When the MB-SMF decides to release an MBS Session:

-	based on a request from the AF (directly or via the NEF/MBSF); or

-	based on local policy (e.g. for load rebalancing).

2.	When the SMF decides to remove a UE from an MBS session:

-	based on a request from the UDM (subscription change) or from the CHF; or

-	if the SMF receives an event notification from the AMF that the UE is out of the MBS service area for local MBS services.

When the SMF receives the notification indicating multicast session release from the MB-SMF, the SMF initiates procedures to remove the joined UEs from the MBS session.

For the active MBS session, to release radio resources as early as possible, the MB-SMF may trigger Multicast Session Deactivation towards the NG-RAN as specified in steps 5-9 of clause 7.2.5.3, prior to or in parallel with triggering MBS Session Release to the SMF.

Figure 7.2.2.3-1: Multicast session leave or session release requested by the network

1.	For MB-SMF triggered MBS session release, the SMF receives Nmbsmf_MBSSession_ContextStatusNotify (MBS Session ID, multicast session release) from the MB-SMF with MBS Session ID. The SMF checks joined UEs.

2.	For UEs without activated UP, the SMF may perform the same procedure as defined in step 3-7 in clause 7.2.5.2.

Alternatively, for UEs without activated UP, the SMF does not trigger a message to the AMF, instead the SMF marks that the UE is to be informed of the MBS Session release. In this case,

the SMF initiates PDU Session Modification to inform the UE of the MBS Session release at next UP activation of the associated PDU Session, if needed.

3.	For the joined UEs with UP activated, the SMF invokes Namf_Communicate_N1N2MessageTransfer to the AMF. The N1 SM container indicates MBS session release. In N2 SM information, the SMF informs the NG-RAN to remove the UE from the MBS session. If there are associated QoS Flow(s) for individual delivery, the SMF also releases those QoS Flow(s) as specified in TS 23.502 [6] clause 4.3.3.2.

4.	The AMF sends N2 Request to the NG-RAN.

5.	The NG-RAN transports the N1 SM container (PDU Session Modification Command (MBS Session ID, leave request)) to the UE.

6.	The NG-RAN performs radio resource modification. If there are no joined UEs in the MBS session, the NG-RAN releases the radio resources.

7.	If there are no joined UEs in the MBS session, for unicast transport of N3mb, the NG-RAN initiates the DL tunnel release towards MB-UPF via AMF and MB-SMF. For multicast transportation of N3mb, the NG-RAN performs IGMP/MLD Leave for the MBS session. See clause 7.2.2.4 for details.

8.	The NG-RAN sends N2 Response to the AMF. If there are no joined UEs in the MBS session, the MBS Session context is removed from the NG-RAN.

9.	The AMF transfers the N2 message received in step 8 to the SMF via the Nsmf_PDUSession_UpdateSMContext service operation. The SMF removes the UE from the MBS Session.

******************** End of Change ********************