SA WG2 Meeting #S2-147E  	S2-2108128

# 18 - 22 October, 2021, Electronic meeting

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3245 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on TSN AF and TSCTSF parameter handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The description in 5.28.3.1 could be misunderstood that the TSN AF and TSCTSF maintains an association with the UE MAC address, while in fact it is the DS-TT MAC address. This is clear in other parts of the spec, i.e. section 5.28.2 states: “The SMF report the MAC address of the DS-TT port of the related PDU Session to TSN AF via PCF. The association between the DS-TT MAC address, 5GS Bridge ID and port number on DS-TT is maintained at TSN AF and further used to assist to bind the TSN traffic with the UE's PDU session.” Further, the current text is not clear that for IP type PDU sessions only the TSCTSF is applicable and not the TSN AF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the MAC address in the TSN AF or TSCTSF is the DS-TT MAC address that is used in the association. Clarify that in case of IP type PDU Sessions, only the TSCTSF is applicable and not the TSN AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous and inconsistent specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.3.1, 5.28.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First Change *****

#### 5.28.3.1	General

Port number of the DS-TT for the PDU Session is assigned by the UPF during PDU session establishment. The port number of the DS-TT port for a PDU Session shall be reported to the SMF from the UPF and further stored at the SMF. The SMF provides the DS-TT port number via PCF to the TSN AF or TSCTSF. TSN AF or TSCTSF maintains an association between the DS-TT port number and the DS-TT port MAC address (with Ethernet type PDU session) or IP address (applicable for TSCTSF only, with IP type PDU Session) of the UE. If a PDU session for which SMF has reported a DS-TT port number to TSN AF or TSCTSF is released, then SMF informs TSN AF or TSCTSF accordingly.

NOTE:	Port number can refer either to Ethernet port or PTP port. In Ethernet type PDU Sessions, it is assumed that the PTP port number is the same as the associated Ethernet port number.

5GS shall support transfer of standardized and deployment-specific port management information transparently between TSN AF or TSCTSF and DS-TT or NW-TT, respectively inside a Port Management Information Container. NW-TT may support one or more ports. In this case, each port uses separate Port Management Information Container. 5GS shall also support transfer of standardized and deployment-specific user plane node management information transparently between TSN AF or TSCTSF and NW-TT, respectively inside a User Plane Node Management Information Container. Table 5.28.3.1-1 and Table 5.28.3.1-2 list standardized port management information and user plane node management information, respectively.

If TSN AF is deployed, i.e. if 5GS is integrated with an IEEE TSN network, the port and user plane node management information is exchanged between CNC and TSN AF. The port management information is related to ports located in DS-TT or NW-TT. The user plane node management information container is related to 5GS bridge management.

If TSN AF is not deployed, the port and user plane node management information is exchanged between TSCTSF and DS-TT/NW-TT.

Table 5.28.3.1-1: Standardized port management information

| Port management information | Applicability (see NOTE 6) |  | Supported operations by TSN AF | Supported operations by TSCTSF | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  |  | (see NOTE 1) | (see NOTE 1) |  |
| General |  |  |  |  |  |
| Port management capabilities (see NOTE 2) | X | X | R | R |  |
| Bridge delay related information |  |  |  |  |  |
| txPropagationDelay | X | X | R | - | IEEE Std 802.1Qcc [95] clause 12.32.2.1 |
| txPropagationDelayDeltaThreshold (see NOTE 23) | X | X | RW |  |  |
| Traffic class related information |  |  |  |  |  |
| Traffic class table | X | X | RW | - | IEEE Std 802.1Q [98] clause 12.6.3 and clause 8.6.6. |
| Gate control information |  |  |  |  |  |
| GateEnabled | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-29 |
| AdminBaseTime | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-29 |
| AdminControlList | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-29 |
| AdminCycleTime (see NOTE 3) | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-29 |
| AdminControlListLength (see NOTE 3) | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-28 |
| Tick granularity | X | X | R | - | IEEE Std 802.1Q [98] Table 12-29 |
| General Neighbor discovery configuration (NOTE 4) |  |  |  |  |  |
| adminStatus | D | X | RW | - | IEEE Std 802.1AB [97] clause 9.2.5.1 |
| lldpV2LocChassisIdSubtype | D | X | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocChassisId | D | X | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxInterval | D | X | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxHoldMultiplier | D | X | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| NW-TT port neighbor discovery configuration |  |  |  |  |  |
| lldpV2LocPortIdSubtype |  | X | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocPortId |  | X | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| DS-TT port neighbor discovery configuration |  |  |  |  |  |
| lldpV2LocPortIdSubtype | D |  | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocPortId | D |  | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| Neighbor discovery information for each discovered neighbor of NW-TT |  |  |  |  |  |
| lldpV2RemChassisIdSubtype |  | X | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId |  | X | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype |  | X | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemPortId |  | X | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| TTL |  | X | R | - | IEEE Std 802.1AB [97] clause 8.5.4 |
| Neighbor discovery information for each discovered neighbor of DS-TT (NOTE 5) |  |  |  |  |  |
| lldpV2RemChassisIdSubtype | D |  | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId | D |  | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype | D |  | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemPortId | D |  | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| TTL | D |  | R | - | IEEE Std 802.1AB [97] clause 8.5.4.1 |
| Stream Parameters (NOTE 11) |  |  |  |  |  |
| MaxStreamFilterInstances | X |  | R | - | IEEE Std 802.1Q [98] clause 12.31.1.1 |
| MaxStreamGateInstances | X |  | R | - | IEEE Std 802.1Q [98] clause 12.31.1.2 |
| MaxFlowMeterInstances | X |  | R | - | IEEE Std 802.1Q [98] clause 12.31.1.3 |
| SupportedListMax | X |  | R | - | IEEE Std 802.1Q [98] clause 12.31.1.4 |
| Per-Stream Filtering and Policing information (NOTE 10) |  |  |  |  |  |
| Stream Filter Instance Table (NOTE 8) |  |  |  | - | IEEE Std 802.1Q [98] Table 12-32 |
| > StreamFilterInstanceIndex | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-32 |
| > Stream Identification type | X | X | RW | - | IEEE 802.1CB [83] clause 9.1.1.6 |
| > Stream Identification Controlling Parameters | X | X | RW | - | IEEE 802.1CB [83] clauses 9.1.2, 9.1.3, 9.1.4 (NOTE 12) |
| > PrioritySpec | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-32 |
| > StreamGateInstanceID | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-32 |
| Stream Gate Instance Table (NOTE 9) |  |  |  |  | IEEE Std 802.1Q [98] Table 12-33 |
| StreamGateInstanceIndex | X | X | R | - | IEEE Std 802.1Q [98] Table 12-33 |
| PSFPAdminBaseTime | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-33 |
| PSFPAdminControlList | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-33 |
| PSFPAdminCycleTime | X | X | RW | - | IEEE Std 802.1Q [98] Table 12-33 |
| PSFPTickGranularity | X | X | R | - | IEEE Std 802.1Q [98] Table 12-33 |
| Time Synchronization Information |  |  |  |  |  |
| TSN Time domain number (NOTE 24) | X | X | RW |  |  |
| Supported PTP instance types (NOTE 13) | X |  | R | R | IEEE Std 1588 [126] clause 8.2.1.5.5 |
| Supported transport types (NOTE 14) | X |  | R | R |  |
| Supported delay mechanisms (NOTE 15) | X |  | R | R | IEEE Std 1588 [126] clause 8.2.15.4.4 |
| PTP grandmaster capable (NOTE 16) | X |  | R | R |  |
| gPTP grandmaster capable (NOTE 17) | X |  | R | R |  |
| Supported PTP profiles (NOTE 18) | X |  | R | R |  |
| Number of supported PTP instances | X |  | R | R |  |
| PTP Instance ID | X | X | RW | RW |  |
| > PTP profile (NOTE 19) | X | X | RW | RW |  |
| > Transport type (NOTE 20) | X | X | RW | RW |  |
| > Grandmaster candidate enabled |  | X | RW | RW |  |
| > Grandmaster enabled (NOTE 21) | X |  | RW | RW |  |
| IEEE Std 1588 [126] data sets (NOTE 22) |  |  |  |  |  |
| > defaultDS.clockIdentity | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.1.2.2 |
| > defaultDS.clockQuality.clockClass | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.1.3.1.2 |
| > defaultDS.clockQuality.clockAccuracy | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.1.3.1.3 |
| > defaultDS.clockQuality.offsetScaledLogVariance | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.1.3.1.4 |
| > defaultDS.priority1 | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.1.4.1 |
| > defaultDS.priority2 | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.1.4.2 |
| > defaultDS.domainNumber | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.1.4.3 |
| > defaultDS.sdoId | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.1.4.5 |
| > defaultDS.instanceEnable | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.1.5.2 |
| > defaultDS.externalPortConfigurationEnabled |  | X | RW | RW | IEEE Std 1588 [126] clause 8.2.1.5.3 |
| > defaultDS.instanceType | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.1.5.5 |
| > portDS.portIdentity | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.2.1 |
| > portDS.portState | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.3.1 |
| > portDS.logMinDelayReqInterval | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.3.2 |
| > portDS.logAnnounceInterval | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.1 |
| > portDS.announceReceiptTimeout |  | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.2 |
| > portDS.logSyncInterval | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.3 |
| > portDS.delayMechanism | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.4 |
| > portDS.logMinPdelayReqInterval | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.5 |
| > portDS.versionNumber | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.6 |
| > portDS.minorVersionNumber | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.7 |
| > portDS.delayAssymetry | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.8 |
| > portDS.portEnable | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.15.5.1 |
| > timePropertiesDS.currentUtcOffset | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.4.2 |
| > timePropertiesDS.timeSource | X | X | RW | RW | IEEE Std 1588 [126] clause 8.2.4.9 |
| > externalPortConfigurationPortDS.desiredState |  | X | RW | RW | IEEE Std 1588 [126] clause 15.5.3.7.15.1 |
| IEEE Std 802.1AS [104] data sets (NOTE 22) |  |  |  | RW |  |
| > defaultDS.clockIdentity | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.2 |
| > defaultDS.clockQuality.clockClass | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.2 |
| > defaultDS.clockQuality.clockAccuracy | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.3 |
| > defaultDS.clockQuality.offsetScaledLogVariance | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.4 |
| > defaultDS.priority1 | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.5 |
| > defaultDS.priority2 | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.6 |
| > defaultDS.timeSource | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.15 |
| > defaultDS.domainNumber | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.16 |
| > defaultDS.sdoId | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.3 |
| > defaultDS.externalPortConfigurationEnabled |  | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.18 |
| > defaultDS.instanceEnable | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.2.19 |
| > portDS.portIdentity | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.2 |
| > portDS.portState |  | X | R | R | IEEE Std 802.1AS [104] clause 14.8.3 |
| > portDS.ptpPortEnabled | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.4 |
| > portDS.delayMechanism | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.5 |
| > portDS.isMeasuringDelay | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.6 |
| > portDS.asCapable | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.7 |
| > portDS.meanLinkDelay | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.8 |
| > portDS.meanLinkDelayThresh | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.9 |
| > portDS.delayAssymetry | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.10 |
| > portDS.neighborRateRatio | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.11 |
| > portDS.initialLogAnnounceInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.12 |
| > portDS.currentLogAnnounceInterval | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.13 |
| > portDS.useMgtSettableLogAnnounceInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.14 |
| > portDS.mgtSettableLogAnnounceInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.15 |
| > portDS.announceReceiptTimeout |  | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.16 |
| > portDS.initialLogSyncInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.17 |
| > portDS.currentLogSyncInterval | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.18 |
| > portDS.useMgtSettableLogSyncInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.19 |
| > portDS.mgtSettableLogSyncInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.20 |
| > portDS.syncReceiptTimeout |  | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.21 |
| > portDS.syncReceiptTimeoutTimeInterval |  | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.22 |
| > portDS.initialLogPdelayReqInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.23 |
| > portDS.currentLogPdelayReqInterval | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.24 |
| > portDS.useMgtSettableLogPdelayReqInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.25 |
| > portDS.mgtSettableLogPdelayReqInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.26 |
| > portDS.initialLogGptpCapableMessageInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.27 |
| > portDS.currentLogGptpCapableMessageInterval | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.28 |
| > portDS.useMgtSettableLogGptpCapableMessageInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.29 |
| > portDS.mgtSettableLogGptpCapableMessageInterval | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.30 |
| > portDS.initialComputeNeighborRateRatio | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.31 |
| > portDS.currentComputeNeighborRateRatio | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.32 |
| > portDS.useMgtSettableComputeNeighborRateRatio | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.33 |
| > portDS.mgtSettableComputeNeighborRateRatio | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.34 |
| > portDS.initialComputeMeanLinkDelay | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.35 |
| > portDS.currentComputeMeanLinkDelay | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.36 |
| > portDS.useMgtSettableComputeMeanLinkDelay | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.37 |
| > portDS.mgtSettableComputeMeanLinkDelay | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.38 |
| > portDS.allowedLostResponses | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.39 |
| > portDS.allowedFaults | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.40 |
| > portDS.gPtpCapableReceiptTimeout | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.41 |
| > portDS.versionNumber | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.42 |
| > portDS.nup | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.43 |
| > portDS.ndown | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.44 |
| > portDS.oneStepTxOper | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.45 |
| > portDS.oneStepReceive | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.46 |
| > portDS.oneStepTransmit | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.47 |
| > portDS.initialOneStepTxOper | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.48 |
| > portDS.currentOneStepTxOper | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.49 |
| > portDS.useMgtSettableOneStepTxOper | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.50 |
| > portDS.mgtSettableOneStepTxOper | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.51 |
| > portDS.syncLocked | X | X | R | R | IEEE Std 802.1AS [104] clause 14.8.52 |
| > portDS.pdelayTruncatedTimestampsArray | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.53 |
| > portDS.minorVersionNumber | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.8.54 |
| > timePropertiesDS.currentUtcOffset | X | X | RW | RW | IEEE Std 802.1AS [104] clause 14.5.2 |
| > externalPortConfigurationPortDS.desiredState |  | R | RW | RW | IEEE Std 802.1AS [104] clause 14.12.2 |
| NOTE 1: R = Read only access; RW = Read/Write access; ― = not supported. NOTE 2: Indicates which standardized and deployment-specific port management information is supported by DS-TT or NW-TT. NOTE 3: AdminCycleTime and AdminControlListLength are optional for gate control information. NOTE 4: If DS-TT supports neighbor discovery, then TSN AF sends the general neighbor discovery configuration for DS-TT Ethernet ports to DS-TT. If DS-TT does not support neighbor discovery, then TSN AF sends the general neighbor discovery configuration for DS-TT Ethernet ports to NW-TT using the Bridge Management Information Container (refer to Table 5.28.3.1-2) and NW-TT performs neighbor discovery on behalf on DS-TT. When a parameter in this group is changed, it is necessary to provide the change to every DS-TT and the NW-TT that belongs to the 5GS TSN bridge. It is mandatory that the general neighbor discovery configuration is identical for all DS-TTs and the NW-TTs that belongs to the bridge. NOTE 5: If DS-TT supports neighbor discovery, then TSN AF retrieves neighbor discovery information for DS-TT Ethernet ports from DS-TT. If DS-TT does not support neighbor discovery, then TSN AF retrieves neighbor discovery information for DS-TT Ethernet ports from NW-TT, using the Bridge Management Information Container (refer to Table 5.28.3.1-2), the NW-TT performing neighbor discovery on behalf on DS-TT. NOTE 6: X = applicable; D = applicable when validation and generation of LLDP frames is processed at the DS-TT. NOTE 7: Void. NOTE 8: There is a Stream Filter Instance Table per Stream. NOTE 9: There is a Stream Gate Instance Table per Gate. NOTE 10: TSN AF indicates the support for PSFP to the CNC only if each DS-TT and NW-TT of the 5GS bridge has indicated support of PSFP. DS-TT indicates support of PSFP using port management capabilities, i.e. by indicating support for the Per-Stream Filtering and Policing information and by setting higher than zero values for MaxStreamFilterInstances, MaxStreamGateInstances, MaxFlowMeterInstances, SupportedListMax parameters. When available, TSN AF uses the PSFP information for determination of the traffic pattern information as described in Annex I. The PSFP information can be used at the DS-TT (if supported) and at the NW-TT (if supported) for the purpose of per-stream filtering and policing as defined in clause 8.6.5.1 of IEEE Std 802.1Q [98]. NOTE 11: TSN AF composes a Stream Parameter Table towards the CNC. It is up to TSN AF how it composes the Stream Parameter Table based on the numerical values as received from DS-TT and NW-TT port(s) and for the bridge for each individual parameter. NOTE 12: The set of Stream Identification Controlling Parameters depends on the Stream Identification type value as defined in IEEE Std 802.1CB [83] Table 9-1 and clauses 9.1.2, 9.1.3, 9.1.4. NOTE 13: Enumeration of supported PTP instance types. Allowed values as defined in clause 8.2.1.5.5 of IEEE Std 1588 [126]. NOTE 14: Enumeration of supported transport types. Allowed values: IPv4 (as defined in Annex C of IEEE Std 1588 [126]), IPv6 (as defined in IEEE Std 1588 [126] Annex D), Ethernet (as defined in Annex E of IEEE Std 1588 [126]). NOTE 15: Enumeration of supported PTP delay mechanisms. Allowed values as defined in clause 8.2.15.4.4 of IEEE Std 1588 [126]. NOTE 16: Indicates whether DS-TT supports acting as a PTP grandmaster. NOTE 17: Indicates whether DS-TT supports acting as a gPTP grandmaster. NOTE 18: Enumeration of supported PTP profiles, each identified by PTP profile ID, as defined in clause 20.3.3 of IEEE Std 1588 [126]. NOTE 19: PTP profile to apply, identified by PTP profile ID, as defined in clause 20.3.3 of IEEE Std 1588 [126]. NOTE 20: Transport type to use. Allowed values: IPv4 (as defined in Annex C of IEEE Std 1588 [126]), IPv6 (as defined in IEEE Std 1588 [126] Annex D), Ethernet (as defined in Annex E of IEEE Std 1588 [126]). NOTE 21: Indicates whether to act as grandmaster or not, i.e. whether to send Announce, Sync and optionally Follow_Up messages. NOTE 22: The IEEE Std 802.1AS [104] data sets apply if the IEEE 802.1AS PTP profile is used; otherwise the IEEE Std 1588 [126] data sets apply. NOTE 23: Indicates how much the txPropagationDelay needs to change so that DS-TT/NW-TT report a change in txPropagationDelay to TSN AF. This is optional for NW-TT. NOTE 24: Indicates the (g)PTP domain (identified by a domain number) that is assumed by the CNC as the reference clock for time information in the scheduled traffic (gate control) information, PSFP information and bridge delay related information. This is optional for NW-TT. |  |  |  |  |  |

Table 5.28.3.1-2: Standardized user plane node management information

| User plane node management information | Supported operations by TSN AF | Supported operations by TSCTSF | Reference |
| --- | --- | --- | --- |
|  | (see NOTE 1) | (see NOTE 1) |  |
| Information for 5GS Bridge |  |  |  |
| User plane node Address | R | R |  |
| User plane node ID | R | R |  |
| NW-TT port numbers | R | R |  |
| Traffic forwarding information |  |  |  |
| Static Filtering Entry (NOTE 3) | RW | - | IEEE Std 802.1Q [98] clause 8.8.1 |
| General Neighbor discovery configuration (NOTE 2) |  |  |  |
| adminStatus | RW | - | IEEE Std 802.1AB [97] clause 9.2.5.1 |
| lldpV2LocChassisIdSubtype | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocChassisId | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxInterval | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxHoldMultiplier | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| DS-TT port neighbor discovery configuration for DS-TT ports (NOTE 4) |  |  |  |
| >DS-TT port neighbor discovery configuration for each DS-TT port |  |  |  |
| >> DS-TT port number | RW | - |  |
| >> lldpV2LocPortIdSubtype | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2LocPortId | RW | - | IEEE Std 802.1AB [97] Table 11-2 |
| Discovered neighbor information for DS-TT ports (NOTE 4) |  |  |  |
| >Discovered neighbor information for each DS-TT port (NOTE 4) |  |  |  |
| >> DS-TT port number | R | - |  |
| >> lldpV2RemChassisIdSubtype | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2RemChassisId | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2RemPortIdSubtype | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2RemPortId | R | - | IEEE Std 802.1AB [97] Table 11-2 |
| >> TTL | R | - | IEEE Std 802.1AB [97] clause 8.5.4.1 |
| Stream Parameters (NOTE 5) |  |  |  |
| MaxStreamFilterInstances | R | - | IEEE Std 802.1Q [98] |
| MaxStreamGateInstances | R | - | IEEE Std 802.1Q [98] |
| MaxFlowMeterInstances | R | - | IEEE Std 802.1Q [98] |
| SupportedListMax | R | - | IEEE Std 802.1Q [98] |
| Time synchronization information |  |  |  |
| Supported PTP instance types (NOTE 6) | R | R |  |
| Supported transport types (NOTE 7) | R | R |  |
| Supported delay mechanisms (NOTE 8) | R | R |  |
| PTP grandmaster capable (NOTE 9) | R | R |  |
| gPTP grandmaster capable (NOTE 10) | R | R |  |
| Supported PTP profiles (NOTE 11) | R | R |  |
| Number of supported PTP instances | R | R |  |
| Time synchronization information for DS-TT ports |  |  |  |
| > Time synchronization information for each DS-TT port |  |  |  |
| > DS-TT port number | RW | RW |  |
| >> Time synchronization information for each PTP Instance |  |  |  |
| >> PTP Instance ID | RW | RW |  |
| >> PTP profile (NOTE 12) | RW | RW |  |
| >> Transport type (NOTE 13) | RW | RW |  |
| >> Grandmaster on behalf of DS-TT enabled (NOTE 14) | RW | RW |  |
| IEEE Std 1588 [126] data sets (NOTE 15) |  |  |  |
| >> defaultDS.clockIdentity | RW | RW | IEEE Std 1588 [126] clause 8.2.1.2.2 |
| >> defaultDS.clockQuality.clockClass | RW | RW | IEEE Std 1588 [126] clause 8.2.1.3.1.2 |
| >> defaultDS.clockQuality.clockAccuracy | RW | RW | IEEE Std 1588 [126] clause 8.2.1.3.1.3 |
| >> defaultDS.clockQuality.offsetScaledLogVariance | RW | RW | IEEE Std 1588 [126] clause 8.2.1.3.1.4 |
| >> defaultDS.priority1 | RW | RW | IEEE Std 1588 [126] clause 8.2.1.4.1 |
| >> defaultDS.priority2 | RW | RW | IEEE Std 1588 [126] clause 8.2.1.4.2 |
| >> defaultDS.domainNumber | RW | RW | IEEE Std 1588 [126] clause 8.2.1.4.3 |
| >> defaultDS.sdoId | RW | RW | IEEE Std 1588 [126] clause 8.2.1.4.5 |
| >> defaultDS.instanceEnable | RW | RW | IEEE Std 1588 [126] clause 8.2.1.5.2 |
| >> defaultDS.externalPortConfigurationEnabled | RW | RW | IEEE Std 1588 [126] clause 8.2.1.5.3 |
| >> defaultDS.instanceType | RW | RW | IEEE Std 1588 [126] clause 8.2.1.5.5 |
| >> portDS.portIdentity | RW | RW | IEEE Std 1588 [126] clause 8.2.15.2.1 |
| >> portDS.portState | R | R | IEEE Std 1588 [126] clause 8.2.15.3.1 |
| >> portDS.logMinDelayReqInterval | RW | RW | IEEE Std 1588 [126] clause 8.2.15.3.2 |
| >> portDS.logAnnounceInterval | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.1 |
| >> portDS.announceReceiptTimeout | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.2 |
| >> portDS.logSyncInterval | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.3 |
| >> portDS.delayMechanism | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.4 |
| >> portDS.logMinPdelayReqInterval | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.5 |
| >> portDS.versionNumber | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.6 |
| >> portDS.minorVersionNumber | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.7 |
| >> portDS.delayAssymetry | RW | RW | IEEE Std 1588 [126] clause 8.2.15.4.8 |
| >> portDS.portEnable | RW | RW | IEEE Std 1588 [126] clause 8.2.15.5.1 |
| >> timePropertiesDS.currentUtcOffset | RW | RW | IEEE Std 1588 [126] clause 8.2.4.2 |
| >> timePropertiesDS.timeSource | RW | RW | IEEE Std 1588 [126] clause 8.2.4.9 |
| >> externalPortConfigurationPortDS.desiredState | RW | RW | IEEE Std 1588 [126] clause 15.5.3.7.15.1 |
| IEEE Std 802.1AS [104] data sets (NOTE 15) |  |  |  |
| >> defaultDS.clockIdentity | RW | RW | IEEE Std 802.1AS [104] clause 14.2.2 |
| >> defaultDS.clockQuality.clockClass | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.2 |
| >> defaultDS.clockQuality.clockAccuracy | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.3 |
| >> defaultDS.clockQuality.offsetScaledLogVariance | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.4 |
| >> defaultDS.priority1 | RW | RW | IEEE Std 802.1AS [104] clause 14.2.5 |
| >> defaultDS.priority2 | RW | RW | IEEE Std 802.1AS [104] clause 14.2.6 |
| >> defaultDS.timeSource | RW | RW | IEEE Std 802.1AS [104] clause 14.2.15 |
| >> defaultDS.domainNumber | RW | RW | IEEE Std 802.1AS [104] clause 14.2.16 |
| >> defaultDS.externalPortConfigurationEnabled | RW | RW | IEEE Std 802.1AS [104] clause 14.2.18 |
| >> defaultDS.sdoId | RW | RW | IEEE Std 802.1AS [104] clause 14.2.4.3 |
| >> defaultDS.instanceEnable | RW | RW | IEEE Std 802.1AS [104] clause 14.2.19 |
| >> portDS.portIdentity | RW | RW | IEEE Std 802.1AS [104] clause 14.8.2 |
| >> portDS.portState | R | R | IEEE Std 802.1AS [104] clause 14.8.3 |
| >> portDS.ptpPortEnabled | RW | RW | IEEE Std 802.1AS [104] clause 14.8.4 |
| >> portDS.delayMechanism | RW | RW | IEEE Std 802.1AS [104] clause 14.8.5 |
| >> portDS.isMeasuringDelay | R | R | IEEE Std 802.1AS [104] clause 14.8.6 |
| >> portDS.asCapable | R | R | IEEE Std 802.1AS [104] clause 14.8.7 |
| >> portDS.meanLinkDelay | R | R | IEEE Std 802.1AS [104] clause 14.8.8 |
| >> portDS.meanLinkDelayThresh | RW | RW | IEEE Std 802.1AS [104] clause 14.8.9 |
| >> portDS.delayAssymetry | RW | RW | IEEE Std 802.1AS [104] clause 14.8.10 |
| >> portDS.neighborRateRatio | R | R | IEEE Std 802.1AS [104] clause 14.8.11 |
| >> portDS.initialLogAnnounceInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.12 |
| >> portDS.currentLogAnnounceInterval | R | R | IEEE Std 802.1AS [104] clause 14.8.13 |
| >> portDS.useMgtSettableLogAnnounceInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.14 |
| >> portDS.mgtSettableLogAnnounceInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.15 |
| >> portDS.announceReceiptTimeout | RW | RW | IEEE Std 802.1AS [104] clause 14.8.16 |
| >> portDS.initialLogSyncInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.17 |
| >> portDS.currentLogSyncInterval | R | R | IEEE Std 802.1AS [104] clause 14.8.18 |
| >> portDS.useMgtSettableLogSyncInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.19 |
| >> portDS.mgtSettableLogSyncInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.20 |
| >> portDS.syncReceiptTimeout | RW | RW | IEEE Std 802.1AS [104] clause 14.8.21 |
| >> portDS.syncReceiptTimeoutTimeInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.22 |
| >> portDS.initialLogPdelayReqInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.23 |
| >> portDS.currentLogPdelayReqInterval | R | R | IEEE Std 802.1AS [104] clause 14.8.24 |
| >> portDS.useMgtSettableLogPdelayReqInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.25 |
| >> portDS.mgtSettableLogPdelayReqInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.26 |
| >> portDS.initialLogGptpCapableMessageInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.27 |
| >> portDS.currentLogGptpCapableMessageInterval | R | R | IEEE Std 802.1AS [104] clause 14.8.28 |
| >> portDS.useMgtSettableLogGptpCapableMessageInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.29 |
| >> portDS.mgtSettableLogGptpCapableMessageInterval | RW | RW | IEEE Std 802.1AS [104] clause 14.8.30 |
| >> portDS.initialComputeNeighborRateRatio | RW | RW | IEEE Std 802.1AS [104] clause 14.8.31 |
| >> portDS.currentComputeNeighborRateRatio | R | R | IEEE Std 802.1AS [104] clause 14.8.32 |
| >> portDS.useMgtSettableComputeNeighborRateRatio | RW | RW | IEEE Std 802.1AS [104] clause 14.8.33 |
| >> portDS.mgtSettableComputeNeighborRateRatio | RW | RW | IEEE Std 802.1AS [104] clause 14.8.34 |
| >> portDS.initialComputeMeanLinkDelay | RW | RW | IEEE Std 802.1AS [104] clause 14.8.35 |
| >> portDS.currentComputeMeanLinkDelay | R | R | IEEE Std 802.1AS [104] clause 14.8.36 |
| >> portDS.useMgtSettableComputeMeanLinkDelay | RW | RW | IEEE Std 802.1AS [104] clause 14.8.37 |
| >> portDS.mgtSettableComputeMeanLinkDelay | RW | RW | IEEE Std 802.1AS [104] clause 14.8.38 |
| >> portDS.allowedLostResponses | RW | RW | IEEE Std 802.1AS [104] clause 14.8.39 |
| >> portDS.allowedFaults | RW | RW | IEEE Std 802.1AS [104] clause 14.8.40 |
| >> portDS.gPtpCapableReceiptTimeout | RW | RW | IEEE Std 802.1AS [104] clause 14.8.41 |
| >> portDS.versionNumber | RW | RW | IEEE Std 802.1AS [104] clause 14.8.42 |
| >> portDS.nup | RW | RW | IEEE Std 802.1AS [104] clause 14.8.43 |
| >> portDS.ndown | RW | RW | IEEE Std 802.1AS [104] clause 14.8.44 |
| >> portDS.oneStepTxOper | R | R | IEEE Std 802.1AS [104] clause 14.8.45 |
| >> portDS.oneStepReceive | R | R | IEEE Std 802.1AS [104] clause 14.8.46 |
| >> portDS.oneStepTransmit | R | R | IEEE Std 802.1AS [104] clause 14.8.47 |
| >> portDS.initialOneStepTxOper | RW | RW | IEEE Std 802.1AS [104] clause 14.8.48 |
| >> portDS.currentOneStepTxOper | RW | RW | IEEE Std 802.1AS [104] clause 14.8.49 |
| >> portDS.useMgtSettableOneStepTxOper | RW | RW | IEEE Std 802.1AS [104] clause 14.8.50 |
| >> portDS.mgtSettableOneStepTxOper | RW | RW | IEEE Std 802.1AS [104] clause 14.8.51 |
| >> portDS.syncLocked | R | R | IEEE Std 802.1AS [104] clause 14.8.52 |
| >> portDS.pdelayTruncatedTimestampsArray | RW | RW | IEEE Std 802.1AS [104] clause 14.8.53 |
| >> portDS.minorVersionNumber | RW | RW | IEEE Std 802.1AS [104] clause 14.8.54 |
| >> timePropertiesDS.currentUtcOffset | RW | RW | IEEE Std 802.1AS [104] clause 14.5.2 |
| >> externalPortConfigurationPortDS.desiredState | RW | RW | IEEE Std 802.1AS [104] clause 14.12.2 |
| NOTE 1: R = Read only access; RW = Read/Write access; ― = not supported. NOTE 2: General neighbor discovery information is included only when NW-TT performs neighbor discovery on behalf of DS-TT. When a parameter in this group is changed, it is necessary to provide the change to every DS-TT and the NW-TT that belongs to the 5GS TSN bridge. NOTE 3: If the Static Filtering Entry information is present for uplink traffic, UPF/NW-TT uses Static Filtering Entry information to determine the NW-TT egress port for forwarding UL TSC traffic. The user plane forwarding is further specified in clause 5.8.2.5.3. NOTE 4: DS-TT discovery configuration and DS-TT discovery information are used only when DS-TT does not support LLDP and NW-TT performs neighbor discovery on behalf of DS-TT. NOTE 5: TSN AF indicates the support for PSFP to the CNC only if each DS-TT and NW-TT of the 5GS bridge have indicated support of PSFP. The support of PSFP at the NW-TT ports is expressed by setting higher than zero values for MaxStreamFilterInstances, MaxStreamGateInstances, MaxFlowMeterInstances, SupportedListMax parameters. NOTE 6: Enumeration of supported PTP instance types. Allowed values as defined in clause 8.2.1.5.5 of IEEE Std 1588 [126]. NOTE 7: Enumeration of supported transport types. Allowed values: IPv4 (as defined in IEEE Std 1588 [126] Annex C), IPv6 (as defined in IEEE Std 1588 [126] Annex D), Ethernet (as defined in Annex E of IEEE Std 1588 [126]). NOTE 8: Enumeration of supported PTP delay mechanisms. Allowed values as defined in clause 8.2.15.4.4 of IEEE Std 1588 [126]. NOTE 9: Indicates whether NW-TT supports acting as a PTP grandmaster. NOTE 10: Indicates whether NW-TT supports acting as a gPTP grandmaster. NOTE 11: Enumeration of supported PTP profiles, each identified by PTP profile ID, as defined in clause 20.3.3 of IEEE Std 1588 [126]. NOTE 12: PTP profile to apply, identified by PTP profile ID, as defined in clause 20.3.3 of IEEE Std 1588 [126]. NOTE 13: Transport type to use. Allowed values: IPv4 (as defined in Annex C of IEEE Std 1588 [126]), IPv6 (as defined in IEEE Std 1588 [126] Annex D), Ethernet (as defined in Annex E of IEEE Std 1588 [126]). NOTE 14: Indicates whether to act as grandmaster on behalf of a DS-TT port or not if 5GS is determined to be the grandmaster clock, i.e. whether to send Announce, Sync and optionally Follow_Up messages on behalf of DS-TT. NOTE 15: The IEEE Std 802.1AS [104] data sets apply if the IEEE 802.1AS PTP profile is used; otherwise the IEEE Std 1588 [126] data sets apply. |  |  |  |

Exchange of port and user plane node management information between TSN AF or TSCTSF and NW-TT or between TSN AF or TSCTSF and DS-TT allows TSN AF or TSCTSF to:

1)	retrieve port management information for a DS-TT or NW-TT port or user plane node management information;

2)	send port management information for a DS-TT or NW-TT port or user plane node management information;

3)	subscribe to and receive notifications if specific port management information for a DS-TT or NW-TT port changes or user plane node management information changes.

Exchange of port management information between TSN AF or TSCTSF and NW-TT or DS-TT is initiated by DS-TT or NW-TT to:

-	notify TSN AF or TSCTSF if port management information has changed that TSN AF or TSCTSF has subscribed for.

Exchange of user plane node management information between TSN AF or TSCTSF and NW-TT is initiated by NW-TT to:

-	notify TSN AF or TSCTSF if user plane node management information has changed that TSN AF or TSCTSF has subscribed for.

Exchange of port management information is initiated by DS-TT to:

-	provide port management capabilities, i.e. provide information indicating which standardized and deployment-specific port management information is supported by DS-TT.

TSN AF or TSCTSF indicates inside the Port Management Information Container or user plane node Management Information Container whether it wants to retrieve or send port or user plane node management information or intends to (un-)subscribe for notifications.

***** Next Change *****

#### 5.28.3.2	Transfer of port or user plane node management information

Port management information is transferred transparently via 5GS between TSN AF or TSCTSF and DS-TT or NW-TT, respectively, inside a Port Management Information Container (PMIC). User plane node management information is transferred transparently via 5GS between TSN AF or TSCTSF and NW-TT inside a user plane node Management Information Container (UMIC). The transfer of port or user plane node management information is as follows:

-	To convey port management information from DS-TT or NW-TT to TSN AF or TSCTSF:

-	DS-TT provides a PMIC and the DS-TT port MAC address (if available) to the UE, which includes the PMIC as an optional Information Element of an N1 SM container and triggers the UE requested PDU Session Establishment procedure or PDU Session Modification procedure to forward the PMIC to the SMF. SMF forwards the PMIC and the port number of the related DS-TT port to TSN AF or TSCTSF as described in clauses 4.3.2.2 and 4.3.3.2 of TS 23.502 [3];

-	NW-TT provides PMIC(s) and/or UMIC to the UPF, which triggers the N4 Session Level Reporting Procedure to forward the PMIC(s) and/or UMIC to SMF. UPF selects an N4 session corresponding to any of the N4 sessions for this NW-TT. SMF in turn forwards the PMIC(s) and the port number(s) of the related NW-TT port(s), or the UMIC, to TSN AF or TSCTSF as described in clause 4.16.5.1 of TS 23.502 [3].

NOTE 1:	There has to be at least one established PDU session for DS-TT port before the UPF can report PMIC/UMIC information towards the TSN AF or TSCTSF.

-	To convey port management information from TSN AF or TSCTSF to DS-TT:

-	TSN AF or TSCTSF provides a PMIC, DS-TT port MAC address or UE IP address (applicable for TSCTSF only) reported for a PDU Session (i.e. MAC address of the DS-TT port or IP address related to the PDU session) and the port number of the DS-TT port to manage to the PCF by using the AF Session level Procedure, which forwards the information to SMF based on the MAC or IP address using the PCF initiated SM Policy Association Modification procedure as described in clause 4.16.5.2 of TS 23.502 [3]. SMF determines that the port number relates to a DS-TT port and based on this forwards the PMIC to DS-TT using the network requested PDU Session Modification procedure as described in clause 4.3.3.2 of TS 23.502 [3].

-	To convey port or user plane node management information from TSN AF or TSCTSF to NW-TT:

-	TSN AF or TSCTSF selects a PCF-AF session corresponding to any of the DS-TT MAC or IP addresses (applicable for TSCTSF only) for the related PDU sessions of this bridge and provides a PMIC(s) and the related NW-TT port number(s) and/or UMIC to the PCF. The PCF uses the PCF initiated SM Policy Association Modification procedure to forward the information received from TSN AF or TSCTSF to SMF as described in clause 4.16.5.2 of TS 23.502 [3]. SMF determines that the included information needs to be delivered to the NW-TT either by determining that the port number(s) relate(s) to a NW-TT port(s) or based on the presence of UMIC, and forwards the container(s) and/or related port number(s) to NW-TT using the N4 Session Modification procedure described in clause 4.4.1.3 of TS 23.502 [3].

***** End of Changes *****