SA WG2 Meeting #S2-147E	S2-2107043

18 - 22 October, 2021, Electronic meeting

3GPP TSG-RAN WG2 Meeting #115-e						R2-2108995

E-Meeting, August 2021

Title:	LS on Tx Profile

Response to:

Release:	Rel-17

Work Item:	NR_SL_enh-Core

Source:	RAN2

To:	SA2, CT1

Cc:

Contact Person:

#### Name:	Qianxi Lu

E-mail Address:	<qianxi.lu@oppo.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

In RAN2#115-e meeting, RAN2 decided to introduce Tx profile and thus made the following agreement. The TX profile is at least used for Rel-17 UE to know whether it should apply SL DRX or not, considering Rel-16 UE does not support SL DRX.

Agreements on TX profiles:

1: 	For GC/BC, TX profile is introduced in Rel-17 for sidelink enhancement. FFS whether a TX profile identifies a Release, or one or more sidelink feature groups.

2:	RAN2 understand a service type can be mapped to a TX profile, i.e. V2X and ProSe.

3:	A TX profile is indicated from upper layer to AS layer. FFS whether a TX profile needs to be provided with service type information or L2 id.

4:	For GC/BC, a Rel-17 TX UE shall only assume SL DRX for the RX UEs when the associated TX profile corresponding to support of SL DRX. FFS whether a TX profile needs to be provided with service type information or L2 id.

5:	For GC/BC only communication, a Rel-17 RX UE determines SL DRX is used if all service types/L2 ids of interest have an associated TX profile corresponding to support of SL DRX. A Rel-17 RX UE enables SL DRX operation for a service type/L2 id with the associated TX profile.

6:	For UC, for SL transmissions after PC5-RRC connection is established, no backward compatibility issue of SL DRX is assumed, i.e. backward compatibility is handled based on PC5-RRC UE capability signalling.

7:	Send an LS to SA2 to inform them of the RAN2 agreements related to TX profile.

2. Actions:

To SA2: RAN2 respectfully requests SA2 to perform the following actions

- Send feedback to RAN2 if any concern on the agreements above;

- Take the above agreements into account in the future work if no concern.

3. Dates of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting #116-e	1 November– 12 November 2021	eMeeting

TSG RAN WG2 Meeting #117	21 February – 25 February 2022	Athens, GR