3GPP TSG-SA2 Meeting #147E	S2-2107168

# Electronic, 18th - 22nd October 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 006 | rev - |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | C2 Authorization for EPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS |  |  |  |  |  | Date: |  |  | 2021-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Finalization of EPS C2 Authorization and alignment with 5GS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected procedures for C2 authorization in EPS and aligned them with 5GS. Corrected name on 5.2.5.2.1 and faulty reference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect description of EPS C2 Authorization. Faulty names and references in 5GS C2 Authorization description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.5.2.1, 5.2.5.3, 5.2.5.3.0 (new), 5.2.5.3.1, 5.2.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------First Change---------------------------------------------

#### 5.2.5.2	Procedure for C2 authorization in 5GS

##### 5.2.5.2.1	C2 Authorization request during UUAA-SM procedure in 5GS

If C2 authorization is requested during the UUAA-SM procedure the procedure described in clause 5.2.3.2 takes place with the following additions:

-	In Step 0, the UE includes pairing information (if available) in a C2 Aviation Payload. which is forwarded further to the USS;

-	In step 4, the USS performs C2 authorization taking into account the included pairing information, the Service Level Device Identity/CAA-Level UAV ID and 3GPP UAV ID/GPSI. The USS includes the resulting C2 Authorization result in the Naf_Auth_Response returned to the UAS-NF/NEF and UAS NF/NEF forwards to the UAV/UE in step 7.

-	The USS shall:

-	in step 4 include a DN Authorization profile Index specifying a predefined set of PCC-rules in the PCF with initial restriction on the type of traffic allowed to pass on the PDU-session. For example, only traffic exchanged with the USS might be allowed to pass.

Once the authentication is complete, after step 4, the USS subscribes to PDU Session Status Events for the PDU session used for C2 communication, applicable for the GPSI received in step 2.

-	when the USS in step 8 receives a PDU Session State Event Report indicating session start and including the PDU Session IP address the USS invokes the USS initiated pairing policy configuration procedure (see figure 5.2.5.4.1-1) with the received PDU Session IP address and authorized paired UAV-C IP-address as input to request corresponding traffic to be allowed on the PDU session in the UPF.

----------------------------------------Next Change---------------------------------------------

#### 5.2.5.3	Procedure for C2 authorization in EPS

##### 5.2.5.3.0	C2 Authorization request during UUAA-SM procedure in EPS

If C2 authorization is requested during the UUAA-SM procedure the procedure described in clause 5.2.3.3 takes place with the following additions:

-	In Step 0, the UE includes pairing information (if available) in a C2 Aviation Payload. which is forwarded further to the USS;

-	In step 3, the USS performs C2 authorization taking into account the included pairing information, the Service Level Device Identity/CAA-Level UAV ID and 3GPP UAV ID/GPSI. The USS includes the resulting C2 Authorization result in the Naf_Auth_Response returned to the UAS-NF/NEF and UAS NF/NEF forwards to the UAV/UE in step 7.

-	The USS shall:

-	in step 4 include a DN Authorization profile Index specifying a predefined set of PCC-rules in the PCF with initial restriction on the type of traffic allowed to pass on the PDN Connection. For example, only traffic exchanged with the USS might be allowed to pass.

Once the authentication is complete, after step 4, the USS subscribes to PDN Connectivity Status Events for the PDN Connection used for C2 communication, applicable for the GPSI received in step 2.

-	when the USS in step 8 receives a PDN Connectivity Status Event Report indicating session start and including the PDN Connection IP address, the USS invokes the USS initiated pairing policy configuration procedure (see figure 5.2.5.4.2-1) with the received PDN Connection IP address and authorized paired UAV-C IP-address as input to request corresponding traffic to be allowed on the PDN Connection in the PGW-U.

##### 5.2.5.3.1	UE requested PDN connectivity for C2 authorization

When the UAV requests to establish connectivity to an additional PDN over E-UTRAN for C2, the procedure described in clause 5.10.2 of TS 23.401 [6] takes place with the following modifications:

Figure 5.2.5.3.1-1: UE requested PDN Connectivity for C2 authorization

0.	The UAV has performed a successful UUAA with the USS (UUAA-SM) and the USS has for the corresponding GPSI subscribed for PDN Connectivity Status Event reports from the NEF.

1.	Steps 1 - 3 performed as in Figure 5.10.2-1 of TS 23.401 [6].

When the UAV needs to establish C2 communication, the UAV determines that a new PDN Connection is required for connectivity to UAV-C. The UE initiates a UE Requested PDN Connectivity procedure for connectivity to UAV-C. In the PCO in the PDN Connectivity Request, the Service Level Device Identity (e.g. the CAA-Level UAV ID) and a C2 aviation payload to be used for C2 authorization shall be included and forwarded to the MME. The pairing information includes the Service Level Device Identity (e.g. CAA-Level UAV IDs) of the requesting UAV and identification information for the UAV-C to pair may be included in C2 aviation payload. The UAV may also include other information such as Flight Authorization information. The USS may also use its locally configured pairing information for UAV - UAV-C pairing authorization which then takes precedence over UAV provided pairing information.

2.	For a UAV with aerial subscription the SMF+PGW-C determines based on the APN/DNN of the PDN Connection and the presence of the Service Level Device Identity (e.g. the CAA-Level UAV ID of the UVA) that authorization is required. The SMF+PGW-C then sends a Nnef_Auth_Request, which is used to request authorization to pair the UAV with UAV-C, to the UAS NF/NEF that includes the GPSI, Service Level Device Identity (e.g. the CAA-Level UAV ID) and C2 Aviation Payload and optionally the UAV location (e.g. Cell ID) if provided by the MME and the APN/DNN of the PDN Connection.

For a UAV with aerial subscription, if the SMF+PGW-C determines based on the APN/DNN of the PDN Connection and the presence of the Service Level Device Identity (e.g. the CAA-Level UAV ID of the UAV) that the authorization procedure with the USS is required, but the UAV has not provided the Service Level Device Identity (e.g. the CAA-Level UAV ID), the SMF+PGW-C rejects the PDN Connectivity Request with a cause indicating that USS authorization is required.

3.	The UAS NF/NEF checks that a valid UUAA is stored for the GPSI and forwards the received authorization request as a Naf_Auth_Request to the USS.

4. 	The USS performs C2 authorization based on the received information and includes in the Naf_Auth_Response sent to the UAS NF/NEF Service Level Device Identity (e.g. the CAA-Level UAV-ID) (potentially new) and included in the authorization message the C2 authorization result, i.e. whether the UAV is allowed to be paired with the UAV-C, and security information.

5.	The UAS NF/NEF forwards the information received from the USS the Nnef_Auth_Response sent to the SMF+PGW-C.

6.	To inform the UE about the C2 authorization result the SMF+PGW-C includes the authorization result and, optionally, a new Service Level Device Identity (e.g. CAA-Level UAV ID) if received from the USS, in the PCO in the PDN Connectivity Accept sent to the UE and let the PDN Connectivity Request procedure continue until finalized.

If a failed C2 authorization result is received from the USS, the SMF+PGW-C instead rejects the PDN Connectivity Request and includes a cause code indicating not authorized.

7.	If the C2 authorisation is successful the USS subscribes via the UAS NF/NEF to a PDN Connection Status Event report for the PDN Connection used for C2 including in the request the GPSI of the UAV. The UAS NF/NEF determines the APN/DNN and uses this APN/DNN to subscribe to SMF+PGW-C for PDN Connection Status Event. The SMF+PGW-C detects, as described in step 6-7 of figure 4.15.3.2.3-1 in TS 23.502 [3], when the PDN Connection is established and sends the PDN Connection Status Event report to the UAS NF/NEF by means of Nsmf_EventExposure_Notify message, including GPSI and UE IP Address. The UAS NF/NEF then forwards the event message to the USS.

8.	The USS stores the received UE IP address and invokes, with the received PDN Connection IP address and the IP-address of the authorized paired UAV-C as input, the USS initiated C2 pairing policy configuration in EPS procedure (see figure 5.2.5.4.2-1) to request corresponding traffic to be allowed on the PDN Connection by the PGW-U.

Unless a dedicated QoS is requested for the C2 flows, this procedure does not invoke any interaction with the UE, MME or RAN.

##### 5.2.5.3.2	UE requested bearer resource modification of an existing PDN connection for C2 authorization

C2 authorization is requested at UE requested bearer resource modification (see clause 5.4.5 of TS 23.401 [6]):

-	After UUAA-SM is performed and a common PDN Connection is used for connectivity to USS and C2 communication to a UAV-C (as configured in the UAV); or

-	If the UE has already established a PDN Connection for C2 communication to a UAV-C.

Figure 5.2.5.3.2-1: UE requested bearer resource modification of an existing PDN connection for C2 authorization

1.	The UE establishes a PDN Connection for USS communication as described in clause 5.2.3.

2-3.	When the UAV needs to establish C2 communication, the UAV determines that an existing PDN Connection can be used and initiates a UE requested bearer resource modification procedure. In the PCO in the request, the UE includes a Service Level Device Identity (e.g. CAA-Level UAV ID) and shall include a C2 Aviation Payload that includes C2 authorization information. The USS may also use its locally configured pairing information for UAV - UAV-C pairing authorization which takes precedence over UAV provided pairing information. The pairing information includes the Service Level Device Identity (e.g. CAA-level UAV ID) of the requesting UE and also includes identification information of UAV-C to pair if available. The UAV may also include other information such as Flight Authorization information.

NOTE:	How the pairing information is configured in the UAV is outside the scope of 3GPP specifications.

4-5.	For a UAV with aerial subscription the SMF+PGW-C determines based on the APN/DNN of the PDN Connection and presence of Service Level Device Identity (e.g. CAA-Level UAV ID) that authorization is required. The SMF+PGW-C invokes the authorization procedure with the USS/UTM (via UAS NF/NEF) by invoking a Nnef_Auth_Reauth request including the UAS information provided by the UAV in step 2 (including the C2 Authorization Payload), the Service Level Device Identity (e.g. CAA-Level UAV ID), GPSI, PDN Connection IP address, and optionally the UAV location (e.g. Cell ID) provided by the MME.

6.	The UE requested bearer resource modification procedure completes as in clause 5.4.5-1 of TS 23.401 [6].

7.	Triggered by step 5, the USS performs C2 authorization based on the received information and invokes, in order to forward the C2 authorization result to the UAV/UE, the UAV Re-authorization procedure (see figure 5.2.4.4-1) including GPSI, Service Level Device Identity (e.g. CAA-Level UAV-ID) (potentially new) and included in the authorization message, the C2 authorization result and security information.

8.	The USS invokes, with the received PDN Connection IP address and the IP address of the authorized paired UAV-C as input, the USS initiated pairing policy configuration procedure (see figure 5.2.5.4.2-1) to request corresponding traffic to be allowed on the PDN Connection in the UPF/PGW-U.

Unless a dedicated QoS is requested for the C2 flows, this procedure does not invoke any interaction with the UE, MME or RAN.

----------------------------------------End of Changes--------------------------------------------