3GPP TSG-SA WG2 Meeting #147E (e-meeting)				S2-2107218

October 18 – 22, 2021, Elbonia

Title:	[Draft] Reply LS on Tx Profile

Response to:	LS from RAN2 on Tx Profile (S2-2107043/R2-2108995)

Release:	Rel-17

Work Item:	NR_SL_enh-Core, eV2XARC_Ph2

Source:	SA2

To:	RAN2

Cc:	CT1

Contact Person:

Name:	LaeYoung Kim

E-mail Address:	laeyoung.kim@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.287 CR (S2-2107217)

1	Overall description

SA2 thanks RAN2 for their LS on Tx Profile in S2-2107043/R2-2108995.

SA2 discussed the Tx Profile for NR PC5 DRX operation and had no concern on the RAN2 agreements listed in the LS from RAN2.

SA2 agreed the following approach/principles regarding NR Tx Profile for broadcast and groupcast:

1)	NR Tx Profile is used to indicate the UE whether PC5 DRX is applied over NR PC5 RAT.

-	A definition for "NR Tx Profile" can be added to TS 23.287.

-	The contents of NR Tx Profile are considered transparent to V2X layer.

2)	V2X service type can be mapped to NR Tx Profile.

-	The mapping of V2X service types to NR Tx Profiles needs to be defined as one of parameters provisioned/configured in the UE for broadcast and groupcast modes of V2X communication over NR PC5.

3)	For broadcast and groupcast, the V2X layer determines the NR Tx Profile for the respective V2X service type based on the configuration described in bullet 2) above and provides the NR Tx Profile to the AS layer when providing the Destination Layer-2 ID and the PC5 QoS parameters to the AS layer.

-	This applies to Transmitting UE as well as to Receiving UEs.

-	To avoid an undesirable case that different V2X services are mapped to different NR Tx Profiles (e.g. one indicates that PC5 DRX is applied while the other indicates that PC5 DRX is not applied) but these V2X services use same Destination Layer-2 ID resulting in inconsistent NR Tx Profiles for same Destination Layer-2 ID, one assumption can be described that same Destination Layer-2 ID is not used for different NR Tx Profiles.

SA2 agreed the attached TS 23.287 CR that captures the above approach/principles.

2. Actions:

To RAN2

ACTION:  	SA2 kindly asks RAN2 to take the above information into account for further work.

3. Dates of Next SA2 Meetings:

SA2#148E	November 15 – 22, 2021	Electronic meeting

SA2#149E	February 14 – 25, 2022	Electronic meeting