3GPP TSG-SA WG2 Meeting #147E (e-meeting)	S2-2107361

18 – 22 Oct. 2021, Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3290 | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on RTD and Terminology Alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent, Tencent Cloud |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2021-10-08 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NR satellite related abbreviations e.g. LEO,MEO and GEO are missing. In 5QI table, RTD (Roud Trip Delay) is mentioned but is not well-defined and not explained in 23.501 which is quite close to RTT but have different meaning. To avoid potential missunderstanding, change RTD into more concrete description e.g. two way propagation delay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change RTD into two way propagation delay. Add LEO, MEO and GEO into abbreviations part. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear term may cause confusion. NR satellite access related abbr. is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5G DDNMF	5G Direct Discovery Name Management Function

5G LAN	5G Local Area Network

5GS	5G System

5G-AN	5G Access Network

5G-AN PDB	5G Access Network Packet Delay Budget

5G-EIR	5G-Equipment Identity Register

5G-GUTI	5G Globally Unique Temporary Identifier

5G-BRG	5G Broadband Residential Gateway

5G-CRG	5G Cable Residential Gateway

5G GM	5G Grand Master

5G-RG	5G Residential Gateway

5G-S-TMSI	5G S-Temporary Mobile Subscription Identifier

5G VN	5G Virtual Network

5QI	5G QoS Identifier

ADRF	Analytics Data Repository Function

AF	Application Function

AKMA	Authentication and Key Management for Applications

AnLF	Analytics Logical Function

AMF	Access and Mobility Management Function

AS	Access Stratum

ATSSS	Access Traffic Steering, Switching, Splitting

ATSSS-LL	ATSSS Low-Layer

AUSF	Authentication Server Function

BMCA	Best Master Clock Algorithm

BSF	Binding Support Function

CAG	Closed Access Group

CAPIF	Common API Framework for 3GPP northbound APIs

CH	Credentials Holder

CHF	Charging Function

CN PDB	Core Network Packet Delay Budget

CP	Control Plane

DAPS	Dual Active Protocol Stacks

DCCF	Data Collection Coordination Function

DCS	Default Credentials Server

DL	Downlink

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

DRX	Discontinuous Reception

DS-TT	Device-side TSN translator

ePDG	evolved Packet Data Gateway

EBI	EPS Bearer Identity

EUI	Extended Unique Identifier

FAR	Forwarding Action Rule

FN-BRG	Fixed Network Broadband RG

FN-CRG	Fixed Network Cable RG

FN-RG	Fixed Network RG

FQDN	Fully Qualified Domain Name

GEO	Geostationary Orbit

GFBR	Guaranteed Flow Bit Rate

GIN	Group ID for Network Selection

GMLC	Gateway Mobile Location Centre

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HMTC	High-Performance Machine-Type Communications

HR	Home Routed (roaming)

IAB	Integrated access and backhaul

IMEI/TAC	IMEI Type Allocation Code

IPUPS	Inter PLMN UP Security

I-SMF	Intermediate SMF

I-UPF	Intermediate UPF

LADN	Local Area Data Network

LBO	Local Break Out (roaming)

LEO	Low Earth Orbit

LMF	Location Management Function

LoA	Level of Automation

LPP	LTE Positioning Protocol

LRF	Location Retrieval Function

MBS	Multicast/Broadcast Service

MBSF	Multicast/Broadcast Service Function

MBSTF	Multicast/Broadcast Service Transport Function

MB-SMF	Multicast/Broadcast Session Management Function

MB-UPF	Multicast/Broadcast User Plane Function

MEO	Middle Earth Orbit

MFAF	Messaging Framework Adaptor Function

MCX	Mission Critical Service

MDBV	Maximum Data Burst Volume

MFBR	Maximum Flow Bit Rate

MICO	Mobile Initiated Connection Only

ML	Machine Learning

MPS	Multimedia Priority Service

MPTCP	Multi-Path TCP Protocol

MTLF	Model Training Logical Function

N3IWF	Non-3GPP InterWorking Function

N5CW	Non-5G-Capable over WLAN

NAI	Network Access Identifier

NEF	Network Exposure Function

NF	Network Function

NGAP	Next Generation Application Protocol

NID	Network identifier

NPN	Non-Public Network

NR	New Radio

NRF	Network Repository Function

NSAC	Network Slice Admission Control

NSACF	Network Slice Admission Control Function

NSI ID	Network Slice Instance Identifier

NSSAA	Network Slice-Specific Authentication and Authorization

NSSAAF	Network Slice-specific and SNPN Authentication and Authorization Function

NSSAI	Network Slice Selection Assistance Information

NSSF	Network Slice Selection Function

NSSP	Network Slice Selection Policy

NSSRG	Network Slice Simultaneous Registration Group

NW-TT	Network-side TSN translator

NWDAF	Network Data Analytics Function

ONN	Onboarding Network

ON-SNPN	Onboarding Standalone Non-Public Network

PCF	Policy Control Function

PDB	Packet Delay Budget

PDR	Packet Detection Rule

PDU	Protocol Data Unit

PEI	Permanent Equipment Identifier

PER	Packet Error Rate

PFD	Packet Flow Description

PNI-NPN	Public Network Integrated Non-Public Network

PPD	Paging Policy Differentiation

PPF	Paging Proceed Flag

PPI	Paging Policy Indicator

PSA	PDU Session Anchor

PTP	Precision Time Protocol

PVS	Provisioning Server

QFI	QoS Flow Identifier

QoE	Quality of Experience

RACS	Radio Capabilities Signalling optimisation

(R)AN	(Radio) Access Network

RG	Residential Gateway

RIM	Remote Interference Management

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

RSN	Redundancy Sequence Number

SA NR	Standalone New Radio

SBA	Service Based Architecture

SBI	Service Based Interface

SCP	Service Communication Proxy

SD	Slice Differentiator

SEAF	Security Anchor Functionality

SEPP	Security Edge Protection Proxy

SMF	Session Management Function

SMSF	Short Message Service Function

SN	Sequence Number

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SO-SNPN	Subscription Owner Standalone Non-Public Network

SSC	Session and Service Continuity

SSCMSP	Session and Service Continuity Mode Selection Policy

SST	Slice/Service Type

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

SV	Software Version

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNL	Transport Network Layer

TNLA	Transport Network Layer Association

TSC	Time Sensitive Communication

TSCAI	TSC Assistance Information

TSCTSF	Time Sensitive Communication and Time Synchronization function

TSN	Time Sensitive Networking

TSN GM	TSN Grand Master

TSP	Traffic Steering Policy

TT	TSN Translator

TWIF	Trusted WLAN Interworking Function

UAS NF	Uncrewed Aerial System Network Function

UCMF	UE radio Capability Management Function

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UL	Uplink

UL CL	Uplink Classifier

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

URRP-AMF	UE Reachability Request Parameter for AMF

URSP	UE Route Selection Policy

VID	VLAN Identifier

VLAN	Virtual Local Area Network

W-5GAN	Wireline 5G Access Network

W-5GBAN	Wireline BBF Access Network

W-5GCAN	Wireline 5G Cable Access Network

W-AGF	Wireline Access Gateway Function

# * * * * Second change * * * *

5.7.4	Standardized 5QI to QoS characteristics mapping

Standardized 5QI values are specified for services that are assumed to be frequently used and thus benefit from optimized signalling by using standardized QoS characteristics. Dynamically assigned 5QI values (which require a signalling of QoS characteristics as part of the QoS profile) can be used for services for which standardized 5QI values are not defined. The one-to-one mapping of standardized 5QI values to 5G QoS characteristics is specified in table 5.7.4-1.

Table 5.7.4-1: Standardized 5QI to QoS characteristics mapping

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget (NOTE 3) | Packet Error Rate | Default Maximum Data Burst Volume (NOTE 2) | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | GBR | 20 | 100 ms (NOTE 11, NOTE 13) | 10-2 | N/A | 2000 ms | Conversational Voice |
| 2 | (NOTE 1) | 40 | 150 ms (NOTE 11, NOTE 13) | 10-3 | N/A | 2000 ms | Conversational Video (Live Streaming) |
| 3 |  | 30 | 50 ms (NOTE 11, NOTE 13) | 10-3 | N/A | 2000 ms | Real Time Gaming, V2X messages (see TS 23.287 [121]). Electricity distribution – medium voltage, Process automation monitoring |
| 4 |  | 50 | 300 ms (NOTE 11, NOTE 13) | 10-6 | N/A | 2000 ms | Non-Conversational Video (Buffered Streaming) |
| 65 (NOTE 9, NOTE 12) |  | 7 | 75 ms (NOTE 7, NOTE 8) | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g. MCPTT) |
| 66 (NOTE 12) |  | 20 | 100 ms (NOTE 10, NOTE 13) | 10-2 | N/A | 2000 ms | Non-Mission-Critical user plane Push To Talk voice |
| 67 (NOTE 12) |  | 15 | 100 ms (NOTE 10, NOTE 13) | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| 75 (NOTE 14) |  |  |  |  |  |  |  |
| 71 |  | 56 | 150 ms (NOTE 11, NOTE 13, NOTE 15) | 10-6 | N/A | 2000 ms | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 72 |  | 56 | 300 ms (NOTE 11, NOTE 13, NOTE 15) | 10-4 | N/A | 2000 ms | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 73 |  | 56 | 300 ms (NOTE 11, NOTE 13, NOTE 15) | 10-8 | N/A | 2000 ms | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 74 |  | 56 | 500 ms (NOTE 11, NOTE 15) | 10-8 | N/A | 2000 ms | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 76 |  | 56 | 500 ms (NOTE 11, NOTE 13, NOTE 15) | 10-4 | N/A | 2000 ms | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 5 | Non-GBR | 10 | 100 ms NOTE 10, NOTE 13) | 10-6 | N/A | N/A | IMS Signalling |
| 6 | (NOTE 1) | 60 | 300 ms (NOTE 10, NOTE 13) | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) |
| 7 |  | 70 | 100 ms (NOTE 10, NOTE 13) | 10-3 | N/A | N/A | Voice, Video (Live Streaming) Interactive Gaming |
| 8 |  | 80 | 300 ms (NOTE 13) | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive |
| 9 |  | 90 |  |  |  |  | video, etc.) |
| 10 |  | 90 | 1100ms (NOTE 13) (NOTE 17) | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) and any service that can be used over satellite access type with these characteristics |
| 69 (NOTE 9, NOTE 12) |  | 5 | 60 ms (NOTE 7, NOTE 8) | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g. MC-PTT signalling) |
| 70 (NOTE 12) |  | 55 | 200 ms (NOTE 7, NOTE 10) | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9) |
| 79 |  | 65 | 50 ms (NOTE 10, NOTE 13) | 10-2 | N/A | N/A | V2X messages (see TS 23.287 [121]) |
| 80 |  | 68 | 10 ms (NOTE 5, NOTE 10) | 10-6 | N/A | N/A | Low Latency eMBB applications Augmented Reality |
| 82 | Delay-critical GBR | 19 | 10 ms (NOTE 4) | 10-4 | 255 bytes | 2000 ms | Discrete Automation (see TS 22.261 [2]) |
| 83 |  | 22 | 10 ms (NOTE 4) | 10-4 | 1354 bytes (NOTE 3) | 2000 ms | Discrete Automation (see TS 22.261 [2]); V2X messages (UE - RSU Platooning, Advanced Driving: Cooperative Lane Change with low LoA. See TS 22.186 [111], TS 23.287 [121]) |
| 84 |  | 24 | 30 ms (NOTE 6) | 10-5 | 1354 bytes (NOTE 3) | 2000 ms | Intelligent transport systems (see TS 22.261 [2]) |
| 85 |  | 21 | 5 ms (NOTE 5) | 10-5 | 255 bytes | 2000 ms | Electricity Distribution- high voltage (see TS 22.261 [2]). V2X messages (Remote Driving. See TS 22.186 [111], NOTE 16, see TS 23.287 [121]) |
| 86 |  | 18 | 5 ms (NOTE 5) | 10-4 | 1354 bytes | 2000 ms | V2X messages (Advanced Driving: Collision Avoidance, Platooning with high LoA. See TS 22.186 [111], TS 23.287 [121]) |
| 87 |  | 25 | 5 ms (NOTE 4) | 10-3 | 500 bytes | 2000 ms | Interactive Service - Motion tracking data, (see TS 22.261 [2]) |
| 88 |  | 25 | 10 ms (NOTE 4) | 10-3 | 1125 bytes | 2000 ms | Interactive Service - Motion tracking data, (see TS 22.261 [2]) |
| 89 |  | 25 | 15 ms (NOTE 4) | 10-4 | 17000 bytes | 2000 ms | Visual content for cloud/edge/split rendering (see TS 22.261 [2]) |
| 90 |  | 25 | 20 ms (NOTE 4) | 10-4 | 63000 bytes | 2000 ms | Visual content for cloud/edge/split rendering (see TS 22.261 [2]) |
| NOTE 1: A packet which is delayed more than PDB is not counted as lost, thus not included in the PER. NOTE 2: It is required that default MDBV is supported by a PLMN supporting the related 5QIs. NOTE 3: The Maximum Transfer Unit (MTU) size considerations in clause 9.3 and Annex C of TS 23.060 [56] are also applicable. IP fragmentation may have impacts to CN PDB, and details are provided in clause 5.6.10. NOTE 4: A static value for the CN PDB of 1 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. NOTE 5: A static value for the CN PDB of 2 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. NOTE 6: A static value for the CN PDB of 5 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. NOTE 7: For Mission Critical services, it may be assumed that the UPF terminating N6 is located "close" to the 5G_AN (roughly 10 ms) and is not normally used in a long distance, home routed roaming situation. Hence a static value for the CN PDB of 10 ms for the delay between a UPF terminating N6 and a 5G_AN should be subtracted from this PDB to derive the packet delay budget that applies to the radio interface. NOTE 8: In both RRC Idle and RRC Connected mode, the PDB requirement for these 5QIs can be relaxed (but not to a value greater than 320 ms) for the first packet(s) in a downlink data or signalling burst in order to permit reasonable battery saving (DRX) techniques. NOTE 9: It is expected that 5QI-65 and 5QI-69 are used together to provide Mission Critical Push to Talk service (e.g. 5QI-5 is not used for signalling). It is expected that the amount of traffic per UE will be similar or less compared to the IMS signalling. NOTE 10: In both RRC Idle and RRC Connected mode, the PDB requirement for these 5QIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 11: In RRC Idle mode, the PDB requirement for these 5QIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 12: This 5QI value can only be assigned upon request from the network side. The UE and any application running on the UE is not allowed to request this 5QI value. NOTE 13: A static value for the CN PDB of 20 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. NOTE 14: This 5QI is not supported in this Release of the specification as it is only used for transmission of V2X messages over MBMS bearers as defined in TS 23.285 [72] but the value is reserved for future use. NOTE 15: For "live" uplink streaming (see TS 26.238 [76]), guidelines for PDB values of the different 5QIs correspond to the latency configurations defined in TR 26.939 [77]. In order to support higher latency reliable streaming services (above 500ms PDB), if different PDB and PER combinations are needed these configurations will have to use non-standardised 5QIs. NOTE 16: These services are expected to need much larger MDBV values to be signalled to the RAN. Support for such larger MDBV values with low latency and high reliability is likely to require a suitable RAN configuration, for which, the simulation scenarios in TR 38.824 [112] may contain some guidance. NOTE 17: The worst case one way propagation delay for GEO satellite is expected to be ~270ms, ,~ 21 ms for LEO at 1200km, and 13 ms for LEO at 600km. The UL scheduling delay that needs to be added is also typically two way propagation delay e.g. ~540ms for GEO, ~42ms for LEO at 1200km, and ~26 ms for LEO at 600km. Based on that, the 5G-AN Packet delay budget is not applicable for 5QIs that require 5G-AN PDB lower than the sum of these values when the specific types of satellite access are used (see TS 38.300 [27]). 5QI-10 can accommodate the worst case PDB for GEO satellite type. |  |  |  |  |  |  |  |

NOTE:	It is preferred that a value less than 64 is allocated for any new standardised 5QI of Non-GBR resource type. This is to allow for option 1 to be used as described in clause 5.7.1.3 (as the QFI is limited to less than 64).

# * * * * End of changes * * * *