3GPP TSG-SA2 Meeting #147E (e-meeting)	S2-2107391

# Elbonia, October 18 - 22, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0019 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Relay Discovery Additional Information message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, ZTE, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the last SA2#146E meeting, it was agreed to support the Cell ID announcement procedure, in which the 5G ProSe Layer-3 UE-to-Network Relay announces the NCGI of the serving cell by sending Relay Discovery Additional Information message. This NCGI parameter along with other parameters included in Relay Discovery Additional Information message need to be clarified in cluase 5.8.3.2, and the descriptions about Relay Discovery Additional Information message in clauses 6.3.2.3.1, 6.3.2.3.2 and 6.5.1.3 need to be aligned. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add NCGI and other parameters for Relay Discovery Additional Information message in clause 5.8.3.2. Align the descriptions about Relay Discovery Additional Information message in clause 6.3.2.3.1, 6.3.2.3.2 and 6.5.1.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned descriptions between different clauses. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.8.3.2, 6.3.2.3.1, 6.3.2.3.2, 6.5.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** FIRST CHANGE ***************

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5G DDNMF	5G Direct Discovery Name Management Function

AS layer	Access Stratum layer

NCGI	NR Cell Global ID

PDUID	ProSe Discovery UE ID

PFI	PC5 QoS Flow Identifier

PQI	PC5 5QI

ProSe	Proximity based Services

RPAUID	Restricted ProSe Application User ID

RSC	Relay Service Code

RSD	Rule Selection Descriptor

****************** SECOND CHANGE ***************

5.8.3.2	Identifiers for 5G ProSe Layer-3 UE-to-Network Relay

The following parameters may be used in addition to the parameters specified in 5.8.3.1, in 5G ProSe Layer-3 UE-to-Network Relay supporting N3IWF discovery procedure (for both Model A and Model B) to assist with N3IWF selection:

-	Relay TAI: indicates the Tracking Area Identity corresponding to the serving cell of the 5G ProSe Layer-3 UE-to-Network Relay.

For 5G ProSe Layer-3 UE-to-Network relay, a Relay Service Code in the Announcement Message is associated with a set of PDU session parameters (e.g. PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference). The Relay Service Code may also represent if the relay UE can provide secure N3IWF connection.

For 5G ProSe Layer-3 Remote UE discovering 5G ProSe Layer-3 UE-to-Network relay, the Relay Service Code in the Solicitation Message represents the PDU session parameters that a PDU session of the relay should be able to support. The Relay Service Code may also represent if the remote UE requires secure N3IWF connection.

The following parameters may be used in the Relay Discovery Additional Information message (for Model A) for 5G ProSe Layer-3 UE-to-Network Relay:

-	Relay Service Code: the Relay Service Code associated with the message. The Relay Service Code is used to identify the security parameters needed by the receiving UE to process the discovery message.

-	Announcer Info: provides information about the announcing user.

-	NCGI: indicates the NCGI of the serving cell of the 5G ProSe Layer-3 UE-to-Network Relay.

****************** THIRD CHANGE ***************

6.3.2.3.1	General

5G ProSe UE-to-Network Relay Discovery is applicable to both 5G ProSe Layer-3 and Layer-2 UE-to-Network relay discovery for public safety use and commercial services. To perform 5G ProSe UE-to-Network Relay Discovery, the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay are pre-configured or provisioned with the related information as described in clause 5.1.

In 5G ProSe UE-to-Network Relay Discovery, the UEs use pre-configured or provisioned information for the relay discovery procedures as defined in clause 5.1.4.1.

The Relay Service Code (RSC) is used in the 5G ProSe UE-to-Network Relay discovery, to indicate the connectivity service the 5G ProSe UE-to-Network Relay provides to the 5G ProSe Remote UE. The RSCs are configured on the 5G ProSe UE-to-Network Relay and the 5G ProSe Remote UE as defined in clause 5.1.4. The 5G ProSe UE-to-Network Relay and the 5G ProSe Remote UE are aware of whether a RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service based the policy as specified in clause 5.1.4. A 5G ProSe UE-to-Network Relay supporting multiple RSCs can advertise the RSCs using multiple discovery messages, with one RSC per discovery message.

Additional information (e.g. NCGI of the serving cell) not directly used for discovery can also be advertised using the PC5-D protocol stack in single or separate discovery messages of type "Relay Discovery Additional Information".

Both Model A and Model B discovery are supported:

-	Model A uses a single discovery protocol message (Announcement).

-	Model B uses two discovery protocol messages (Solicitation and Response).

For Relay Discovery Additional Information, only Model A discovery is used.

****************** FOURTH CHANGE ***************

6.3.2.3.2	Procedure for 5G ProSe UE-to-Network Relay Discovery with Model A

Depicted in Figure 6.3.2.3.2-1 is the procedure for 5G ProSe UE-to-Network Discovery with Model A.

Figure 6.3.2.3.2-1: 5G ProSe UE-to-Network Relay Discovery with Model A

1.	The 5G ProSe UE-to-Network Relay sends a UE-to-Network Relay Discovery Announcement message. The UE-to-Network Relay Discovery Announcement message contains the Type of Discovery Message, Announcer Info and RSC, and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.3.

For 5G ProSe Layer-3 UE-to-Network Relay, the 5G ProSe Layer-3 UE-to-Network Relay shall only include a RSC in the UE-to-Network Relay Discovery Announcement when the S-NSSAI associated with that RSC belongs to the Allowed NSSAI of the UE-to-Network Relay.

The 5G ProSe Remote UE (1 to 3) determines the Destination Layer-2 ID for signalling reception. The Destination Layer-2 ID is configured with the UE(s) as specified in clause 5.1.4.1.

5G ProSe Remote UE (1 to 3) monitors announcement messages with the 5G ProSe UE-to-Network RSC corresponding to the desired services.

Optionally, the 5G ProSe UE-to-Network Relay may also send Relay Discovery Additional Information messages. The parameters contained in this message are described in clause 5.8.3.2.

The 5G ProSe Remote UE selects the 5G ProSe UE-to-Network Relay based on the information received in step 1.

NOTE:	Access Stratum layer information used for 5G ProSe UE-to-Network Relay selection is specified in RAN specifications.

****************** FIFTH CHANGE ***************

6.5.1.3	Cell ID announcement procedure

Cell ID announcement procedure outlined in figure 6.5.1.3-1 allows an authorized 5G ProSe Layer-3 remote UE, based on its application specific requirements outside the scope of the present document, to request a 5G ProSe Layer-3 UE-to-Network Relay to announce the NR Cell Global ID (NCGI) of the Cell serving the 5G ProSe Layer-3 UE-to-Network Relay.

Figure 6.5.1.3-1: Cell ID announcement request procedure

1)	5G ProSe Layer-3 Remote UE has discovered a 5G ProSe Layer-3 UE-to-Network Relay and an application requires serving cell NCGI reporting even when it is accessing via 5G ProSe Layer-3 UE-to-Network Relay (i.e. the application benefits from this value even if it is the value of the cell serving the 5G Layer-3 ProSe UE-to-Network Relay).

2)	The 5G ProSe Layer-3 Remote UE sends to the 5G ProSe Layer-3 UE-to-Network Relay a Cell ID Announcement Request to obtain the NCGI of the 5G ProSe Layer-3 UE-to-Network Relay's serving cell.

3)	The 5G ProSe Layer-3 UE-to-Network Relay acknowledges receipt of the request in step 2 with a Cell ID Announcement Response (NCGI_Announcement_Request_Refresh Timer). The NCGI_Announcement_Request_Refresh Timer, (configurable in the 5G ProSe Layer-3 UE-to-Network Relay) is provided to the 5G ProSe Layer-3 Remote UE so that when this timer expires the 5G ProSe Layer-3 Remote UE repeats the Cell ID Announcement Request procedure if it still needs obtain the NCGI. If the 5G ProSe Layer-3 Remote UE does not initiate new Cell ID Announcement Request procedure when this NCGI_Announcement_Request_Refresh Timer expires and no other UE request NCGI announcement before the NCGI_Announcement_Request_Refresh timer expires in the 5G ProSe Layer-3 UE-to-Network relay, then the relay shall stop announcing the NCGI of the serving cell.

4)	The 5G ProSe Layer-3 UE-to-Network Relay announces the NCGI of the serving cell by sending Relay Discovery Additional Information Message as defined in clause 5.8.3.2. This is repeated periodically with a configurable frequency (normally Higher than the one related to the NCGI_Announcement_Request_Refresh Timer) until there is no UE requesting to announce the NCGI as determined by the NCGI_Announcement_Request_Refresh Timer running in the 5G ProSe Layer-3 UE-to-Network Relay.

5)	The 5G ProSe Layer-3 UE-to-Network Relay detects new NCGI of a new serving cell it is happening to be camping on.

6)	Detection of new serving cell NCGI in Step 5 triggers the 5G ProSe Layer-3 UE-to-Network Relay to announce the NCGI of the serving cell by sending a Relay Discovery Additional Information Message (NCGI) immediately and to repeat it periodically with a configurable frequency as in step 4 until there are no UEs requesting to announce the NCGI, i.e until the NCGI_Announcement_Request_Refresh Timer expires in the 5G ProSe UE-to-Network relay.

****************** END OF CHANGES ***************