SA WG2 Meeting #147E	S2-2107401

Oct 18 - 22, 2021, Electronic

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0459 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on analytics aggregation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - As indicated in clause 6.1A.3, analytic aggregation covers scenarios where NWDAF service consumer requests or subscribes to analytics information with or without provisioning Area of Interest. However in clause 6.1A.2, some descriptions limit application of analytics aggregation only to the scenarios of provisioning AoI. - The descriptions on "information on the discovered NWDAFs" maintained by aggregator NWDAF, and aggregator NWDAF discovery/selection by NWDAF service consumer are obscure in clause 6.1A.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that analytics aggregation applies to analytics with and without AoI. - Other corrections, including clarifications on "information on the discovered NWDAFs", aggregator NWDAF discovery/selection by NWDAF service consumer, and editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned specifications on application scenarios of analytic aggregation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1A.2 |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change ********************

6.1A.2	Analytics Aggregation

The analytics aggregation from multiple NWDAFs is used to address cases where an NWDAF service consumer requests Analytics ID(s) that requires multiple NWDAFs to collectively serve the request. Analytic aggregation applies to scenarios where NWDAF service consumer requests or subscribes to analytics information with or without provisioning Area of Interest.

Aggregator NWDAF or aggregation point:

-	Is an NWDAF instance with additional capabilities to aggregate output analytics provided by other NWDAFs. This is in addition to regular NWDAF behaviour such as collecting data from other data sources to be able to generate its own output analytics.

-	Is able to divide the area of interest if received from the consumer into sub area of interest based on the serving area of each NWDAF to be requested for analytics, and then send analytics requests including the sub area of interest as an Analytics Filter to corresponding NWDAFs. The Aggregator NWDAF may maintain information on the discovered NWDAFs, including their supported Analytics IDs, NWDAF Serving Areas, etc.

-	Has "analytics aggregation capability" registered in its NF Profile within the NRF.

-	Supports the requesting and exchange of "Analytics Metadata Information" between NWDAFs when required for the aggregation of output analytics.

"Analytics Metadata Information" is additional information associated with the requested Analytics ID(s) as defined in clause 6.1.3.

-	Supports dataset statistical properties, output strategy, and data time window parameters per type of analytics (i.e. analytics ID) as defined in clause 6.1.3.

NRF:

-	Stores the NF Profile of the NWDAF instances, including "analytics aggregation capability" for Aggregator NWDAFs and "analytics metadata provisioning capability" when supported by the NWDAF.

-	Returns the NWDAF(s) matching the attributes provided in the Nnrf_NFDiscovery_Request, as specified in clause 5.2.7.3 of TS 23.502 [3].

NWDAF service consumer:

-	Requests or subscribes to receive analytics for one or more Analytic IDs, as specified in clause 6.1 of the present document.

-	Uses the discovery mechanism from NRF as defined in clause 6.3.13 of TS 23.501 [2] to identify NWDAFs with analytics aggregation capability and other capabilities (e.g. providing data/analytics for specific TAI(s)).

-	Can differentiate and select the preferred NWDAF in case multiple NWDAFs are returned by the NRF based on its internal selection criteria.

******************** End of Change ********************