SA WG2 Meeting #147E	S2-2107403

Oct 18 - 22, 2021, Electronic

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0013 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Subscription to multicast services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - As specified in clause 4.15.6.2 of TS 23.502, "NOTE 2: For non-roaming case and no authorisation or validation by the UDM required and if the request is not associated with a 5G VN group, the NEF can directly forward the external parameter to the UDR via Nudr_DM_Update Request message. And in this case, the UDR responds to NEF via Nudr_DM_Update Response message. 3. UDM may read from UDR, by means of Nudr_DM_Query, corresponding subscription information in order to validate required data updates and authorize these changes for this subscriber or Group for the corresponding AF." The UDR can be updated by the NEF and stores the MBS data directly. Also, it should be possible for the MBSF to update the MBS data in the UDR, when adopting Configuration 2 or 3 in Annex A. - The indication received from the MB-SMF on whether the multicast session is open for any user is optional and complementary to UE MBS subscription data, and therefore should be considered after the SMF checks the UE MBS subscription data. - In Table 6.4.3-1 and 6.4.3-2 the header of "MBS data" should be Data Set instead of Subscription Data Types. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the UDR can also be updated by the NEF/MBSF on the MBS data. - Correct the description on the SMF authorizes a UE to join an MBS session in the NOTE. - Correct in Table 6.4.3-1 and 6.4.3-2 the header of "MBS data" as Data Set. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete and incorrect specification on multicast service authorization. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1, 6.4.2, 6.4.3 |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change ********************

6.4.1	General

The UDM stores the subscription information to give the user permission to use multicast services, which include the MBS subscription data for a UE in UE subscription data.

At any time, the operator may change the MBS subscription data for a UE in the UDM.

The MBS subscription data in UE subscription data contains the following information:

-	Indication of whether the UE is authorized to use multicast MBS service (i.e. receive any multicast MBS session data at all).

-	MBS Session ID(s) of the multicast session(s) that the UE is allowed to join.

NOTE:	The MBS session IDs only apply for MBS sessions which are not open to any UE. The SMF decides whether an MBS session is open to any UE based on the indication if received from the MB-SMF.

The MBS subscription data is provided by the UDM to the SMF during or after the establishment procedure of PDU Session associated with multicast session(s) using Nudm_SDM service for subscription data type "MBS subscription data" as defined in clause 7.2.1.2.

During multicast session join procedure, the SMF retrieves MBS Session information from the MB-SMF, and authorizes the MBS Session join request for the UE based on MBS subscription data of the UE and the MBS Session information as described in clause 7.2.1.

The UDR is updated by the UDM or the NEF/MBSF and stores the MBS data (as specified in clause 4.15.6.2 of TS 23.502 [6]), i.e. authorization information by the AF and/or the 5GC per MBS Session, which contains the following information:

-	MBS Session ID(s);

-	For each MBS Session ID:

-	Internal Group Identifier, SUPI list or Any UE indication which indicates which UEs are entitled to join the MBS Session identified by the MBS Session ID;

6.4.2	MBS subscription data in UDM

The information stored in the UDM as defined in clause 5.2.3.3.1 of TS 23.502 [6] is extended as follows:

-	MBS subscription data for a UE as part of UE subscription data, as defined in Table 6.4.2-1, with keys defined in Table 6.4.2-2.

Table 6.4.2-1: MBS subscription data type in UE subscription data

| Subscription data type | Field | Description |
| --- | --- | --- |
| MBS subscription data | MBS allowed | Indicates whether the UE is authorized to use multicast MBS service. |
|  | MBS Session ID(s) | Identifies the MBS Session(s) that the UE is allowed to join. |

Table 6.4.2-2: MBS subscription data type keys

| Subscription Data Types | Data Key | Data Sub Key |
| --- | --- | --- |
| MBS Subscription data | SUPI | MBS Session ID |

6.4.3	MBS information in UDR

The information stored in the UDR as defined in clause 5.2.12.2.1 of TS 23.502 [6] is extended as follows:

1.	MBS data as defined in Tables 6.4.3-1 with keys defined in Table 6.4.3-2.

Table 6.4.3-1: MBS data type

| Data Set | Field | Description |
| --- | --- | --- |
| MBS data | MBS Session ID | MBS Session ID as defined in Clause 6.5.1 |
|  | Internal Group Identifier (NOTE 1) | Identifies internal group of UEs that are entitled to join a multicast session |
|  | SUPI list (NOTE 1) | Identifies UEs that are entitled to join a multicast session |
|  | Any UE indication (NOTE 1) | Indicates that any UE that is entitled to use multicast may join multicast session. |
| NOTE 1: For multicast groups, one of "Internal Group Identifier", "SUPI list", or "Any UE indication" is present. |  |  |

Table 6.4.3-2: MBS data type keys

| Data Set | Data Key | Data Sub Key |
| --- | --- | --- |
| MBS data | MBS session ID | Internal Group Identifier or SUPI |

******************** End of Change ********************