SA WG2 Meeting #147E	S2-2107406

Oct 18 - 22, 2021, Electronic

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0016 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on PCF and MB-PCF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are "MB-PCF" described in the specification, which should be replaced with "PCF", as PCF performs the functions to support MBS as defined in clause 5.3.2.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace "MB-PCF" with "PCF" in the specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification with mixed usage of "MB-PCF" and "PCF" for the same functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9.1 |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change ********************

6.9.1	MBS Session Context

The MBS Session Context contains all information describing a particular MBS session in the 5GS and is created in each node involved in the delivery of the MBS data.

The content of the Multicast MBS Session Context is described in Table 6.9.1-1.

Table 6.9.1-1: Multicast MBS Session context

| Parameter | Description | NG-RAN | MB-SMF | SMF | AMF |
| --- | --- | --- | --- | --- | --- |
| State | State of MBS session ('Active multicast session' or 'Inactive multicast session' or 'Configured multicast session') | X (note 2) | X | X (note 2) |  |
| SSM (source specific IP multicast address) | IP multicast address identifying the MBS session. |  | X (note 1) | X (note 1) |  |
| TMGI | Temporary Mobile Group Identity allocated to the MBS Session. | X | X | x | X |
| Area Session Identifier | Used for MBS session with location dependent content. When present, the Area Session Identifier together with the TMGI uniquely identify the MBS Session in a specific MBS service area. | X (note 1) | X (note 1) | X (note 1) | X |
| MB-SMF | The MB-SMF that handles the MBS session. | X |  | X | X |
| QoS information | QoS information of the MBS session. | X | X | X |  |
| MBS Service Area | Area over which the MBS session data is distributed (i.e. Cell ID list or TAI list). | X (note 1) | X (note 1) | X (note 1) | X |
| NG-RAN Node ID(s) | NG-RAN nodes which are involved in the multicast session |  |  |  | X |
| AMF | The AMF(s) which are selected for the MBS session | X | X |  |  |
| IP multicast and source address for data distribution | IP addresses identifying the SSM user plane transport for shared delivery between MB-UPF and NG-RAN and for individual delivery between MB-UPF and UPF when the IP multicast transport is used. | X (note 1) | X (note 1) | X (note 1) |  |
| SMF | The SMF(s) that manages the associated PDU session. |  | X |  |  |
| UE ID | ID identifying the UE that successfully join the Multicast MBS Session. For NG-RAN it is NGAP UE ID and for SMF it is SUPI. | X (note 3) |  | X (note 3) |  |
| IP address for distribution | The IP addresses and TEID of NG-RAN used for the user plane between NG-RAN and MB-UPF and between MB-UPF and UPF when Point to Point tunnel is used. | X (note 1) | X (note 1) | X (note 1) |  |
| TEID for data distribution | The tunnel ID used for receiving the multicast data for shared delivery by NG-RAN and for individual delivery by UPF | X | X | X |  |
| PCF | The PCF that provides policy control for the MBS session. |  | X (note 1) |  |  |
| NOTE 1: It is an optional parameter. NOTE 2: The value 'Configured multicast session' is not applicable for NG-RAN and SMF. NOTE 3: the UE ID is available within the UE Context which contains the MBS information. |  |  |  |  |  |

In Broadcast MBMS mode, an MBS Session Context is created in the NG-RAN, AMF, MB-SMF and MBSF as a result of the MBS Session Start procedure.

The content of the Broadcast MBS Session Context is described in Table 6.9.1-2.

Table 6.9.1-2: Broadcast MBS Session context

| Parameter | Description | NG-RAN | AMF | MB-SMF |
| --- | --- | --- | --- | --- |
| TMGI | Temporary Mobile Group Identity allocated to the MBS Session. | X | X | X |
| Area Session Identifier | Used for MBS session with location dependent content. When present, the Area Session Identifier together with the TMGI uniquely identify the MBS Session in a specific MBS service area. | X (note 1) | X (note 1) | X (note 1) |
| AMF | The AMF(s) which are selected for the MBS session | X |  | X |
| QoS information | QoS information for the MBS Session, including the QoS parameters of QoS flows. | X |  | X |
| MBS Service Area | Area over which the MBS session data is distributed (i.e. Cell ID list or TAI list). | X | X | X |
| NG-RAN Node ID(s) | NG-RAN nodes which are selected for the broadcast session |  | X |  |
| IP multicast address for data distribution | IP addresses identifying the user plane transport used for shared delivery between MB-UPF and NG-RAN when the IP multicast transport is used. | X (note 1) |  | X (note 1) |
| NG-RAN IP Address for data distribution | The IP address of NG-RAN used for the user plane between NG-RAN and MB-UPF when Point to Point tunnel is used. | X (note 1) |  | X (note 1) |
| TEID for data distribution | The tunnel ID used for receiving the broadcast data for shared delivery by NG-RAN | X |  | X |
| PCF | The PCF that provides policy control for the MBS session. |  |  | X (note 1) |
| NOTE 1: It is an optional parameter. |  |  |  |  |

******************** End of Change ********************