3GPP TSG-SA2 Meeting # 147E (e-meeting)	S2-2107474

# Elbonia, October 18 - 22, 2021													   (revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0021 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the local MBS service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - For location dependent MBS session, the SMF get the MB-SMF profile include all related area session ID and MBS service area it support. Similar the UE also get the whole MBS servcie area for the MBS session based on the service announcment. However for the UE joining procedure only one area session ID and its related MBS service area need be included and sent to NG-RAN node. It is not need to send other Area Session ID and MBS service Area to the NG-RAN. Thus some unnecessary plural description need be removed. - To support the individual delivery, the SMF need subscribe the UE mobility event from AMF. - For the location dependent MBS session, the Area Session ID and MBS service area are also need be registered to NRF. So it is unsuitable to assume if the MBS session ID is TMGI, the registration can be skipped. - For the HOV procedure, it give some further clarificaiton on the location dependent MBS session and limited service area MBS session. It is proposed that target side determine whether the MBS session is kept or removed. - Add new clause about the local MBS session activation and termination of the local MBS session. - For the broadcast MBS service, there are no SMF need be involved. So the NOTE about the MB-SMF need be same for the one SMF service area is unclear on how these two are related. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove unnecessary plural description - Add the UE mobility event subscritption description - Remove the MB-SMF can skip the registration part description. - Add some more detail description of the HOV procedure - Add new clause about the local MBS session activation and termination of the local MBS session. - Remove unclear NOTE description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Local MBS for individual delivery will not be supported for the HOV, Activtion, termination procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.4.2.1,7.2.4.2.2, 7.2.4.2.3, 7.2.4.2.4/7.2.4.2.5(new), 7.2.4.3.1,7.2.4.3.3, 7.2.4.3.4/7.2.4.3.5(new), 7.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

7.2.4	Support of Local multicast service

7.2.4.1	General

The procedures for Local multicast service contains the ones for Local multicast service with the location-dependent content, and the ones for limited area local multicast service distribution, as described in clause 6.2.

If 5GC Individual MBS traffic delivery is supported to be used, the SMF subscribes to the UE mobility event notification (e.g. UE moving into or out of Area Of Interest) from the AMF using the Namf_EventExposure service. The SMF provides the MBS service area of the multicast session as Area Of Interest.

At any time SMF always determine whether the UE location is inside or outside the MBS service area from the change notification (e.g. intra NG-RAN node mobility) or via the UE Location Information included in Nsmf_PDUSession_UpdateSMContext Request (e.g. inter NG-RAN node mobility).

7.2.4.2	Support of multicast service with location-dependent content

7.2.4.2.1	UE join location dependent multicast session and establishment procedure

The local multicast session join and establishment procedure is performed as defined in clause 7.2.1 with the following additions:

-	The local multicast session is configured as described in clause 7.2.4.2.2.

-	If SMF has no information about the multicast MBS session context for the indicated MBS Session ID, the SMF requests MB-SMF information via Nnrf_NFDiscovery_Request Request (MBS Session ID, UE location), the NRF provides information about the MB-SMF(s) serving the multicast session at the indicated location and service areas and service area IDs for the multicast session, via Nnrf_NFDiscovery_Request Response (MB-SMF profile (Area Session ID(s), MBS service area(s)). The SMF selects the MB-SMF based on the location area where the UE is residing and interacts with MB-SMF to retrieve QoS information of the multicast QoS flow(s) for the MBS Session ID.

-	SMF requests the AMF to transfer an N2 message to the RAN node using the Nsmf_PDUSession_UpdateSMContext response, to provide the NG-RAN with multicast session information which additionally includes the Area Session ID and MBS service area that the NG-RAN node belongs to. The SMF provides the whole MBS service area information associated with the indicated Area Session ID (Area session ID, MBS service area) to NG-RAN.

-	If RAN supports MBS, the RAN uses the received MBS Session ID and Area Session ID to determine the local multicast session context and whether the user plane for the local multicast session is already established.

7.2.4.2.2	Configuration for local MBS

For local MBS, the configuration procedure for the UE is optional and performed as defined in clause 7.1.1.2 with the following additions:

-	Multiple AFs may start the same multicast session with different content in different MBS service areas. The NEF selects MB-SMF as ingress control node(s) for different MBS service areas.

-	If presented, the NEF maps possible external identifiers for MBS service areas to network-internal identifiers (e.g. list of cells, TAIs).

-	MB-SMF allocates Area Session ID, and updates its NF profile towards the NRF with the MBS Session ID, session area and Area Session ID.

NOTE:	For a location dependent service provided in different MBS service areas within the same SMF service area, it is assumed that one MB-SMF is used for an MBS Session.

-	The policy of Multicast session is determined based on the service requirements per MBS service area.

-	The MB-SMF may select the MB-UPF based on the MBS service area.

-	The whole MBS service area(s) for the MBS session are indicated to the UE in the Service Announcement as defined in clause 6.11.

7.2.4.2.3	Handover procedure

The Handover procedure for the UE is performed as defined in clause 7.2.3 with the following additions:

Before the Handover, the UE is camping at Source RAN and receiving multicast data corresponding to the MBS Session ID and Area Session ID, the handover procedure is executed as defined in clause 7.2.3.2/7.2.3.3 with the following difference:

-	For the Xn Handover:

-	Source RAN includes MBS Session ID, Area Session ID and MBS service area to the Target RAN.

-	Target RAN determines whether to establish the resources for multicast distribution for MBS Session ID and Area Session ID provided by Source RAN, based on MBS Session ID, Area Session ID and MBS service area.

NOTE: Data forwarding issue needs the feedback of RAN WGs

-	Target RAN responses to Source RAN, with the accepted MBS Session ID, Area Session ID. When Target RAN supports multicast but the UE is no longer in the MBS service area provided by the source RAN, Target RAN rejects to handover the multicast session with a cause indication.

-	For the N2 Handover:

-	The SMF includes all MBS session area information (MBS Session ID, Area Session ID and MBS service area) to the Target RAN in Handover request.

-	Target RAN determines whether to establish the resources for multicast distribution for MBS Session ID and Area Session ID provided by SMF, based on MBS Session ID, Area Session ID, MBS service area and location area. If UE is not in the in the MBS service area provided by the SMF, Target RAN rejects to handover the multicast session with a cause indication.

-	If the target RAN determines the shared delivery is not established for the multicast session ID and area session ID, the target NG-RAN initiates the shared delivery establishment as specified in clause 7.2.1.4.

- For the Xn/N2 Handover, if the target RAN not support MBS, the associated QoS flow is established regardless whether the UE is still in the MBS service area associated with the original area session ID or not.

Before the Handover, the UE is camping at Source RAN and receiving multicast data corresponding to the MBS Session ID and Area Session ID via the 5GC Individual MBS traffic delivery, the handover procedure is executed as defined in clause 7.2.3.4 with the following difference:

- For Xn handover the associated QoS flow is established regardless whether the UE is still in the MBS service area associated with the original area session ID or not.

- For the N2 Handover:

- The SMF includes all MBS session area information (MBS Session ID, Area Session ID and MBS service area) in N2 SM information to the Target RAN in Handover request.

- If target RAN support MBS, target RAN determines whether to establish the resources for multicast distribution for MBS Session ID and Area Session ID provided by SMF, based on MBS Session ID, Area Session ID, MBS service area and location area.

- If the target RAN determines the shared delivery is not established for the multicast session ID and area session ID, the target NG-RAN initiates the shared delivery establishment as specified in clause 7.2.1.4.

After handover procedure, if UE is out of the original MBS service area, the SMF checks whether UE is in another MBS service area within the same MBS session.

- For the case UE is out of the original MBS service area but in another MBS service area of the MBS session,

- If target NG-RAN node support MBS, the SMF provides the MBS session information related to the new Area session ID to NG-RAN via AMF using the Namf_Communication_N1N2MessageTransfer service operation. Per the received the MBS session information, the 5GC shared delivery is established.

- If the target NG-RAN node does not support MBS, the SMF update the UPF to forward the MBS data packet from the tunnel associated with the old Area session ID to the new Area session ID. If the SMF does not configure the UPF receiving the MBS data packet from the tunnel associated with the new Area session ID before, the SMF update the MB-SMF by providing a new Area session ID and UPF DL N19mb Tunnel information. MB-SMF configure the MB-UPF to transmit the multicast session data towards UPF using the received downlink tunnel ID.

- The SMF updates the AMF of the area of interest event subscription by supplying the new service area as Area Of Interest.

- For the case UE is out of the whole service area of the MBS session,

- If target NG-RAN node does not support MBS, the SMF delete the associated QoS flow from NG-RAN and UPF.

- Per operator’s policy the SMF may decide to remove the UE from the MBS session

- If SMF decides to remove the UE from the MBS session due to UE is out of the service area of the MBS session,

- The SMF removes the UE from MBS session as defined in clause 7.2.2.3.

- The SMF unsubscribes at the AMF from the notifications about the UE location or the "UE moving in or out of a subscribed "Area Of Interest" event using the Namf_EventExposure service.

##### 7.2.4.2.4	Activation of location dependent MBS session

When the location dependent MBS session is activated or deactived, the MBS session is activated or deactivated in the whole MBS service area of the MBS session. It is not supported that the same MBS session is in active state in one MBS service area but in inactive state in another MBS service area.

For the location dependent MBS session activation, the difference comparing to the procedure defined in clause 7.2.5.2.

- The SMF invokes Namf_MT_EnableGroupReachability service operation to AMF, which includes the whole MBS service area associated with the MBS session, i.e. the sum of all MBS service area associated with the MBS session ID regardless of the Area session ID.

- For the UE in CM-IDLE state, when the AMF trigger the paging, it take the receiving MBS service area information into account. Only the NG-RAN node which is included in the MBS service area need trigger the paging.

- SMF checks whether the UE is in or out of the MBS service area based on the change notification or UE location information included in Nsmf_PDUSession_UpdateSMContext Request. Based on that information, the SMF determines how to activate the MBS session same as the handling after the handover procedure defined in clause 7.2.4.2.3

##### 7.2.4.2.5	SMF removing joined UEs from location dependent MBS session

Besides the condition as defined in clause 7.2.2.3 the SMF can also remove the UE from the local dependent MBS session if the UE is out of the whole service area of the MBS session. When the UE is removed from the location dependent MBS session, the SMF also unsubscribes to the AMF from the notifications about the "UE moving in or out of a subscribed "Area Of Interest" event.

7.2.4.3	Support of multicast service available within a limited area

7.2.4.3.1	Local MBS service area information provided by AF

For local multicast services, the MBS service area information is provided to the UE and the 5GC as specified in clauses 7.1.1.2 and 7.1.1.3 with the following differences and clarifications:

-	For the Service Announcement, MBS service area information is included unless it is preconfigured.

-	When performing the MBS session request to the 5GC, the MBS service area information for a multicast session is provided by the AF unless it is preconfigured.

7.2.4.3.2	Multicast session join and session establishment procedure for multicast service available within a limited area

For the case that the multicast service is only available within a limited area, the UEs outside the MBS service area are not allowed to join the multicast service.

The multicast session join and session establishment for multicast service available within a limited area are performed as specified in clause 7.2.1.3 with the following differences and clarifications:

-	The local multicast session has been configured where the AF provided the MBS service area information as specified in clause 7.2.4.3.1.

-	The UE may have information about local multicast service including MBS service area and local MBS service indication via Service Announcement as specified in clause 7.2.4.3.1.

-	In step 1, if the UE determines that it is in the MBS service area based on the information about local multicast service e.g. obtained via Service Announcement, the UE sends the PDU Session Modification Request (MBS Session ID) as a Join Request to join the multicast group. If the UE determines that it is outside the MBS service area, the UE does not send the Join Request.

-	In step 4, the SMF obtains the MBS service area (i.e. Cell ID list or TAI list) of the indicated MBS session from the MB-SMF, and the SMF determines the indicated MBS session corresponds to local multicast service based on the MBS service area.

-	In step 5, the SMF checks whether the UE is inside or outside the MBS service area.

-	If the SMF detects that the UE is inside the MBS service area, the SMF continues the process of multicast session join as specified in clause 7.2.1.3 with the following additions:

-	The SMF sends the UE a PDU Session Modification Command indicating a Join Accept as a response to the Join Request. The Joint Accept includes the MBS service area (i.e. Cell ID list or TAI list).

-	The multicast session information sent by the SMF to the NG-RAN includes the MBS service area (i.e. Cell ID list or TAI list).

-	If the SMF detects that the UE is outside the MBS service area, the SMF rejects the multicast session join, so sends a Join Reject to the UE indicating that the MBS join is rejected. The Joint Reject includes the reject reason (outside of local service area) and the MBS service area (i.e. Cell ID list or TAI list).

In this case, the MBS Session establishment (i.e. resources establishment for MBS traffic delivery) for the UE is not performed.

NOTE 1:	There may be the case that the UE determines that it is inside the MBS service area based on the geographical area information or civic address information as MBS service area information provided by the AF, so sends a Join Request but the UE is outside the MBS service area.

NOTE 2:	Which SM NAS message is used to deliver the Join Reject (e.g. PDU Session Modification Reject) is defined in stage 3 specifications.

-	If the Join Request from the UE is accepted and 5GC individual MBS traffic delivery is used, the SMF subscribes to the UE mobility event notification from the AMF (e.g. UE moving into or out of Area Of Interest, which is set by MBS service area), by invoking Namf_EventExposure_Subscribe service operation as specified in clause 5.2.2.3.2 of TS 23.502 [6].

-	For the UE that received the Join Reject from the SMF, later, if the UE detects that it is inside the MBS service area based on the MBS service area provided in the Join Reject, the UE sends the PDU Session Modification Request (MBS Session ID) to join the multicast group.

##### 7.2.4.3.3	Handover procedure with limited area MBS session

The Handover procedure for the UE is performed as defined in clause 7.2.3 with the following additions:

Before the Handover, the UE is camping at Source RAN and receiving multicast data corresponding to the MBS Session ID, the handover procedure is executed as defined in clause 7.2.3.2/7.2.3.3 with the following difference:

-	For Xn based handover in clause 7.2.3.2, Source RAN includes MBS Session ID and MBS service area to the Target RAN during Handover preparation phase. For N2 based handover in clause 7.2.3.3, this step corresponds to Handover Request and Handover Required message, respectively.

-	For Xn based handover in clause 7.2.3.2, target RAN responses to Source RAN, with the accepted MBS Session ID. When Target RAN supports MBS but the UE is no longer in the MBS service area, Target RAN does not allocate RAN resources for the MBS Session to the UE. For N2 based handover in clause 7.2.3.3, this step corresponds to Handover Request acknowledge message.

- For the Xn/N2 Handover, if the target RAN does not support MBS, the associated QoS flow is established regardless whether the UE is still in the MBS service area associated with the MBS session or not.

Before the Handover, the UE is camping at Source RAN and receiving multicast data corresponding to the MBS Session ID and Area Session ID via the 5GC individual MBS traffic delivery, the handover procedure is executed as defined in clause 7.2.3.4 with the following difference:

- For Xn handover the associated QoS flow is established regardless whether the UE is still in the MBS service area associated with the MBS session or not.

- For the N2 Handover:

- The SMF includes all MBS session area information (MBS Session ID and MBS service area) to the Target RAN in Handover request.

- If target RAN support MBS, target RAN determines whether to establish the resources for multicast distribution for MBS Session ID provided by SMF, based on MBS Session ID, MBS service area and location area. If UE is not in the in the MBS service area provided by the SMF, Target RAN rejects to handover the MBS session with a cause indication.

After handover procedure SMF checks whether the new UE location is inside or outside the service area of the MBS session.

- For the case UE is out of the service area of the MBS session,

- If target NG-RAN node does not support MBS, the SMF delete the associated QoS flow from NG-RAN and UPF.

- Per operator’s policy the SMF may decide to remove the UE from the MBS session

- If SMF decides to remove the UE from the MBS session due to UE is out of the service area of the MBS session,

- The SMF removes the UE from MBS session as defined in clause 7.2.2.3.

- The SMF unsubscribes to the AMF from the notifications about the UE location or the "UE moving in or out of a subscribed "Area Of Interest" event using the Namf_EventExposure service.

##### 7.2.4.3.4	Activation of limited area MBS session

For the limited area MBS session activation, the difference comparing to the procedure defined in clause 7.2.5.2.

- The SMF invokes Namf_MT_EnableGroupReachability service operation to AMF, which include the MBS service area associated with the MBS session.

- For the UE in CM-IDLE state, when the AMF trigger the paging, it take the receiving MBS service area information into account. Only the NG-RAN node which is included in the MBS service area need trigger the paging.

- SMF checks whether the UE is in or out of the MBS service area based on the change notification or UE location information included in Nsmf_PDUSession_UpdateSMContext Request.Based on that information, the SMF determines how to activate the MBS session same as the handling after the handover procedure defined in clause 7.2.4.3.3

##### 7.2.4.3.5	SMF removing joined UEs from limited area MBS session

Besides the condition as defined in clause 7.2.2.3 the SMF can also remove the UE from the local dependent MBS session if the UE is out of the whole service area of the MBS session. When the UE is removed from the limited area MBS session, the SMF also unsubscribes to the AMF from the notifications about the "UE moving in or out of a subscribed "Area Of Interest" event.

# * * * * 2nd change * * * *

7.3.4	Support for Local Broadcast Service

The procedures for Local broadcast service contains the ones for local broadcast service with the location-dependent content as described in clause 6.2.

The configuration procedure is performed as defined in clause 7.1.1.2 with the following additions:

-	Multiple AFs may start the same broadcast session with different content in different MBS service areas. The NEF selects MB-SMF as ingress control node(s) for different MBS service areas.

-	If presented, the NEF maps possible external identifiers for MBS service areas to network-internal identifiers (e.g. list of cells, TAIs).

-	MB-SMF allocates Area Session ID, and updates its NF profile towards the NRF with the TMGI and Area Session ID.

-	The policy of broadcast session is determined based on the service requirements per MBS service area.

-	The MB-SMF may select the MB-UPF based on the MBS service area.

-	The MBS service area(s) are indicated to the UE in the Service Announcement as defined in clause 6.11.

The MBS session establishment procedure is performed as defined in clause 7.3.1 with the following additions:

-	MB-SMF requests the AMF to transfer an N2 message (i.e. MBS Session Resource Setup Request) to the NG-RAN nodes of the MBS service area with broadcast session information which additionally includes the Area Session ID and MBS service area.

-	The NG-RAN uses the received MBS Session ID and Area Session ID to determine the local broadcast session context.

# * * * * End of change * * * *