3GPP TSG-WG SA2 Meeting #147E e-meeting  	        S2-2107480

Elbonia, October 18 – 22, 2021

Title:	[DRAFT] Reply LS on DCAMP related issues

Response to:	LS (S2-2107029/C3-214425) on DCAMP related issues

Release:	Rel-17

Work Item:	TEI17_DCAMP

Source:	SA2

To:	CT3

Cc:

Contact person:	Susan Shi

E-mail Address:	susan.shishufeng@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2107481 (CR 3189), S2-2107482 (CR 0662)

1. Overall Description:

SA2 thanks CT3 for their LS on DCAMP related issues and would like to provide the following clarifications.

Question 1:	Is it enough for the BSF to send an indication of registration/deregistration when the PCF for a PDU session registered/deregistered at the BSF (i.e. the PCF address(es) are not needed in the notification)?

Answer to Question 1: Yes, SA2 confirms it is enough. SA2 clarifies this as in the attachments.

Question 2:	Is it enough for the BSF to send the notification of registration when the PCF binding information of the first PDU session corresponding to the same UE, DNN and S-NSSAI combination is registered and send the notification of deregistration when the PCF binding information of the last PDU session corresponding to the same UE, DNN and S-NSSAI combination is deregistered?

Answer to Question 2: Yes, SA2 confirms it is enough. SA2 clarified this as in the attachments.

Question 3: Is the Application Identifier(s) and/or the (DNN, S-NSSAI) combination provided during the procedure of AF requesting Access and Mobility related Policy Authorization for a UE using the Nnef_AMPolicyAuthorization or Npcf_AMPolicyAuthorization services?

Answer to Question 3: Yes. SA2 clarified this as in the attachments.

Question 4: Could there be more information than the allowed TAIs included in the service area coverage, so there may be any other information that can be provided by the AF or reported by the PCF besides the list of allowed TAIs?

Answer to Question 4: According to the current requirement, the AF may request the PCF to extend the coverage of allowed TAI temporarily . So there is no need to introduce other information besides the allowed TAIs in the service area coverage from stage 2 point of view.

Question 5: What is the definition of “outcome”? Can it be successful, unsuccessful or anything else? Is the “successful” outcome restricted to cases in which the applied service area restrictions are exactly as indicated in the relevant AF request? Shall the applied service area coverage be reported as part of the event notification? If yes, does this apply only for the unsuccessful case?

Answer to Question 5: As defined in clause 6.1.3.18 of TS 23.503, when the PCF evaluates that the Access and Mobility related policies need any changes and if the AF has subscribed to the notification on outcome of service area coverage change, the PCF reports any change of service area coverage decided by the PCF regardless of whether they are exactly the same or not. There’s no need to report whether the AF request is successful or unsuccessful when the service area coverage is changed. SA2 clarifies this as in the attachments.

Question 6: CT3 is discussing the possibilities to implement this retrieval either via a Nudr_DM_Query request/response before subscribing or via implementing immediate reporting of existing data in the response of Nudr_DM_Subscribe. Would any of the two approaches violate stage 2 requirements? And does SA2 have a strong preference and want to mandate any of the two options or should CT3 make a decision based on the protocol level implications?

Answer to Question 6: The AM influence information is stored as Application Data in the UDR by the NEF as defined in clause 4.15.6.9.3 of TS 23.502. As the PCF for a UE does not know whether the AM influence information applicable to the UE exists in the UDR, the PCF for a UE cannot determine when to initiate the Nudr_DM_Query request to retrieve the AM influence information. Therefore, it is not appropriate to use the Nudr_DM_Query service operation to retrieve the Application Data. While SA2 agrees that the existing data can be reported in the response of Nudr_DM_Subscribe service operation for the optimization and leave details to CT3 to decide.

Question 7: Can traffic filtering information be provided indeed as an input parameter to Nnef_AMInfluence_Create (as an alternative to the application identifier) or should it be only application identifier?

Answer to Question 7: Yes, SA2 has agreed related CR S2-2106965 in SA2#146, i.e. traffic filtering information can be provided as an input parameter to Nnef_AMInfluence_Create.

2	Actions

To CT3

ACTION: 	SA2 kindly asks CT3 to take the above answers into account.

3	Dates of next TSG SA WG 2 meetings

TSG-SA2 Meeting #148E		November 15 – 19, 2021			Elbonia