3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-210723

# Elbonia, October 18 – 22, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0030 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on ProSe Direct Discovery with 5G DDNMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In 5G ProSe, the PCF is responsible for the PDUID allocation for each ProSe UE, and the 5G DDNMF gets the PDUID from the PCF for the UE during the Discovery Request procedures. So a 5G DDNMF should be able to discovery the PCF for the UE. During the AM Policy Association Establishment, the PCF registers with the BSF as the PCF that handles the AM Policy Association for this UE. An AF can find the PCF for the UE using BSF service with SUPI as input. This CR reuses the same mechanism to enable a 5G DDNMF to discovery the PCF for a UE. - In current text of 23.304 (see clause 7.2.1), the consumers of Naf_ProSe service only lists DDNMF. However, when an AF is a third-party ProSe Application Server, the NEF is needed to exposure the 5GC network function (i.e., DDNMF) to the AF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Describe how 5G DDNMF searches for the PCF for a UE using the BSF. - To add NEF as the consumer of Naf_ProSe Service and add corresponding NEF service, which is a mirror of the Naf_ProSe Service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - No support for the 5G DDNMF finding the PCF for a UE. - No support for third-party AF interactions with the DDNMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.4, 4.3.2.1, 7.2.1, 7.3(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 6.3.1.4	Discovery Request procedures

The Discovery Request procedure can be used by the "announcing UE" or "monitoring UE" in order to be authorised to access the discovery resources and perform 5G ProSe Direct Discovery. The exact signalling procedures involving the UE, the 5G DDNMFs, and the ProSe Application Server are specified in TS 23.303 [3] clause 5.3.3, with the following modifications:

-	the 5G DDNMF takes the role of "ProSe Function" in the procedure;

-	Upon receiving a Discovery Request for restricted discovery from a UE, if the 5G DDNMF does not have a valid PDUID for that UE, the 5G DDNMF searches the PCF for the UE using Nbsf_Management_Subscribe as defined in the clause 5.2.13.2.6 in TS 23.502 [5], and BSF provides the address of the PCF for the UE via Nbsf_Management_Notify as defined in the clause 5.2.13.2.8 in TS 23.502 [5]. The 5G DDNMF gets the PDUID and subscribes to notifications on Change of PDUID using Npcf_AMPolicyAuthorization_Subscribe from the PCF for the UE as defined in the clause 5.2.5.8.6 in TS 23.502 [5], including the SUPI, Event ID set to "Change of PDUID", and immediate reporting flag to indicate that the current PDUID value should be provided to the consumer. The PCF provides the PDUID and its validity timer.

-	At the time the PCF generates a new PDUID, if the subscription to "Change of PDUID" is active it sends Npcf_AMPolicyAuthorization_Notify to the 5G DDNMF to report a new PDUID and its validity timer.

-	the HSS is replaced by UDM;

-	corresponding 5GS identifiers replace the EPS identifiers, e.g. use SUPI instead of IMSI, and use GPSI instead of MSISDN;

-	PC5_tech parameter is omitted and the intended PC5 radio technology is NR.

The Discovery Request procedure can also be used by the Discoveree UE or the Discoverer UE in order to be authorised to access the discovery resources and perform 5G ProSe Direct Discovery, Model B. The exact signalling procedures are defined in TS 23.303 [3] clause 5.3.3A, with the same modifications as in the above list apply.

The events reported by the PCF, described in clause 6.1.3.18 in TS 23.503 [9] are extended to report "Change of PDUID" to the 5G DDNMF.

# * * * * Second change * * * *

### 4.3.2	5G DDNMF

#### 4.3.2.1	General

The 5G DDNMF is the logical function handling network related actions required for dynamic 5G ProSe Direct Discovery. In this version of the specification, it is assumed that there is only one logical 5G DDNMF in each PLMN that supports 5G ProSe Direct Discovery service.

NOTE:	If multiple 5G DDNMFs are deployed within the same PLMN (e.g., for load reasons), the method to locate the 5G DDNMF that has allocated a specific ProSe Application Code or ProSe Restricted Code (e.g. through a database lookup, etc.) is not defined in this version of the specification.

The 5G DDNMF interacts with the 5G ProSe-enabled UE using procedures over PC3a reference point defined in clause 6.3.1 to allocate and resolve the mapping of ProSe Applications IDs and ProSe Application Codes used in 5G ProSe Direct Discovery. It uses ProSe related subscriber data stored in UDM for the authorisation of each discovery request. It also provides the UE with the necessary security material in order to protect discovery messages transmitted over the air. In restricted 5G ProSe Direct Discovery, it also interacts with the Application Server via Npc2 reference points or with other 5G DDNMFs via Npc6/Npc7 reference points for the authorization of the discovery requests.

The 5G ProSe-enabled UE use procedure defined in clause 4.3.2.2 to discovery the 5G DDNMF in the HPLMN. Based on the UE Local Configuration or URSP as defined in TS 23.503 [9], an existing PDU session is selected or a new PDU session is established, to carry the control signalling between the UE and the 5G DDNMF in the HPLMN.

The 5G DDNMF provides the necessary charging functionality or charging information for the usage of 5G ProSe Direct Discovery and/or ProSe Direct Communication to interact with CHF or for the provision to CEF.

The 5G DDNMF in the HPLMN may interact with the 5G DDNMF in a VPLMN or Local PLMN in order to manage the 5G ProSe Direct Discovery service.

The 5G DDNMF gets the address of the PCF for the UE from the BSF.

The 5G DDNMF gets the PDUID from the PCF and subscribes to notifications on Change of PDUID.

# * * * * Third change * * * *

## 7.2	AF Services

### 7.2.1	General

This service enables consumer NF to request authorization for Discovery Request. This service is also used by producer NF to update the authorization of discovery request.

Table 7.2.1-1: Services provided by AF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Naf_ProSe | DiscoveryAuthorization | Request/Response | 5G DDNMF, NEF |
|  | DiscoveryAuthorizationUpdateNotify | Subscribe/Notify | 5G DDNMF, NEF |
|  | DiscoveryAuthorizationResultUpdate | Request/Response | 5G DDNMF, NEF |

# * * * * Forth change * * * *

## 7.3	NEF Services

### 7.3.1	General

In addition to those services defined in TS 23.501 [2] clause 7.2.8 and TS 23.502 [3] clause 5.2.6, the following NEF services are defined to support ProSe.

Table 7.2.1-1: Services provided by NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_ProSe | DiscoveryAuthorization | Request/Response | 5G DDNMF |
|  | DiscoveryAuthorizationUpdateNotify | Subscribe/Notify | 5G DDNMF |
|  | DiscoveryAuthorizationResultUpdate | Request/Response | 5G DDNMF |

### 7.3.2	Nnef_ProSe service

#### 7.3.2.1	General

Service description: This service enables consumer NF to request authorization for Discovery Request. The service also supports to update the authorization information to revoke the Restricted ProSe Direct Discovery permission.

#### 7.3.2.2	Nnef_ProSe_DiscoveryAuthorization service operation

Service operation name: Nnef_ProSe_DiscoveryAuthorization

Description: Provide authorization result of the Discovery Request from the consumer NF.

Input, Required: ProSe Application ID, Request Type.

Input, Optional: Application Level Container, Allowed number of suffixes, RPAUID, ProSe Application ID, Target RPAUID.

Output, Required: ProSe Application Code Suffix pool, Response Type, PDUID(s), Target PDUID.

Output, Optional: PDUID(s), Target PDUID, ProSe Application Code Suffix pool, Mask(s) for the ProSe Application Code Suffix(es) corresponding to ProSe Application ID, N sets of Target PDUID - Target RPAUID - Metadata Indicator, Application Level Container.

#### 7.3.2.3	Nnef_ProSe_DiscoveryAuthorizationUpdateNotify service operation

Service operation name: Nef_ProSe_DiscoveryAuthorizationUpdateNotify

Description: Provides the NF consumer updated authorization information to revoke discovery permissions relating to some other users in the NF consumer.

Input, Required: Discovery type = "restricted", RPAUID, Banned RPAUID, Banned PDUID.

Input, Optional: None.

Output, Required: Result.

Output, Optional: None.

#### 7.3.2.4	Nnef_ProSe_DiscoveryAuthorizationResultUpdate service operation

Service operation name: Nnef_ProSe_DiscoveryAuthorizationResultUpdate

Description: NF consumer reports the revocation result because of update in authorization information.

Input, Required: Discovery type = "restricted", RPAUID, Banned RPAUID, Banned PDUID.

Input, Optional: None.

Output, Required: Result.

Output, Optional: None.

# * * * * End of changes * * * *