SA WG2 Meeting #147E	S2-2107555

October 18 – 22, 2021, E-Meeting

# Title:	[DRAFT] Reply LS on Slice list and priority information for cell reselection

# Response to:	(S2-2107039/ R2-2108928)

# Release:	Rel-17

# Work Item:	NR_Slice-Core

Source:	[Ericsson, will be SA2]

To:	RAN2

Cc:	CT1, SA1

Contact Person:

Name:	Peter Hedman

E-mail Address:	peter (dot) hedman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	S2-2107556

1. Overall Description:

SA2 thanks RAN2 for the LS on Slice list and priority information for cell reselection.

RAN2 stated:

Following relevant agreements were made in RAN2#114e:

- Frequency priority mapping for each slice (slice -> frequency(ies) -> absolute priority of each of the frequency) is provided to a UE. - Frequency priority mapping for each of the slice (slice -> frequency(ies) -> absolute priority of each of the frequency) is part of the “slice info” agreed to be provided to the UE using both broadcast and dedicated signalling.

RAN2 asked:

1.	Can UE NAS provide to UE AS a list of slices (or slice group(s)), each with its corresponding priority for cell reselection evaluation in RRC_IDLE/RRC_INACTIVE?

[SA2 answer]

SA2 understands the logic of providing prioritization per network slice by UE NAS to UE AS would be UE internal and therefore details of it is better answered by CT1. However, it was questioned to keep the logic of deciding the priorities fully internal to the UE. This will open up for different behaviors among UEs, that are likely not in line with the preferences of the network operator. The logic of cell re-selection impacts e.g. 5GC and NG-RAN based paging and other functionality such as load balancing. SA2 agreed the attached CR allowing the 5GC to enable and control the functionality of UE NAS providing above mentioned priority to UE AS.

2.	Can the concept of Slice group and its signalling (Slice Group and its identifier) be supported using NAS signalling?

[SA2 answer]

SA2 understands that the usage of such Slice Group is to avoid the broadcast of S-NSSAIs, and SA2 agreed to extend the NAS signalling (Registration procedure and UE Configuration Update procedure) to enable the UE to retrieve the Slice Group that the S-NSSAIs of the Configured NSSAI belongs to. The name of the Slice Group was discussed and it was discussed whether the Slice Group should be tied to cell reselection or whether the same Slice Groups also can be used by other AS functionality like the slice based RACH functionality, as to allow the usage to be more than cell reselection the agreed CR uses the name "Network Slice RRM Group" (NSRRMG). If another name is more suitable, SA2 will align the naming.

SA2 question to RAN2:

SA2 would like to understand how RAN2 came to the conclusion that "one slice belongs to one and only one slice group"?

2. Actions:

To RAN2:

ACTION: 	SA2 asks RAN2 to take the above information into account and, if possible, answer the above question.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #148E		November 15 – 19, 2021		Elbonia

TSG-SA2 Meeting #149E	February 14 – 25, 2022		Elbonia