3GPP TSG-SA2 Meeting #147-e	S2-2107608

# Online, , 18th-22nd Oct 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 035 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification on handover procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo Mobile Communications Co. LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | [7.2.3.2], [7.2.3.3] 1. It is not clear how END MARK is used for MBS session. Further more, SMF involves in PATH SWITCH but MB-SMF does not, UPF is not know when to send END MARK if shared tunnel for target is newly created. 2. Duplicate description for N19mb shared tunnel management, which has been described in session join procedure. 3. Forwarding is still discussed by RAN WG3, no conclusion [7.2.4.2.3] 4. RAN3 indicates a issue that if MBS related informatin sent to target that not support MBS, “full configuration” may be used and all services will be interrupted. 5. This clause only focus on handling of Area Session ID, but service area also needs to be handled. [7.2.4.3.3] 6. Source RAN and Target RAN both check the MBS service area isagreed in RAN WG. SMF performs area restriction if needed. 7. Only consider the source RAN is MBS capable. 8. Does not consider the case when handover from outside to inside. 9. Some editorial modification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | [7.2.3.2], [7.2.3.3] 1. Remove END MARK in figure. 2. Refer to session join procedure for N19mb shared tunnel management. 3. Remove forwarding in figure. [7.2.4.2.3] 4. Add a EN that whether source RAN send MBS related information in the same container for PDU Session related information needs RAN3 confirmation. 5. Add a bullet that 7.2.4.3.3 for handling of service area also is applied. [7.2.4.2.3] 6. Specify that source RAN also performs MBS service area check and SMF also performs if needed. 7. Add bullets for the cases when source RAN is non-MBS capable, including Xn and N2 HO. 8. Add bullet for handover from outside to inside. 9. Some editorial modification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is not clear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.2, 7.2.3.3, 7.2.3.4, 7.2.4.2.3, 7.2.4.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.2.3.2	Xn based handover from MBS supporting NG-RAN node

This clause describes an Xn based handover with MBS traffic delivered to the UE at the source NG-RAN node supporting 5G MBS.

Figure 7.2.3.2-1: Xn based handover with MBS Session

The following additions apply compared to clause 4.9.1.2 of TS 23.502 [6]:

Before Handover:

The source NG RAN has been provided with MBS Session Resource information (including the MBS Session ID and multicast QoS flow information) and the UE Context information contains a mapping information within the PDU Session Resource associated with the MBS Session Resource, e.g. including mapped unicast QoS Flows associated with the multicast QoS flow(s) of the MBS Session Resource.

Handover Preparation Phase:

At Xn handover, the target NG-RAN is provided with information by the source NG-RAN which causes:

-	a 5G MBS non-supporting target NG-RAN node to prepare the unicast resources according to associated QoS flow(s) information.

‐	a 5G MBS supporting target NG-RAN node to allocate to the UE shared NG-RAN resources according to the MBS session information. If the 5GC Shared MBS traffic delivery for the indicated multicast MBS Session has not been established in target NG-RAN, target NG-RAN triggers setup of the resources for the 5GC shared MBS traffic delivery, see clause 7.2.1.4 for details. This is parallel with following steps 1-6.

1.	Target NG-RAN to AMF: the target NG-RAN sends N2 Path Switch Request to AMF.

The target NG-RAN node indicates supporting of 5G MBS to SMF in N2 SM information if it supports 5G MBS. According to whether the indication of supporting 5G MBS is received or not, the SMF determines the delivery method, i.e., whether the 5GC Shared MBS traffic delivery or 5GC Individual MBS traffic delivery is used for MBS data transferring.

The SMF differentiates two cases:

Case A)	 The target NG-RAN supports 5G MBS. Step 3 applies and step 4 is skipped.

3.	SMF to UPF: The SMF invokes N4 Session Modification procedure with the UPF (PSA) only for unicast PDU Session.

Case B)	 The target NG-RAN does not support 5G MBS. Step 3 is skipped, steps 4 applies.

4.	This steps is same as described in step 11 of clause 7.2.1.3.

Editor's note:	Details on data forwarding, if applicable, needs to wait for RAN WGs.

5.	SMF to AMF: The SMF responds to AMF through Nsmf_PDUSession_UpdateSMContext response.

6.	AMF to target NG-RAN: The AMF sends the path switch Ack to target NG-RAN.

# * * * * Second change * * * *

#### 7.2.3.3	N2 based handover from MBS supporting NG-RAN node

This clause describes the N2 based handover with MBS traffic delivered to the UE at the source NG-RAN node supporting 5G MBS.

Figure 7.2.3.3-1: N2 based handover with MBS Session

The following additions apply compared to clause 4.9.1.3 of TS 23.502 [6]):

1.	The Source NG-RAN includes MBS Session information, multicast QoS flow information, and mapping information in the message.

Editor's note: Whether the MBS session related information and PDU session related information are included in same RAN container or different RAN container needs to wait for RAN WGs.

4.	The Target NG-RAN prepares the radio resource based on the received information:

-	If the Target NG-RAN does not support 5G MBS, the MBS related information is not used. The Target NG-RAN uses the associated PDU Session information to allocate resource to deliver MBS data. The MBS data are transmitted via the associated QoS flows within the associated PDU Session.

-	If the Target NG-RAN supports 5G MBS, if 5GC Shared MBS traffic delivery for the indicated multicast MBS session has not been established towards the Target NG-RAN, the Target NG-RAN uses the multicast MBS Session related information to allocate RAN resources to deliver the MBS data and initiates the shared delivery establishment towards the MB-SMF as described in clause 7.2.1.4.

5.	If the Target NG-RAN supports 5G MBS, it indicates supporting of 5G MBS to SMF in N2 SM information.

13-14. Same as described in steps 3-4 of clause 7.2.3.2.

Editor's note:	Details on data forwarding, if applicable, needs to wait for RAN WGs.

# * * * * Third change * * * *

#### 7.2.3.4	Xn/N2 based handover from non-MBS supporting NG-RAN node

When the UE has joined the multicast MBS session and the source NG-RAN node does not support 5G MBS, the 5GC Individual MBS traffic delivery method is used for the multicast MBS session data delivery. When the Xn/N2 based handover procedure is triggered, the UE is handed over to the target NG-RAN node per existing Xn /N2 based handover procedure defined in TS 23.502 [6].

The following applies for an Xn based handover from an NG-RAN node not supporting 5G MBS:

-	The source NG-RAN node requests the associated QoS Flow(s) in the associated PDU session to be handed over to the target NG-RAN node.

-	In the Path Switch Request message, the target NG-RAN node indicates whether it supports 5G MBS to the SMF in the N2 SM information.

-	After successful handover, if the target NG-RAN node supports 5G MBS, the SMF triggers modification of the associated PDU Session to remove the resources of the associated QoS flows at the target NG-RAN node by including the multicast MBS session related information in N2 SM Information as described in step 6 of clause 7.2.1.3, which may trigger the target NG-RAN node to initiate establishment of shared delivery as described in clause 7.2.1.4..

-	During the PDU Session modification, if the target NG-RAN node supports 5G MBS, the SMF changes the MBS session data delivery method from 5GC Individual MBS traffic delivery method to 5GC shared MBS traffic delivery method, and sends N4 Session modification message to UPF to configure UPF to not forward the received multicast MBS session data via the associated PDU Session. If there are no multicast MBS session data forwarding via the associated PDU session(s), the SMF may also release the shared tunnel between the UPF and MB-UPF.

The following applies for an N2 based handover from an NG-RAN node not supporting 5G MBS:

-	During handover preparation phase, the SMF includes the multicast MBS session related information in N2 SM Information as described in step 6 of clause 7.2.1.3 and sends it to the target NG-RAN. The target NG-RAN indicates whether it supports 5G MBS to SMF in N2 SM information. If the target NG-RAN node supports 5G MBS, the target NG-RAN node adds the UE into the MBS session context. If 5GC Shared MBS traffic delivery for the indicated multicast MBS session has not been established towards the target NG-RAN, the target NG-RAN uses the multicast MBS Session related information to allocate RAN resources to deliver the MBS data and initiates the shared delivery establishment towards the MB-SMF as described in clause 7.2.1.4.

-	Based on the received indication of supporting 5G MBS in N2 SM information from the target NG-RAN, the SMF determines the MBS data delivery method for the multicast MBS session. The SMF configures UPF to not forward the received multicast service data to the associated PDU Session via N4 Session modification message, i.e. SMF changes 5GC individual MBS traffic delivery to 5GC shared MBS traffic delivery. If there are no multicast MBS session data forwarding via the associated PDU session(s), the SMF may also release the shared tunnel between the UPF and MB-UPF.

# * * * * Fourth change * * * *

##### 7.2.4.2.3	Handover procedure

The handover procedure for the UE is performed as defined in clause 7.2.3 with the following additions:

-	Before the handover, the UE is camping at the Source RAN node and receiving multicast data corresponding to the MBS Session ID and Area Session ID.

-	For the Xn based handover:

-	If the Source RAN node supports 5G MBS, it includes MBS Session ID, Area Session ID and MBS service area corresponding to the Area Session ID to the Target RAN node.

Editor's Notes: Whether source RAN node indicates the MBS related information to target RAN node no matter whether target RAN node supports 5G MBS or not needs RAN confirmation.

-	If both the Source RAN node and the Target RAN node support 5G MBS, when the Target RAN node determines the shared delivery is not established for the received MBS Session ID and Area Session ID, the Target NG-RAN node initiates the shared delivery establishment as specified in clause 7.2.1.4 with the MBS Session ID and Area Session ID.

Editor’s Notes: Data forwarding issue needs the feedback of RAN WGs

-	If both the Source RAN node and the Target RAN node support 5G MBS, the Target RAN node responses to the Source RAN node with the accepted MBS Session ID, Area Session ID.

-	If the Source RAN node does not support 5G MBS and the Target RAN node supports 5G MBS, the SMF includes MBS Session ID, Area Session ID and MBS service area corresponding to the Area Session ID in N2 SM information sent to the Target RAN node during the modification of associated PDU Session.

-	For the N2 Handover:

-	If the Source RAN node does not support 5G MBS, the SMF includes the MBS Session ID, Area Session ID and MBS service area corresponding to the Area Session ID in N2 SM Information sent to the Target RAN node in Handover Request message.

-	If the Target RAN node supports 5G MBS, when it determines 	the shared delivery is not established for the MBS Session ID and Area Session ID, the target NG-RAN initiates the shared delivery establishment as specified in clause 7.2.1.4 with the MBS Session ID and Area Session ID.

-	Clause 7.2.4.3.3 also is applied for the MBS service area corresponding to the Area Session ID.

# * * * * Fifth change * * * *

##### 7.2.4.3.3	Handover procedure with limited area MBS session

The handover procedure for the UE is performed as defined in clause 7.2.3 with the following additions:

-	Before the handover, the UE is camping at the Source RAN node and receiving multicast data corresponding to the MBS Session ID.

-	For Xn based handover in clause 7.2.3.2, if the Source RAN node supports 5G MBS, when the Source RAN node determines to handover the multicast MBS session according to the MBS service area, it indicates MBS Session ID and MBS service area to the Target RAN node during handover preparation phase. For N2 based handover in clause 7.2.3.3, if the Source RAN node supports 5G MBS, this step corresponds to Handover Request message and Handover Required message, respectively, if the Source RAN node does not support 5G MBS, the SMF includes MBS Session ID and MBS service area in N2 SM information sent to the Target RAN node in Handover Request message.

Editor's Notes: During Xn handover, whether source RAN node indicates the MBS related information to target RAN node no matter whether target RAN node supports 5G MBS or not, needs RAN confirmation. During N2 handover, whether source RAN node includes the MBS session related information and PDU session related information in same or different RAN container needs RAN confirmation.

-	For Xn based handover in clause 7.2.3.2, if both the Source RAN node and the Target RAN node support 5G MBS, the Target RAN node responses to Source RAN, with the accepted MBS Session ID based on the MBS service area of the multicast MBS session. For N2 based handover in clause 7.2.3.3, this step corresponds to Handover Request Acknowledge message.

-	For both Xn based handover and N2 based handover, when the Target RAN node supports 5G MBS, if the UE is no longer in the MBS service area of the multicast MBS session, the Target RAN does not allocate RAN resources for the multicast MBS Session to the UE and does not initiate shared delivery establishment as specified in clause 7.2.1.4.

-	After Xn based handover or N2 based handover, if the Target RAN node does not support 5G MBS, when the SMF determines that the UE is handed over to a target cell outside the MBS service area based on the notification of UE location, the SMF modifies the associated PDU session to remove the associated QoS flows related to the multicast MBS session.

-	After Xn based handover, if the Source RAN node does not support 5G MBS while the Target RAN node supports 5G MBS, when the SMF determines that the UE is handed over to a target cell outside the MBS service area based on the notification of UE location, the SMF modifies the associated PDU session to remove the associated QoS flows related to the multicast MBS session, and does not provide the multicast MBS session related information in N2 SM Information to the target RAN node.

NOTE:	Whether the associated QoS Flow(s) are removed from UE, NG-RAN, or only resource in NG-RAN is removed is up to implementation.

-	Before the handover, the UE is camping at the Source RAN node outside the MBS service area corresponding to the MBS Session ID.

-	After Xn based handover or N2 based handover, when the SMF determines that the UE is handed over to a target cell inside the MBS service area based on the notification of UE location, the SMF triggers modification of the associated PDU Session to configure the resources of the multicast MBS session towards the UE by including the multicast MBS session related information in N2 SM Information as described in step 6 of clause 7.2.1.3, which additionally contains MBS service area.

# * * * * End of changes * * * *