SA WG2 Meeting #147E	S2-2108006

Oct 18 - 22, 2021, Electronic	(revision of S2-2107402)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0012 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multicast session join and session establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Qualcomm, Huawei, HiSilicon, ZTE, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The procedure performed by the SMF towards the UE for rejecting the join request when e.g. authorization check fails or the UE joins prior to the start time of the multicast session, is unclear. - The multicast QoS information is sent to the NG-RAN and the accepted multicast QoS flow information is sent by the NG-RAN during the establishment of shared delivery procedure (i.e. step 9), not in step 8 or 11 respectively. - There are duplicated descriptions about the AMF selecting the SMF “based on the DNN and S-NSSAI use for establishing a PDU Session associated with multicast session(s)”. And some other editorial corrections are needed. - In C1-214884 CT1 agreed to the the possiblity to combine the establishment of the associated PDU session with the MBS join. In case a UE discovers a Multicast session it would like to join, and a PDU Session that can be associated to the Multicast session is not established yet, allowing the UE to request PDU session establishment and Multicast join at the same time, prevents the need to perform two session management procedures serially, which reduces greatly delay to join and over the air signalling overhead (as the SM signalling and RRC reconfiguration occurs only once jointly, and not twice). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the handling by the SMF towards the UE when join request is rejected. - Remove the descriptions of multicast QoS flow information from incorrect steps. - Editorial corrections. - Allow PDU session establishment for the associated PDU session and Multicast join to be requested in same NAS-SM message. In that case, PDU session establishment in the network is executed before MBS join procedure. The joint reply is created in SMF, after both PDU session establishment and MBS join are executed in the 5GC. This aligns TS 23.247 with TS 24.501 (Stage 3). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect and incomplete specification on multicast session join and session establishment procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1.3, 9.1.3.3 |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change ********************

7.2.1.3	Multicast session join and session establishment procedure

The following steps are executed before the UE requests to join the MBS session:

-	The MBS Session may have been configured in the 5GC (see clause 7.1.1 for details).

-	The UE registers in the PLMN and may have established a PDU session that can be associated with multicast session(s).

-	The UE has known at least the MBS Session ID of a multicast group that the UE can join, e.g. via service announcement.

Figure 7.2.1.3-1: PDU Session modification for UE joining multicast session

1.	To join the multicast group, the UE sends a PDU Session Modification Request for the associated PDU session which additionally contains one or several MBS Session ID(s) and join request. The MBS Session ID(s) indicate the multicast MBS session(s) that UE wants to join.

The UE may alternatively join the multicast MBS session by sending PDU Session Establishment Request for associated PDU session together with one or several MBS Session ID(s) and join request. In that case, before step 2, the network proceeds with establishment of the associated PDU session executing steps 4 to10 of PDU Session Establishment procedure as specified in TS 23.502 [6] clause 4.3.2.2.

2.	[Conditional] Based on the received MBS Session ID and join request, the SMF determines this is multicast MBS Session join request.

If SMF has no information about MBS Session context for the indicated MBS Session ID(s), SMF discovers and selects an MB-SMF for the MBS Session via the NRF as described in clause 7.1.2. If no MB-SMF is assigned for the MBS session ID (i.e. the NRF provides empty MB-SMF profile), the SMF may select an MB-SMF and request it to configure the multicast MBS session or the SMF may reject the join request and respond to the UE with an appropriate cause value.

NOTE 1:	Details about how the SMF selects an MB-SMF and requests it to configure the multicast MBS session are left to SMF implementation.

3.	[Conditional] For each MBS session in step 1, if the SMF has not subscribed to the MBS session context, it invokes Nmbsmf_ MBSSession_ContextStatusSubscribe request (MBS Session ID) towards the MB-SMF to subscribe to events notifications related to the multicast MBS session and to request information about the MBS session context. The MB-SMF responds with the information about the indicated multicast MBS session in Nmbsmf_MBSSession_ContextStatusSubscribe response (multicast QoS flow information (e.g., QoS profile(s) for multicast MBS session), [start time], [session status indication (active/inactive)], [MBS session authorization information (MBS session open for any user)], [multicast DL tunnel info]).

If it is the first time for the MB-SMF to receive Nmbsmf_ MBSSession_ContextStatusSubscribe request of the indicated MBS Session from any SMF, the MB-SMF learns it is the first UE joining the multicast MBS session. For multicast transport between MB-UPF and content provider, if it is the first UE joining the multicast MBS session, and MB-UPF has not joined the multicast tree in the MBS configuration procedure, described in clause 7.1.1, the MB-SMF requests the MB-UPF to join the multicast tree towards the AF/MBSF, otherwise MB-SMF will not send the request to the MB-UPF.

NOTE 2:	The MB-SMF can answer the Nmbsmf_ MBSSession_ContextStatusSubscribe request either based on information received in the configuration procedures in Clause 7.1.1 or based on preconfigured information. The pre-configuration also includes information about the MBS session stored in the NRF. If the MB-SMF uses preconfigured information, the pre-configuration also includes MB-UPF configuration.

4.	The MB-SMF determines whether the user is authorized to join the multicast session as follows: The MB-SMF checks the user subscription data received from the UDM to determine whether the user is allowed to use any multicast service. If so, the MB-SMF checks the received indication whether the multicast session is open for any user. If the multicast session is not open to any user, the MB-SMF checks the user subscription data received from the UDM to determine whether the MBS session ID is included. If authorization check fails, the SMF rejects the join request with a cause value. If a UE joins prior to the start time of the multicast MBS session, the SMF may accept the join request and indicate to the UE the start time, or it may reject the join request with an appropriate error cause and optionally a back-off timer. If a UE joins while the multicast MBS session is inactive, the SMF accepts the join request.

NOTE 2:	The MB-SMF can answer the Nmbsmf_ MBSSession_ContextStatusSubscribe request either based on information received in the configuration procedures in Clause 7.1.1 or based on preconfigured information. The pre-configuration also includes information about the MBS session stored in the NRF. If the MB-SMF uses preconfigured information, the pre-configuration also includes MB-UPF configuration.

5.	If the join request is accepted, the SMF responds to the AMF through Nsmf_PDUSession_UpdateSMContext response (N2 SM information (PDU Session ID, MBS Session ID, [updated PDU Session information], [mapping information between unicast QoS flow(s) and multicast QoS flow (s)]), N1 SM container (PDU Session Modification Command)) to:

-	create an MBS session context for the indicated MBS session in the RAN, if it does not exist in the RAN already; and

-	inform the NG-RAN about the relation between the multicast MBS Session context and the UE's PDU Session context by including the MBS session ID and the mapping between the multicast QoS flow(s) and associated QoS flow(s).

Based on operator policy, the SMF may prepare for 5GC individual MBS traffic delivery fall-back. The SMF maps the received QoS information of the multicast QoS Flow into PDU Session's unicast QoS Flow information, and includes the information of the QoS Flows and the mapping information about the QoS Flows in the SM information sent to RAN.

NOTE 3:	Detailed information included in N2 SM information will be aligned with by RAN WG3.

NOTE 4:	A PDU Session UP activation is not triggered by the N2 SM information if it only includes information related to the multicast MBS session and associated QoS flows and is received by an MBS capable NG RAN node.

If the MBS session join procedure was triggered by the UE together with PDU Session Establishment procedure for the associated PDU session, the SMF provides the N2 SM information and N1 SM container for the associated PDU session in Namf_Communication_N1N2MessageTransfer service operation towards the AMF, as described in step 11 of clause 4.3.2.2.1 in TS 23.502 [6]. The N2 SM information also includes the MBS Session ID and, if 5GC individual MBS traffic delivery fall-back is supported, the mapping information between unicast QoS flow(s) and multicast QoS flow (s).

Editor's Note: The implication of not triggering PDU Session UP activation in NG-RAN when SMF informs the NG-RAN of UE join requires RAN collaboration.

Editor's note:	Possible PCF interactions related to the multicast QoS flows are FFS.

If the join request is rejected, the SMF responds to the AMF through Nsmf_PDUSession_UpdateSMContext response (N1 SM container (PDU Session Modification Reject)) and the message will not contain any MBS session context or the N2 SM information for the associated PDU session. The PDU Session Modification Reject message is forwarded to the UE via the NG-RAN, and the following steps are skipped.

6.	The N2 message, which includes the multicast MBS session information and PDU session modification information is sent to the NG-RAN.

If the 5G MBS is not supported by NG-RAN, 5GC individual MBS traffic delivery may be used. Otherwise if the MBS is supported by NG-RAN, 5GC shared MBS traffic delivery is adopted.

If the NG-RAN supports 5G MBS, the NG-RAN uses the MBS Session ID to determine that the PDU Session identified by the PDU Session ID is associated with the indicated multicast MBS session.

If the NG-RAN supports MBS, the associated unicast QoS flow information is not used to allocate the radio resource and CN resource.

NOTE 5:	It is NG-RAN that decides whether radio resource is allocated or not, and it is NG-RAN/UPF that decides whether multicast transport or unicast transport is used between the NG-RAN/UPF and the MB-UPF.

7. [Conditional]	 If shared tunnel has not been established for the multicast MBS session towards the NG-RAN node, the procedures in clause 7.2.1.4 for the establishment of shared delivery toward NG-RAN node are executed. This step is executed separately for each multicast MBS session.

8.	The NG-RAN node performs AN specific signalling exchange with the UE to establish radio resource for the multicast MBS session if not established yet. If the NG-RAN does not support MBS, radio resource are reconfigured for unicast transmission of the MBS data over the associated PDU session. As part of the AN specific signalling exchange, the N1 SM container (PDU Session Modification Command) is provided to the UE.

9.	The NG-RAN node sends the PDU session modification response.

If the MBS is not supported by NG-RAN, the accepted unicast QoS flow is included in the N2 SM response container. If the MBS is supported by NG-RAN, the N2 SM response container further includes the indication of supporting MBS.

10.	The AMF invokes Nsmf_PDUSession_UpdateSMContext request ([N2 SM container]) to the SMF.

Per the indication of whether the NG-RAN supports MBS, the SMF determines the delivery mode, i.e. whether 5GC individual MBS traffic delivery is used for multicast data transmission.

NOTE 6:	If the shared tunnel is used, no interaction with UPF is needed for the indicated multicast MBS session

[Conditional] This step is used for 5GC Individual MBS traffic delivery, if the related NG-RAN does not support MBS. If a shared tunnel between the UPF (PSA) and MB-UPF for 5GC individual MBS traffic delivery has not yet been established by the SMF for the multicast MBS session, steps 11a to 11e are executed. Step 11f is executed irrespective of that.

11a.	The SMF contacts the UPF to request the creation of a tunnel and provides the MBS session ID. The UPF indicates to the SMF whether the tunnel for this multicast MBS session is newly allocated (as there can be multiple SMFs interacting with the same UPF for the same multicast MBS Session).

If the UPF determines to use unicast transport over N19mb, the UPF allocates a DL N19mb Tunnel endpoint for the multicast MBS session if the SMF request is the first one to allocate DL N19mb Tunnel endpoint for the multicast MBS Session in the UPF. The UPF includes the DL Tunnel Info in the response to the SMF. The DL tunnel info includes the downlink tunnel ID and the UPF address.

Editor's Notes: whether 13b to 13e can be skipped for multicast N10mb is FFS

11b.	 If the UPF indicates the DL N19mb Tunnel is newly allocated, the SMF invokes Nmbsmf_MBSsession_ ContextUpdate request (MBS Session ID, [DL tunnel info]) towards the MB-SMF for establishing the multicast MBS session transport between MB-UPF and UPF.

11c.	 If the DL tunnel info of the UPF is received, the MB-SMF configures the MB-UPF to transmit the multicast MBS session data towards UPF using the possibly received downlink tunnel ID.

11d.	 The MB-SMF responds to the SMF through Nmbsmf_MBSSession_ ContextUpdate response (MBS Session ID, [multicast DL tunnel info]). If the UPF DL tunnel info for unicast transport is not received by the MB-SMF, multicast transport between MB-UPF and UPF is to be used, and the MB-SMF includes the downlink tunnel information with the low layer transport multicast address for the multicast MBS session.

11e.	 For multicast transport between MB-UPF and UPF, SMF configures UPF to receive the multicast session data. If multicast transport over N19mb is used, the UPF joins the source specific multicast group of MB-UPF. The UPF forwards the data to the PDU session.

11f.	 The MB-SMF configures the MB-UPF to forward the received multicast MBS session data within the PDU session. (This step may be combined with step 11a or 11e).

12.	The SMF responds to the AMF with Nsmf_PDUSession_UpdateSMContext response message.

13.	The MB-UPF receives multicast PDUs, either directly from the content provider or via the MBSTF that can manipulate the data.

Steps 14 to 16 are for 5GC shared MBS traffic delivery:

14.	The MB-UPF sends multicast PDUs in the N3mb tunnel associated to the multicast MBS session to the NG-RAN. There is only one tunnel per multicast MBS session and NG-RAN node, i.e., all the UEs which have joined the multicast MBS session via the NG-RAN node share this tunnel for reception of the multicast MBS session data.

15.	The NG-RAN selects PTM or PTP radio bearers to deliver the multicast PDUs to the UE(s) that have joined the multicast MBS session.

16.	The NG-RAN transmits the multicast MBS session data to the UE(s) using the selected PTM or PTP radio bearer(s).

Steps 17 to 19 are for 5GC individual MBS traffic delivery:

17.	The MB-UPF sends multicast PDUs in the N19mb tunnel associated to the multicast MBS session to the UPF. There is only one tunnel per multicast MBS session and destination UPF, i.e., all associated PDU sessions served by the destination UPF share this tunnel.

18.	The UPF forwards the multicast data towards the NG-RAN via unicast (i.e. in the N3 tunnel of the associated PDU Session).

19.	The NG-RAN forwards the multicast MBS session data to the UE via unicast (i.e. over the radio bearer(s) corresponding to the associated QoS flow(s) of the associated PDU Session).

NOTE 7:	Details of the DL MBS data transmission are described in clause 6.7.

NOTE 8:	When the MBSF is involved in the multicast MBS session, the tunnel between MBSTF and MB-UPF has been established in the configuration procedure.

******************** 2nd Change ********************

#### 9.1.3.3	Nmbsmf_ MBSSession_ContextStatusSubscribe service operation

Service operation name: Nmbsmf_ MBSSession_ContextStatusSubscribe

Description: Service Consumer NF can use this service operation to request information (e.g. QoS information) about a multicast session and to subscribe to notification of events about the multicast session context.

Inputs, Required: Multicast Session ID, notification target address, Events ID(s).

Inputs, Optional: None.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID.

Outputs, Optional: Event information (e.g. QoS information for multicast session, multicast MBS session status (activated, deactivated), multicast session service area for local multicast service), Start time of multicast MBS session, multicast DL tunnel info, if consumer is SMF: indication that the multicast MBS session allows any UE to join.

******************** End of Change ********************