SA WG2 Meeting #S2-147E	S2-2108142

18 October – 22 October 2021, Electronic, Elbonia	(revision of S2-2107663r01)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3210 | rev | 1 | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the Setting up and update AF session with required QoS procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-10-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the clause 4.15.6.6a step 3a, the condition which trigger difference service operation is unclear. In the clause 4.15.6.6a step 3e, the UE-DS-TT residence time also may be provided by step 3d in clause 4.15.6.6. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the condition to trigger Ntsctsf_QoSandTSCAssistance_Update or Ntsctsf_QoSandTSCAssistance_Create service operation. In the step 3 of AF session with required QoS update procedure, it clarifies that the UE-DS-TT residence time can also provided by step 3d in clause 4.15.6.6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In which step the UE-DS-TT is provided is not fully correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.6a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.15.6.6a	AF session with required QoS update procedure

Figure 4.15.6.6a-1: AF session with required QoS update procedure

1.	For an established AF session with required QoS, the AF may send a Nnef_AFsessionWithQoS_Update request message (AF Identifier, Transaction Reference ID, [Flow description(s)], [QoS reference], [Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order)]) to NEF for updating the reserved resources. Optionally, a period of time or a traffic volume for the requested QoS can be included in the AF request. The Transaction Reference ID provided in the AF session with required QoS update request message is set to the Transaction Reference ID that was assigned, by the NEF, to the Nnef_AFsessionWithQoS_Create request message. The AF may in addition provide the following individual QoS parameters: Requested 5GS delay (optional), priority (optional), Requested GFBR, Requested MFBR, flow direction, Burst Size (optional), Burst Arrival Time (optional) at UE (uplink) or UPF (downlink), Periodicity (optional), Time domain (optional), Survival Time (optional). When Alternative Service Requirements are provided by the AF, a set of Alternative QoS Related parameters as specified in clause 5.7.1.2a of TS 23.501 [2] (e.g. one or more from Alternative Requested GFBR, Alternative Requested MFBR) may be provided for each QoS Reference.

2.	The NEF authorizes the AF request of updating AF session with required QoS and may apply policies to control the overall amount of pre-defined QoS authorized for the AF. If the authorisation is not granted, all steps (except step 5) are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed.

If the NEF does not receive any of the individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20] from the AF, then the steps 3, 4, 5, 6, 7 are executed, otherwise, the steps 3a, 3b, 3c, 3d, 3e, 4a, 4b, 5, 6a, 6b, 7 are executed.

3.	If the NEF does not receive any of the individual QoS parameters from the AF as described in clause 6.1.3.22 of TS 23.503 [20], the NEF interacts with the PCF by triggering a Npcf_PolicyAuthorization_Update request and provides UE address, AF Identifier, Flow description(s), the QoS reference and the optional Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order). Any optionally received period of time or traffic volume is also included and mapped to sponsored data connectivity information (as defined in TS 23.503 [20]).

If the AF is considered to be trusted by the operator, the AF uses the Npcf_PolicyAuthorization_Update request message to interact directly with PCF to update the reserving resources for an AF session.

If Alternative QoS Related parameter set(s) are provided by the AF, they are sent to the PCF via TSCTSF. The TSCTSF determines the 5GS delay considering UE-DS-TT residence time.

3a.	If the NEF receives one or more of the individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20] from the AF, the NEF forwards these received individual QoS parameters in the Ntsctsf_QoSandTSCAssistance_Update or Ntsctsf_QoSandTSCAssistance_Create request message to the TSCTSF.

If the AF is considered to be trusted by the operator, the AF uses the Ntsctsf_QoSandTSCAssistance_Update request message to interact directly with TSCTSF to update the reserving resources for an AF session with individual QoS parameters.

If the session between NEF and TSCTSF has been established, the NEF sends a Ntsctsf_QoSandTSCAssistance_Update request to TSCTSF, the steps 3b-3d are skipped.

If the session between NEF and TSCTSF has not been established, the NEF discover the TSCTSF and sends a Ntsctsf_QoSandTSCAssistance_Create request to TSCTSF as follows:

A TSCTSF address may be locally configured (a single TSCTSF per DNN/S-NSSAI) in the NEF, PCF and trusted AF. Alternatively, the TSCTSF is discovered from NRF and the TSCTSF stores the TSCTSF NF ID and Notification Target Address to the UDR (Data Set = Application Data, Data Subset = Time-Sync).

If the TSCTSF address is locally configured in the NEF and PCF, steps 3b and 3c are skipped, otherwise, they are executed to retrieve the Notification Target Address to the TSCTSF.

(If local configuration is not available): The NEF retrieves the Time-Sync Data Subset from the UDR for the given DNN/S-NSSAI. If the Time-Sync data does not contain a TSCTSF NF ID, the NEF discovers the TSCTSF from NRF. If the Time-Sync data contains a TSCTSF NF ID, the NEF uses this TSCTSF NF ID to send the Ntsctsf_QoSandTSCAssistance_Create request message.

3b.	(If local configuration is not available, the PCF subscribes to any Time-Sync information updates from the UDR) The TSCTSF stores the Notification Target Address and TSCTSF NF ID in the UDR (Data Set = Application Data; Data Subset = Time-Sync data, Data Key = S-NSSAI and DNN).

3c.	(if local configuration is not available, the PCF receives any TSCTSF information updates from the UDR) The PCF that has subscribed to modifications of AF requests (Data Set = Application Data; Data Subset = Time-Sync data, Data Key = S-NSSAI and DNN) receive(s) a Nudr_DM_Notify notification of data change from the UDR.

3d.	(if the TSCTSF address is locally configured in the PCF): the PCF sends Npcf_PolicyAuthorization_Notify message, containing the received UE-DS-TT Residence Time, to the TSCTSF at the time the SM Policy Association is established or updated including the UE-DS-TT Residence Time.

(If local configuration is not available in the PCF): the PCF sends Npcf_PolicyAuthorization_Notify message to the TSCTSF after retrieving the TSCTSF address from the Time-Sync data in the UDR.

3e.	The TSCTSF interacts with the PCF by triggering a Npcf_PolicyAuthorization_Update or Npcf_PolicyAuthorization_Create request and provides UE address, AF Identifier, Flow description(s), the QoS reference and the optional Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order). Any optionally received period of time or traffic volume is also included and mapped to sponsored data connectivity information (as defined in TS 23.203 [24]).

If the TSCTSF receives the Requested 5GS delay, the TSCTSF calculates a Requested PDB by subtracting the UE-DS-TT residence time (provided by the PCF in step 3d, or step 3d in clause 4.15.6.6) from the Requested 5GS delay. If the Requested PDB is not provided, the PCF determines the PDB that matches the QoS Reference.

Editor's note:	Whether and how the PCF uses a Priority value provided by an AF other than the TSN AF is FFS.

If the TSCTSF receives any of the flow direction, Burst Arrival Time, Periodicity, Time domain, Survival Time from the NEF, the TSCTSF forwards these parameters in the TSC Assistance Container in the Npcf_PolicyAuthorization_Update request to the PCF. The TSCTSF sends the Requested PDB, the TSC Assistance Container, and other received individual QoS parameters in the Npcf_PolicyAuthorization_Update request to the PCF.

4.	If the PCF received request from the NEF in step 3, the PCF determines whether the request is authorized.

If the request is authorized, the PCF derives the required QoS parameters based on the information provided by the NEF and determines whether this QoS is allowed (according to the PCF configuration), and notifies the result to the NEF. In addition, if the Alternative Service Requirements are provided, the PCF derives the Alternative QoS parameter set(s) from the one or more QoS reference parameters contained in the Alternative Service Requirements in the same prioritized order (as defined in TS 23.503 [20]).

If the NEF provides Requested PDB, Requested GFBR, Requested MFBR or Burst Size, then the PCF sets the PDB and/or MDBV according to the received Requested PDB and Burst Size received from the NEF. It also sets the GFBR and MFBR according to the requested values provided by the NEF. NEF specified parameter values are used to over-ride default values for the 5QI corresponding to the NEF provided QoS Reference. If the NEF provides Alternative QoS Related parameter set(s), then for each set, the PCF sets the PDB and MDBV in the corresponding Alternative QoS parameter set as described in clause 6.1.3.22 of TS 23.503 [20]. It also sets the GFBR and MFBR according to the alternative requested values sent by the NEF. NEF specified parameter values are used to over-ride default values for the 5QI corresponding to the NEF provided QoS Reference in the Alternative Service Requirements.

If the PCF determines that the SMF needs updated policy information, the PCF issues a Npcf_SMPolicyControl_UpdateNotify request with updated policy information about the PDU Session as described in the PCF initiated SM Policy Association Modification procedure in clause 4.16.5.2.

If the AF is considered to be trusted by the operator, the PCF sends the Npcf_PolicyAuthorization_Update response message directly to AF.

If the request is not authorized or the required QoS is not allowed, NEF responds to the AF in step 5 with a Result value indicating the failure cause.

4a. If the PCF received request from the TSCTSF in step 3e, the PCF determines whether the request is authorized.

If the request is authorized, the PCF derives the required QoS parameters based on the information provided by the TSCTSF and determines whether this QoS is allowed (according to the PCF configuration for this AF), and notifies the result to the TSCTSF. In addition, if the Alternative Service Requirements are provided, the PCF derives the Alternative QoS parameter set(s) from the one or more QoS reference parameters contained in the Alternative Service Requirements in the same prioritized order (as defined in TS 23.503 [20]).

If the TSCTSF provides Requested PDB, priority, Requested GFBR, Requested MFBR or Burst Size, then the PCF sets the PDB and/or MDBV according to the received Requested PDB and Burst Size received from the TSCTSF. If the Requested PDB is not provided from TSCTSF, the PCF determines the PDB that matches the QoS Reference. It also sets the GFBR and MFBR according to the requested values provided by the TSCTSF. TSCTSF specified parameter values are used to over-ride default values for the 5QI corresponding to the TSCTSF provided QoS Reference.

If the PCF determines that the SMF needs updated policy information, the PCF issues a Npcf_SMPolicyControl_UpdateNotify request with updated policy information about the PDU Session as described in the PCF initiated SM Policy Association Modification procedure in clause 4.16.5.2.

If the request is not authorized or the required QoS is not allowed, TSCTSF responds to the NEF in step 4b with a Result value indicating the failure cause.

4b.	The TSCTSF sends a Ntsctsf_QoSandTSCAssistance_Update or Ntsctsf_QoSandTSCAssistance_Create response message (Transaction Reference ID, Result) to the NEF. Result indicates whether the request is granted or not.

If the AF is considered to be trusted by the operator, the TSCTSF sends the Ntsctsf_QoSandTSCAssistance_Update response message directly to AF.

5.	The NEF sends a Nnef_AFsessionWithQoS_Update response message (Transaction Reference ID, Result) to the AF. Result indicates whether the request is granted or not.

6.	The PCF sends Npcf_PolicyAuthorization_Notify message to the NEF when the modification of the transmission resources corresponding to the QoS update succeeded or failed.

If the AF is considered to be trusted by the operator, the PCF sends the Npcf_PolicyAuthorization_Notify message directly to AF.

6a.	The PCF sends Npcf_PolicyAuthorization_Notify message to the TSCTSF when the modification of the transmission resources corresponding to the QoS update succeeded or failed.

6b.	The TSCTSF sends Ntsctsf_QoSandTSCAssistance_Notify message with the event reported by the PCF to the NEF.

If the AF is considered to be trusted by the operator, the TSCTSF sends the Ntsctsf_QoSandTSCAssistance_Notify message directly to AF.

7.	The NEF sends Nnef_AFsessionWithQoS_Notify message with the event reported by the PCF to the AF.

END OF CHANGES