3GPP TSG-WG SA2 Meeting #147E 	S2-2107132r05

# Electronic Meeting, 2021-10-18–2021-10-22

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3131 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating the description of UE-DS-TT Residence Time delivery to the PCF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During the previous meeting (SA2#146), a CR (S2-2106765) correcting “Setting up an AF session with required QoS procedure” (clause 4.15.6.6) and “AF session with required QoS update procedure” (clause 4.15.6.6a) in TS 23.502 was accepted to specify how the TSCTSF serving the PDU session is getting the UE-DS-TT Residence Time from the PCF. However, the description included NF discovery and selection aspects, which were agreed to be removed from TS 23.502. Provide Alternative QoS for TSC, resolve ENs related to use of priority requested by the AF |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) The description of the procedures is partly rephrased and the NF discovery aspects are removed, instead a reference to TS 23.501 is provided. 2) We correct typos (for instance, “UE-DS-Residence Time” -> “UE-DS-TT Residence Time”) in Figures 4.15.6.6-1 and 4.15.6.6a-1 and other inconsistencies. 3) We add references to clauses 4.15.6.6 and 4.15.6.6a to Table 5.2.12.2.1-1 to list all clauses where Time-Sync data subset is used. 4) We remove the misleading statement that the Alternative Requested MFBR is a Alternative QoS Related parameter given in clause 5.7.1.2a of TS 23.501. 5) We specify that those are the Alternative QoS Profile’s parameters that are specified in clause 5.7.1.2a and provided to the NG-RAN, we correct the corresponding NOTE. In the setting up / updating an AF session with required QoS clauses 4.15.6.6 and 4.15.6.6a the following is corrected / updated: 1 – In step 1 (AF request), a reference is corrected. The currently referenced clause 5.7.1.2a of TS23.501 refers to Alternative QoS Profies (provided by the SMF to the RAN) not Alternative QoS Related parameters provided by the AF to the PCF. Instead a reference is given to 23.503 clause 6.1.3.22. Clarifying text is added at the end of step 1 with the 23.501 reference. 2 – Step 3 should only be executed when individual parameters for Alt QoS (Alternative QoS Related parameter set(s)) are not specified. Condition is added to the beginning of step 3 and exsting text at the end of step 3 is changed to a Note and a reference is added to point to 23.503 clause 6.1.3.22 3 – In steps 3a and 3e, the Alternative QoS Related parameters (ie: the individual parameters specified by the AF for a Alternative QoS Reference) are also part of the Alternative Service Requirements sent to the TSCTSF (3a) and PCF (3e). Note that the Alternative QoS Related Parameters which are sent from AF to the PCF and specify individual QoS Parameters for Alt QoS references (as per 23.503 clause 6.1.3.22) are not the same as the Alternative QoS parameter set(s) in the PCC rules, which are unchanged from Rel. 16. 4 – In step 4a, the PCF may use the Alternative QoS Related parameter sets (that are part of the Alternative Service Requirements and originated at the AF) to determine the Alternative QoS parameter set(s) in the PCC rules. Note there is no change to the contents of the PCC rules – they already contain Alternative QoS parameter Set(s). 5 – The ENs on whether and how the PCF uses the priority values specfied by an AF other than a TSN-AF is resolved. The AF requested priority may be used by the PCF to set priority of a QoS Flow as specfied in 23.501 clause 5.7.1.2a. 6 – Priority parameter is added to Nnef_AFsessionWithQoS service operations to be consistent with text in clause 4.15.6.6 and 4.15.6.6a |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misunderstanding of the specification, which may lead to incorrect implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.6, 4.15.6.6a, 5.2.12.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

4.15.6.6	Setting up an AF session with required QoS procedure

Figure 4.15.6.6-1: Setting up an AF session with required QoS procedure

1.	The AF sends a request to reserve resources for an AF session using Nnef_AFsessionWithQoS_Create request message (UE address, AF Identifier, Flow description(s) or External Application Identifier, QoS Reference, (optional) Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order), DNN, S-NSSAI) to the NEF. Optionally, a period of time or a traffic volume for the requested QoS can be included in the AF request. The NEF assigns a Transaction Reference ID to the Nnef_AFsessionWithQoS_Create request. The AF may in addition provide the following individual QoS parameters: Requested 5GS delay (optional), priority (optional), Requested GFBR, Requested MFBR, flow direction, Burst Size (optional), Burst Arrival Time (optional) at UE (uplink) or UPF (downlink), Periodicity (optional), Time domain (optional), Survival Time (optional). When Alternative Service Requirements are provided by the AF, a set of Alternative QoS Related parameters as in clause 6.1.3.22 of TS 23.503 [20]  may be provided for each QoS Reference. The Alternative QoS Related parameters may be used by the PCF to formulate PCC rules, which are subsequently used to establish Alternative QoS Profile(s) as specified in clause 5.7.1.2a of TS 23.501 [1].

2.	The NEF authorizes the AF request and may apply policies to control the overall amount of pre-defined QoS authorized for the AF. If the authorisation is not granted, all steps (except step 5) are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed.

If the NEF does not receive any of the individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20] from the AF, the steps 3, 4, 5, 6, 7, 8 are executed, otherwise, the steps 3a, 3b, 3c, 3d, 3e, 4a, 4b, 5, 6a, 7a, 7b, 8 are executed.

3.	If the NEF does not receive any of the individual QoS parameters or Alternative QoS related parameter set(s) from the AF as described in clause 6.1.3.22 of TS 23.503 [20], the NEF interacts with the PCF by triggering a Npcf_PolicyAuthorization_Create request and provides UE address, AF Identifier, Flow description(s), the QoS Reference and the optional Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order). Any optionally received period of time or traffic volume is also included and mapped to sponsored data connectivity information (as defined in TS 23.503 [20]).

If the AF is considered to be trusted by the operator, the AF uses the Npcf_PolicyAuthorization_Create request message to interact directly with PCF to request reserving resources for an AF session without individual QoS parameters.

If Alternative QoS Related parameter set(s) as described in clause 6.1.3.22 of TS 23.503 [20] are provided by the AF, they are sent to the PCF via TSCTSF.

3a.	If the NEF receives any of the individual QoS parameters or Alternative QoS related parameters as described in clause 6.1.3.22 of TS 23.503 [20] from the AF, the NEF forwards these received individual QoS parameters and Alternative QoS Related parameters in the Ntsctsf_QoSandTSCAssistance_Create request message to the TSCTSF.

The TSCTSF address may be locally configured in NF consumers (e.g., NEF, PCF, and trusted AF) for the given DNN/S-NSSAI or a NF consumer uses the TSCTSF discovery procedure as described in clause 6.3.24 of TS 23.501 [2].

If the AF is considered to be trusted by the operator, the AF uses the Ntsctsf_QoSandTSCAssistance_Create request message to interact directly with TSCTSF to request reserving resources for an AF session.

Unless the TSCTSF address is locally configured in the NEF, PCF, and trusted AF, the NEF discovers a TSCTSF and uses the obtained

TSCTSF NF ID to invoke the Ntsctsf_QoSandTSCAssistance_Create request service operation to the TSCTSF. Thereafter:

3b. If the TSCTSF NF ID for given DNN/S-NSSAI does not exist in the UDR, the selected TSCTSF stores its Notification Target Address and TSCTSF NF ID in the UDR (Data Set = Application Data; Data Subset = Time-Sync data, Data Key = S-NSSAI and DNN) using a Nudr_DM_Create service request to the UDR.

3c.	A PCF that has subscribed to receive TSCTSF information updates from the UDR receives a Nudr_DM_Notify notification of data change (i.e., that the TSCTSF stored its Notification Target Address and TSCTSF NF ID in the UDR).

NOTE: At SM Policy Session Establishment/Modification, the PCF attempts to retrieve the TSCTSF NF ID and its Notification Target Address from the UDR . If the retrieval is unsuccessful, the PCF subscribes to any Time-Sync information updates from the UDR (Data Set = Application Data; Data Subset = Time-Sync data, Data Key = S-NSSAI and DNN)

3d.	(if the TSCTSF address is not locally configured in the PCF): After retrieving the TSCTSF address from the UDR, the PCF sends Npcf_PolicyAuthorization_Notify message containing the received UE-DS-TT Residence Time to the TSCTSF .

NOTE: If the TSCTSF address is locally configured in the PCF, steps 3b and 3c are skipped. The PCF sends Npcf_PolicyAuthorization_Notify message, containing the UE-DS-TT Residence Time, to the TSCTSF at the time the SM Policy Association is established or updated.

3e.	The TSCTSF interacts with the PCF by triggering a Npcf_PolicyAuthorization_Create request and provides UE address, AF Identifier, Flow description(s), the QoS reference and the optional Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order) and optionally Alternative QoS Related parameters. Any optionally received period of time or traffic volume is also included and mapped to sponsored data connectivity information (as defined in TS 23.203 [24]).

If the TSCTSF receives the Requested 5GS delay, the TSCTSF calculates a Requested PDB by subtracting the UE-DS-TT Residence Time, provided by the PCF in step 3d, from the Requested 5GS delay. If the TSCTSF receives any of the flow direction, Burst Arrival Time, Periodicity, Time domain, Survival Time from the NEF, the TSCTSF forwards these parameters in the TSC Assistance Container in the Npcf_PolicyAuthorization_Create request to the PCF. The TSCTSF sends the Requested PDB, the TSC Assistance Container, and other received individual QoS parameters in the Npcf_PolicyAuthorization_Create request to the PCF.

4.	For requests received from the NEF in step 3, the PCF determines whether the request is authorized and notifies the NEF if the request is not authorized.

If the request is authorized, the PCF derives the required QoS parameters based on the information provided by the NEF and determines whether this QoS is allowed (according to the PCF configuration), and notifies the result to the NEF. In addition, if the Alternative Service Requirements are provided, the PCF derives the Alternative QoS parameter set(s) from the one or more QoS reference parameters contained in the Alternative Service Requirements in the same prioritized order (as defined in TS 23.503 [20]).

If the NEF provides Alternative QoS Related parameter set(s), then for each set, the PCF sets the PDB in the corresponding Alternative QoS parameter set as in clause 5.7.1.2a TS 23.501 [2]. It also sets the GFBR and MFBR according to the alternative requested values sent by the NEF. NEF specified parameter values are used to over-ride default values for the 5QI corresponding to the NEF provided QoS Reference in the Alternative Service Requirements.

If the AF is considered to be trusted by the operator, the PCF sends the Npcf_PolicyAuthorization_Create response message directly to AF.

NOTE:	The Alternative QoS Profile’s parameters provided to the NG-RAN are specified in clause 5.7.1.2a of TS 23.501 [2].

If the PCF determines that the SMF needs updated policy information, the PCF issues a Npcf_SMPolicyControl_UpdateNotify request with updated policy information about the PDU Session as described in the PCF initiated SM Policy Association Modification procedure in clause 4.16.5.2.

If the request is not authorized, or the required QoS is not allowed, NEF responds to the AF in step 5 with a Result value indicating the failure cause.

4a.	For requests received from the TSCTSF in step 3e, the PCF determines whether the request is authorized and notifies the TSCTSF if the request is not authorized.

If the request is authorized, the PCF derives the required QoS parameters based on the information provided by the TSCTSF and determines whether this QoS is allowed (according to the PCF configuration), and notifies the result to the TSCTSF. In addition, if the Alternative Service Requirements are provided, the PCF derives the Alternative QoS parameter set(s) from the one or more QoS reference parameters and Alternative QoS Related parameter set(s) contained in the Alternative Service Requirements in the same prioritized order (as defined in TS 23.503 [20]). When Alternative QoS Related parameters have been specified for a QoS Reference, they supersede the corresponding parameter associated with the QoS Reference.

The PCF sets the PDB and MDBV according to the received Requested PDB and Burst Size received from the TSCTSF. If the Requested PDB is not provided, the PCF determines the PDB that matches the QoS Reference. It also sets the GFBR and MFBR according to requested values sent by the TSCTSF. The PCF may use the Priority requested by the AF to determine QoS Flow Priority as defined in clause 5.7.3.3 of TS 23.501 [2]. TSCTSF specified parameter values are used to over-ride default values for the 5QI corresponding to the TSCTSF provided QoS Reference.

If the PCF determines that the SMF needs updated policy information, the PCF issues a Npcf_SMPolicyControl_UpdateNotify request with updated policy information about the PDU Session as described in the PCF initiated SM Policy Association Modification procedure in clause 4.16.5.2.

If the request is not authorized, or the required QoS is not allowed, TSCTSF responds to the NEF in step 4b with a Result value indicating the failure cause.

4b.	The TSCTSF sends a Ntsctsf_QoSandTSCAssistance_Create response message (Transaction Reference ID, Result) to the NEF. Result indicates whether the request is granted or not.

If the AF is considered to be trusted by the operator, the TSCTSF sends the Ntsctsf_QoSandTSCAssistance_Create response message directly to AF.

5.	The NEF sends a Nnef_AFsessionWithQoS_Create response message (Transaction Reference ID, Result) to the AF. Result indicates whether the request is granted or not.

6.	The NEF shall send a Npcf_PolicyAuthorization_Subscribe message to the PCF to subscribe to notifications of Resource allocation status and may subscribe to other events described in clause 6.1.3.18 of TS 23.503 [20].

6a.	The TSCTSF shall send a Npcf_PolicyAuthorization_Subscribe message to the PCF to subscribe to notifications of Resource allocation status and may subscribe to other events described in clause 6.1.3.18 of TS 23.503 [20].

7.	When the event condition is met, e.g. that the establishment of the transmission resources corresponding to the QoS update succeeded or failed, the PCF sends Npcf_PolicyAuthorization_Notify message to the NEF notifying about the event.

If the AF is considered to be trusted by the operator, the PCF sends the Npcf_PolicyAuthorization_Notify message directly to AF.

7a.	When the event condition is met, e.g. that the establishment of the transmission resources corresponding to the QoS update succeeded or failed, the PCF sends Npcf_PolicyAuthorization_Notify message to the TSCTSF notifying about the event.

7b.	The TSCTSF sends Ntsctsf_QoSandTSCAssistance_Notify message with the event reported by the PCF to the NEF.

If the AF is considered to be trusted by the operator, the TSCTSF sends the Ntsctsf_QoSandTSCAssistance_Notify message directly to AF.

8.	The NEF sends Nnef_AFsessionWithQoS_Notify message with the event reported by the PCF to the AF.

The AF may send Nnef_AFsessionWithQoS_Revoke request to NEF in order to revoke the AF request. The NEF authorizes the revoke request and triggers the Ntsctsf_QoSandTSCAssistance_Delete/Unsubscribe and/or Npcf_PolicyAuthorization_Delete and the Npcf_PolicyAuthorization_Unsubscribe operations for the AF request.

>>>> Next Change <<<<

4.15.6.6a	AF session with required QoS update procedure

Figure 4.15.6.6a-1: AF session with required QoS update procedure

1.	For an established AF session with required QoS, the AF may send a Nnef_AFsessionWithQoS_Update request message (AF Identifier, Transaction Reference ID, [Flow description(s)], [QoS Reference], [Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order)]) to NEF for updating the reserved resources. Optionally, a period of time or a traffic volume for the requested QoS can be included in the AF request. The Transaction Reference ID provided in the AF session with required QoS update request message is set to the Transaction Reference ID that was assigned, by the NEF, to the Nnef_AFsessionWithQoS_Create request message. The AF may in addition provide the following individual QoS parameters: Requested 5GS delay (optional), priority (optional), Requested GFBR, Requested MFBR, flow direction, Burst Size (optional), Burst Arrival Time (optional) at UE (uplink) or UPF (downlink), Periodicity (optional), Time domain (optional), Survival Time (optional). When Alternative Service Requirements are provided by the AF, a set of Alternative QoS Related parameters as specified in clause 6.1.3.22 of TS 23.503 [20]  may be provided for each QoS Reference. The Alternative QoS Related parameters may be used by the PCF to formulate PCC rules, which are subsequently used to establish Alternative QoS Profile(s) as specified in clause 5.7.1.2a of TS 23.501 [1]

2.	The NEF authorizes the AF request of updating AF session with required QoS and may apply policies to control the overall amount of pre-defined QoS authorized for the AF. If the authorisation is not granted, all steps (except step 5) are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed.

If the NEF does not receive any of the individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20] from the AF, then the steps 3, 4, 5, 6, 7 are executed, otherwise, the steps 3a, 3b, 3c, 3d, 3e, 4a, 4b, 5, 6a, 6b, 7 are executed.

3.	If the NEF does not receive any of the individual QoS parameters or Alternative QoS Related parameter set(s) from the AF as described in clause 6.1.3.22 of TS 23.503 [20], the NEF interacts with the PCF by triggering a Npcf_PolicyAuthorization_Update request and provides UE address, AF Identifier, Flow description(s), the QoS Reference and the optional Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order). Any optionally received period of time or traffic volume is also included and mapped to sponsored data connectivity information (as defined in TS 23.503 [20]).

If the AF is considered to be trusted by the operator, the AF uses the Npcf_PolicyAuthorization_Update request message to interact directly with PCF to update the reserving resources for an AF session.

If Alternative QoS Related parameter set(s) as described in clause 6.1.3.22 of TS 23.503 [20] are provided by the AF, they are sent to the PCF via TSCTSF.

3a.	If the NEF receives one or more of the individual QoS parameters or Alternative QoS Related parameters as described in clause 6.1.3.22 of TS 23.503 [20] from the AF, the NEF forwards these received individual QoS parameters and Alternative QoS Related parameters in the Ntsctsf_QoSandTSCAssistance_Update or Ntsctsf_QoSandTSCAssistance_Create request message to the TSCTSF.

The TSCTSF address may be locally configured in NF consumers (e.g., NEF, PCF, and trusted AF) for the given DNN/S-NSSAI or an NF consumer uses the TSCTSF discovery procedure as described in clause 6.3.24 of TS 23.501 [2].

If the AF is considered to be trusted by the operator, the AF uses the Ntsctsf_QoSandTSCAssistance_Update request message to interact directly with TSCTSF to update the reserving resources for an AF session with individual QoS parameters.

If the NEF sends a Ntsctsf_QoSandTSCAssistance_Update request to the TSCTSF, steps 3b–3d are skipped.

If the NEF sends a Ntsctsf_QoSandTSCAssistance_Create request to the TSCTSF and the TSCTSF address is locally configured in the NEF and PCF, steps 3b and 3c are skipped, otherwise, the NEF discovers the TSCTSF as described in clause 6.3.24 of TS 23.501 [2] and uses the obtained

TSCTSF NF ID to invoke the Ntsctsf_QoSandTSCAssistance_Create request service operation to the TSCTSF. Thereafter:

3b.	If the TSCTSF NF ID for given DNN/S-NSSAI does not exist in the UDR, the selected TSCTSF stores/updates its Notification Target Address and TSCTSF NF ID in the UDR (Data Set = Application Data; Data Subset = Time-Sync data, Data Key = S-NSSAI and DNN) using a Nudr_DM_Create service request to the UDR.

3c.	A PCF that has subscribed to receive TSCTSF information updates from the UDR receives a Nudr_DM_Notify notification of data change (i.e., that the TSCTSF stored its Notification Target Address and TSCTSF NF ID in the UDR).

NOTE: At SM Policy Session Establishment/Modificaton, the PCF attempts to retrieve  the TSCTSF  NF ID and its Notification Target Address from the UDR. If the retrieval is unsuccessful, the PCF subscribes to any Time-Sync information updates from the UDR: (Data Set = Application Data; Data Subset = Time-Sync data, Data Key = S-NSSAI and DNN).

3d.	(if the TSCTSF address is not locally configured in the PCF): The PCF sends Npcf_PolicyAuthorization_Notify message, containing the received UE-DS-TT Residence Time, to the TSCTSF after retrieving the TSCTSF address from the UDR.

NOTE: If the TSCTSF address is locally configured in the PCF, steps 3b and 3c are skipped. The PCF sends Npcf_PolicyAuthorization_Notify message, containing the UE-DS-TT Residence Time, to the TSCTSF at the time SM Policy Association is established or updated.

3e.	The TSCTSF interacts with the PCF by triggering a Npcf_PolicyAuthorization_Update or Npcf_PolicyAuthorization_Create request and provides UE address, AF Identifier, Flow description(s), the QoS reference and the optional Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order) and optionally Alternative QoS Related parameters. Any optionally received period of time or traffic volume is also included and mapped to sponsored data connectivity information (as defined in TS 23.203 [24]).

If the TSCTSF receives the Requested 5GS delay, the TSCTSF calculates a Requested PDB by subtracting the UE-DS-TT residence time, provided by the PCF in step 3d, from the Requested 5GS delay. If the Requested PDB is not provided, the PCF determines the PDB that matches the QoS Reference. The PCF may use the Priority requested by the AF to determine QoS Flow Priority as defined in clause 5.7.3.3 of TS 23.501 [2].

If the TSCTSF receives any of the flow direction, Burst Arrival Time, Periodicity, Time domain, Survival Time from the NEF, the TSCTSF forwards these parameters in the TSC Assistance Container in the Npcf_PolicyAuthorization_Update request to the PCF. The TSCTSF sends the Requested PDB, the TSC Assistance Container, and other received individual QoS parameters in the Npcf_PolicyAuthorization_Update request to the PCF.

4.	If the PCF received request from the NEF in step 3, the PCF determines whether the request is authorized.

If the request is authorized, the PCF derives the required QoS parameters based on the information provided by the NEF and determines whether this QoS is allowed (according to the PCF configuration), and notifies the result to the NEF. In addition, if the Alternative Service Requirements are provided, the PCF derives the Alternative QoS parameter set(s) from the one or more QoS reference parameters contained in the Alternative Service Requirements in the same prioritized order (as defined in TS 23.503 [20]).

If the NEF provides Requested PDB, Requested GFBR, Requested MFBR or Burst Size, then the PCF sets the PDB and/or MDBV according to the received Requested PDB and Burst Size received from the NEF. It also sets the GFBR and MFBR according to the requested values provided by the NEF. NEF specified parameter values are used to over-ride default values for the 5QI corresponding to the NEF provided QoS Reference. If the NEF provides Alternative QoS Related parameter set(s), then for each set, the PCF sets the PDB and MDBV in the corresponding Alternative QoS parameter set as described in clause 6.1.3.22 of TS 23.503 [20]. It also sets the GFBR and MFBR according to the alternative requested values sent by the NEF. NEF specified parameter values are used to over-ride default values for the 5QI corresponding to the NEF provided QoS Reference in the Alternative Service Requirements.

If the PCF determines that the SMF needs updated policy information, the PCF issues a Npcf_SMPolicyControl_UpdateNotify request with updated policy information about the PDU Session as described in the PCF initiated SM Policy Association Modification procedure in clause 4.16.5.2.

If the AF is considered to be trusted by the operator, the PCF sends the Npcf_PolicyAuthorization_Update response message directly to AF.

If the request is not authorized or the required QoS is not allowed, NEF responds to the AF in step 5 with a Result value indicating the failure cause.

4a. If the PCF received request from the TSCTSF in step 3e, the PCF determines whether the request is authorized.

If the request is authorized, the PCF derives the required QoS parameters based on the information provided by the TSCTSF and determines whether this QoS is allowed (according to the PCF configuration for this AF), and notifies the result to the TSCTSF. In addition, if the Alternative Service Requirements are provided, the PCF derives the Alternative QoS parameter set(s) from the one or more QoS reference parameters and Alternative QoS Related parameter set(s) contained in the Alternative Service Requirements in the same prioritized order (as defined in TS 23.503 [20]). When Alternative QoS Related parameters have been specified for a QoS Reference, they supersede the corresponding parameter associated with the QoS Reference.

If the TSCTSF provides Requested PDB, priority, Requested GFBR, Requested MFBR or Burst Size, then the PCF sets the PDB and/or MDBV according to the received Requested PDB and Burst Size received from the TSCTSF. If the Requested PDB is not provided from TSCTSF, the PCF determines the PDB that matches the QoS Reference. It also sets the GFBR and MFBR according to the requested values provided by the TSCTSF. TSCTSF specified parameter values are used to over-ride default values for the 5QI corresponding to the TSCTSF provided QoS Reference.

If the PCF determines that the SMF needs updated policy information, the PCF issues a Npcf_SMPolicyControl_UpdateNotify request with updated policy information about the PDU Session as described in the PCF initiated SM Policy Association Modification procedure in clause 4.16.5.2.

If the request is not authorized or the required QoS is not allowed, TSCTSF responds to the NEF in step 4b with a Result value indicating the failure cause.

4b.	The TSCTSF sends a Ntsctsf_QoSandTSCAssistance_Update or Ntsctsf_QoSandTSCAssistance_Create response message (Transaction Reference ID, Result) to the NEF. Result indicates whether the request is granted or not.

If the AF is considered to be trusted by the operator, the TSCTSF sends the Ntsctsf_QoSandTSCAssistance_Update response message directly to AF.

5.	The NEF sends a Nnef_AFsessionWithQoS_Update response message (Transaction Reference ID, Result) to the AF. Result indicates whether the request is granted or not.

6.	The PCF sends Npcf_PolicyAuthorization_Notify message to the NEF when the modification of the transmission resources corresponding to the QoS update succeeded or failed.

If the AF is considered to be trusted by the operator, the PCF sends the Npcf_PolicyAuthorization_Notify message directly to AF.

6a.	The PCF sends Npcf_PolicyAuthorization_Notify message to the TSCTSF when the modification of the transmission resources corresponding to the QoS update succeeded or failed.

6b.	The TSCTSF sends Ntsctsf_QoSandTSCAssistance_Notify message with the event reported by the PCF to the NEF.

If the AF is considered to be trusted by the operator, the TSCTSF sends the Ntsctsf_QoSandTSCAssistance_Notify message directly to AF.

7.	The NEF sends Nnef_AFsessionWithQoS_Notify message with the event reported by the PCF to the AF.

>>>> Next Change <<<<

5.2.12.2	Nudr_DataManagement (DM) service

5.2.12.1.1	General

The operations defined for Nudr_DM service use following set of parameters defined in this clause:

-	Data Set Identifier: uniquely identifies the requested set of data within the UDR (see clause 4.2.5).

-	Data Subset Identifier: it uniquely identifies the data subset within each Data Set Identifier. As specified in the procedures in clause 4, e.g. subscription data can consist of subsets particularised for specific procedures like mobility, session, etc.

-	Data Keys defined in Table 5.2.12.2.1-1

For Nudr_DM_Subscribe and Nudr_DM_Notify operations:

-	The Target of Event Reporting is made up of a Data Key and possibly a Data Sub Key both defined in Table 5.2.12.2.1-1. When a Data Sub Key is defined in the table but not present in the Nudr_DM_Subscribe this means that all values of the Data Sub Key are targeted.

-	The Data Set Identifier plus (if present) the (set of) Data Subset Identifier(s) corresponds to a (set of) Event ID(s) as defined in clause 4.15.1

An NF Service Consumer may include an indicator when it invokes Nudr_DM Query/Create/Update service operation to subscribe the changes of the data, to avoid a separate Nudr_DM_Subscribe service operation.

Depending on the use case, it is possible to use a Data Key and/or one or multiple Data sub keys to further identify the corresponding data, as defined in Table 5.2.12.2.1-1 below.

Table 5.2.12.2.1-1: Data keys

| Data Set | Data Subset | Data Key | Data Sub Key |
| --- | --- | --- | --- |
|  | Access and Mobility Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | SMF Selection Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | UE context in SMF data | SUPI | PDU Session ID or DNN |
| Subscription Data (see clause 5.2.3.3.1) | SMS Management Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | SMS Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | Session Management Subscription data | SUPI | S-NSSAI |
|  |  |  | DNN |
|  |  |  | Serving PLMN ID and optionally NID |
|  | Slice Selection Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | Group Data | Internal Group Identifier or External Group Identifier | - |
|  | Identifier translation | GPSI |  |
|  |  | SUPI | Application Port ID |
|  | Intersystem continuity Context | SUPI | DNN |
|  | LCS privacy | SUPI | - |
|  | LCS mobile origination | SUPI | - |
|  | UE reachability | SUPI | - |
|  | Group Identifier Translation | Internal Group Identifier or External Group Identifier | - |
|  | UE context in SMSF data | SUPI | - |
|  | V2X Subscription data | SUPI | - |
|  | ProSe Subscription data | SUPI | - |
|  | User consent | SUPI | Purpose |
|  | ECS Address Configuration Information (See Table 4.15.6.3d-1) | SUPI |  |
| Application data | Packet Flow Descriptions (PFDs) | Application Identifier | - |
|  | AF traffic influence request information | AF transaction internal ID |  |
|  | (See clause 5.6.7 and clause 6.3.7.2 of TS 23.501 [2]) | S-NSSAI and DNN and/or Internal Group Identifier or SUPI |  |
|  | Background Data Transfer (NOTE 3) | Internal Group Identifier or SUPI |  |
|  | Service specific information (See clause 4.15.6.7) | S-NSSAI and DNN or Internal Group Identifier or SUPI |  |
|  | EAS Deployment Information | See TS 23.548 [74] clause 7. (See NOTE 4) |  |
|  | AM policy influence request information (See clause 4.15.6.9.3) | AF transaction internal ID |  |
|  |  | S-NSSAI and DNN and/or Internal Group Identifier or SUPI |  |
|  | Time-Sync data (See clause 4.15.6.6, 4.15.6.6a, 4.15.9.2, 4.15.9.4 and 4.16.5.1, NOTE 5) | DNN and S-NSSAI Internal Group Identifier SUPI |  |
| Policy Data | UE context policy control data (See clause 6.2.1.3 of TS 23.503 [20]) | SUPI |  |
|  | PDU Session policy control data | SUPI | S-NSSAI |
|  | (See clause 6.2.1.3 of TS 23.503 [20]) |  | DNN |
|  | Policy Set Entry data (See clause 6.2.1.3 of TS 23.503 [20]) | SUPI (for the UDR in HPLMN) |  |
|  |  | PLMN ID (for the UDR in VPLMN) |  |
|  | Remaining allowed Usage data | SUPI | S-NSSAI |
|  | (See clause 6.2.1.3 of TS 23.503 [20]) |  | DNN |
|  | Sponsored data connectivity profiles (See clause 6.2.1.6 of TS 23.503 [20]) | Sponsor Identity |  |
|  | Background Data Transfer data (See clause 6.2.1.6 of TS 23.503 [20]) | Background Data Transfer Reference ID. (NOTE 2) |  |
|  |  | None. (NOTE 1) |  |
|  | Network Slice Specific Control Data (See clause 6.2.1.3 of TS 23.503 [20]) | S-NSSAI |  |
| Exposure Data | Access and Mobility Information | SUPI or GPSI | PDU Session ID or |
| (see clause 5.2.12.1) | Session Management information | SUPI or GPSI | UE IP address or DNN |
| NOTE 1: Retrieval of the stored Background Data Transfer References for all ASP identifiers in the UDR requires Data Subset but no Data Key or Data Subkey(s). NOTE 2: Update of a Background Data Transfer Reference in the UDR requires a Data key to refer to a Background Data Transfer Reference as input data. NOTE 3: The Background Data Transfer includes the Background Data Reference ID and the ASP Identifier that requests to apply the Background Data Reference ID to the UE(s). Furthermore, the Background Data Transfer includes the relevant information received from the AF as defined in clause 6.1.2.4 of TS 23.503 [20]. NOTE 4: The data Keys, and data structure of EAS Deployment Information are defined in clause 7 of TS 23.548 [74]. NOTE 5: The Time-Sync data subset includes the TSCTSF Notification Target Address, DNN and S-NSSAI, Internal Group Identifier or SUPI. Additionally, the TSCTSF NF ID shall also be included in the Time-Sync data subset for QoS use cases (clauses 4.15.6.6 and 4.15.6.6a), and other AF request information (e.g., Event Filter) may be included for other use cases. |  |  |  |

The content of the UDR storage for (Data Set Id= Application Data, Data Subset Id = AF TrafficInfluence request information) is specified in clause 5.6.7, Table 5.6.7-1 of TS 23.501 [2]. This information is written by the NEF and read by the PCF(s). PCF(s) may also subscribe to changes onto this information.

>>>> End of Changes <<<<