3GPP TSG-SA2 Meeting #147E	S2-2107167r01

# Electronic, 18th - 22nd October 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 005 | rev - |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | USS Initiated procedures update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS |  |  |  |  |  | Date: |  |  | 2021-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Finalization of EPS USS initiated procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding missing EPS USS initiated procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete description of EPS USS initiated procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.4, 5.2.5.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------First Change---------------------------------------------

#### 5.2.4.4	USS initiated UAV Re-authorization procedure in EPS

For USS initiated UAV Re-authorization in EPS, the procedure described in clause 5.2.4.3 is followed with following differences:

-	SMF is replaced by SMF+PGW-C combo function, PDU Session is replace by PDN connection.

-	At step 2 the UAS NF/NEF retrieves the corresponding stored UUAA context which includes information on whether a UUAA-SM has been performed and the identity of the SMF+PGW-C and the IP address of the PDN Connection that is serving the UUAA

-	Steps 3a and 5a are not relevant as UUAA-MM is only performed in 5GS

-	At step 5b the SMF+PGW-C invokes the PDN GW initiated bearer modification without QoS update procedure (figure 5.4.3-1 of TS 23.401 [6]) by sending Update Bearer Request message, including the CAA-Level UAV ID and the authorization message in the PCO.

The Update Bearer Request message including the CAA-Level UAV ID and the authorization message is forwarded by the MME as Downlink NAS Transport to the UE (steps 4 and 5 in figure 5.4.3-1 of TS 23.401 [6]).

----------------------------------------Next Changes--------------------------------------------

#### 5.2.5.4	USS initiated C2 pairing policy configuration

##### 5.2.5.4.2	USS initiated C2 pairing policy configuration in EPS

For USS initiated C2 pairing policy configuration in EPS, the procedure described in clause 5.2.5.4.1 is followed with following differences:

-	SMF is replaced by SMF+PGW-C combo function, PDU Session is replace by PDN connection.

-	Based on the updated policy information received at step 4,

-	the SMF+PGW-C determines and provides N4 rules to enable communication between UAV and UAV-C, e.g. Packet Detection Rules, Forwarding Action Rules.

-	[Conditional] If QoS needs to be updated: the SMF+PGW-C determines N4 rules for QoS update and provides to the PGW-U.

-	At step 5, the SMF+PGW-C invokes the PDN GW initiated bearer modification with bearer QoS update procedure (clause 5.4.2.1 in TS 23.401 [6]) by sending Update Bearer Request message to the SGW. Steps 2 – 11 in clause 5.4.2.1-1 of TS 23.401 [6] are executed to update QoS in the UE and the RAN.

----------------------------------------End of Changes--------------------------------------------