3GPP TSG-SA2 Meeting # 147E (e-meeting)	S2-2107183

# Elbonia, October 18 - 22, 2021													 (revision of)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3136 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RID Update for SNPN UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When the SNPN uses 5G-AKA based or EAP-AKA' based primary authentication and key agreement procedure which use USIM, then the UE obtains Routing Indicator from the USIM. However, it is not clear how the UE obtains the Routing Indicator specially in the case where the SNPN UE does not contain a USIM. For this case TS 33.501 Annex I.5 indicates that the location of the functionality related to SUPI privacy in the UE is out of scope. The Routing Indicator in SNPN UEs that do not contain a USIM must be stored at the ME. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CP procedure for UE Parameter Update is updated so that it is possible to update the Routing Indicator in the ME. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to configure Routing Indicator in SNPN UEs without USIM. SNPN UEs may not get connectivity if routing or authentication requests do not reach the appropriate AUSF/UDM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.20.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

### 4.20.1	General

The purpose of the control plane solution for update of UE parameters is to allow the HPLMN, SNPN, or CH to update the UE with a specific set of parameters, generated and stored in the UDM, by delivering protected UDM Update Data via NAS signalling. The HPLMN, SNPN, or CH updates such parameters based on the operator policies.

The UDM Update Data that the UDM delivers to the UE may contain:

-	one or more UE parameters including:

-	the updated Default Configured NSSAI (final consumer of the parameter is the ME);

-	NSSAA credentials per S-NSSAI as defined in TS 33.501 [15] (final consumer of the parameter is the ME or USIM); and

-	DN-specific credentials for authentication/authorization of the PDU Session establishment as defined in TS 33.501 [15] (final consumer of the parameter is the ME or USIM);

-	the updated Routing Indicator Data (final consumer of the parameter is the USIM when the related credential is stored in the USIM (e.g. PLMN credentials); or the ME when the related credential is stored in the ME).

-	a "UE acknowledgement requested" indication.

-	a "re-registration requested" indication.

****************** END CHANGES ***************