SA WG2 Meeting #S2-146E	S2-2107222

# 16 – 27 August, 2021, Electronic meeting

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3259 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network slice status notifications when multiple NSACFs exist for one S-NSSAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The existing procedures and descriptions in 5.15.11.4 havn’t cover multiple NSACFs for one S-NSSAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR adds description of network slice status notifications while multiple NSACFs exist for one S-NSSAI in clause 5.15.11.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network slice event notification for multiple NSACFs per S-NSSAI cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.11.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< First change >>>

5.15.11.4	Network Slice status notifications and reports to a consumer NF

A consumer NF (e.g. AF) can subscribe with the NSACF for Network Slice status notifications and reports. Upon such subscription, the NSACF can provide event based notifications and reports to the consumer NF (e.g. to AF via NEF) related to the current number of UEs registered for a network slice or the current number of PDU Sessions established on a network slice or both.

If there are multiple NSACFs for one S-NSSAI, and AF subscribes the network slice status through NEF, the AF should provide interesting/requestingarea information to help NEF select a specific NSACF.

NOTE: If NEF finds more than one NSACFs under the interesting/requestingarea AF provides, NEF responses failure for AF subscription in this release.

If the consumer NF is trust AF, it may query the NRF directly to find the NSACF(s) and subscribe the network slice status for each NSACF directly based on trust AF requirement.

<<< End of changes >>>